Code No.RR-210401

Set No.

1

II B.Tech. I-Semester Regular Examinations, November-2003

ELECTRONIC CIRCUIT ANALYSIS

(Electronics and Communication Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	For the emitter follower with Rs = 0.5 K and RL = 5K, calculate AI, Ri, AV, AVS and Ro. Assume hfe = 50, hie = 1K, hoe = 25A/V.

b)	Draw the circuit diagram and Low frequency equivalent circuit of common source 	amplifier and derive an expression for its voltage  gain.

2.a)	Draw the Hybrid- equivalent circuit of  BJT and explain the significance of each parameter. Mention typical values of hybrid- parameters.

b)	Given the following transistor measurements made at IC = 5 mA, VCE 10V and at 	room temperature hfe = 100, hie= 600 , [Aie] = 10 at 10Mhz, Ce = 3 Pf . Find f, 	fT, Ce, rb'e and rbb'.

3.a)	The gain of an RC coupled 2 stage FET amplifier falls by 90% of the midband value at 400 kHz.   If gm of each FET is 10 m A/V, and total output capacitance for each stage is 20 pf.  Calculate the RL required and the stage midband gain.

b)	Write a short note on Bandwidth of amplifiers.

4.a)	Explain about class A, class B, class AB and class C operation of power 	amplifiers.

b)	Design a class B power amplifiers to deliver 25w to a load resistor RL=8Ω, using 	transformer coupling. Vm=Vcc=25V.  Assume reasonable data wherever 	necessary.

5.a)	Draw the circuit diagram of a complementary symmetry push pull amplifier and explain its working.

b)	Distinguish between cross over distortion and harmonic distortion.  How they can 	be eliminated.

(Contd…2)

Code No.RR-210401				-2-			Set No.1

6.a) 	Draw the circuit of a Common – Emitter amplifier that is converted to a tuned band pass  amplifier; using transformer coupling with tuned primary circuit.

b) 	In a single tuned amplifier with tuned primary circuit, Transistor has a trans- conductance   of 30ms at the resonant frequency and an output conductance composed of 100 K ohms in  parallel with 5 pf of capacity. The transformer has a primary inductance of 80 H,  secondary inductance of 100 H., coefficient of coupling of 0.01; primary resistance of 10    and secondary resistance of 10 . The primary is tuned with a 45- pf capacitor and 5 K    resistor loads the secondary. Find (a) the resonant frequency and (b) the effective Q of the

tuned circuit and (c) 3-db bandwidth.

7.a) 	Compare the merits and limitations of different voltage regulator circuits.

b)	For the given Zener regulator circuit, the supply voltage varies from 20 to 30 	Volts. Load current varies from 0 to 95ma.  Zener breakdown occurs at 5ma.  	Find the value of Rs, RLmin, RLmax. (Assume Zener breakdown occurs at 5ma)

8.a) 	What is meant by voltage multiplier?  List out the names of four different 	multipliers.

b)	Draw the circuit of a full-wave voltage doubler circuit and explain its operation.  	Mention the PIV of each diode.

***     ***     ***

Code No.RR210401

Set No.

2

II B.Tech. I-Semester Regular Examinations, November-2003

ELECTRONIC CIRCUIT ANALYSIS

(Electronics and Communication Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	What is swamping resistor? What are its effects when used in CS amplifier circuit? Explain clearly.

b)	Design 1-stage emitter follower having  Ri = 500 K and Ro = 20. Assume      	hfe = 50, hie = 1K, hoe 25A/V

2.a)	Derive the expressions for transconductance and input conductance of CE amplifier using HF model.

b)	The following LF parameters are known for a given transistor at Ic = 10 mA,

VCE =5V and at room temperature  hie = 500, hoe = 10-5A/V, hfe = 100, hre = 10-4. 	At the same operating point fT = 50 MHz, and Cob = 3 PF, Compute the values of 	all the hybrid- parameter.

3.a)	Three identical non interacting amplifier stages in cascade have an overall gain of 0.3dB down at 50 kHz compared to midband.  Calculate the upper cutoff frequency of the individual stages.

b)	Write a note on distortions in amplifiers.

4.a)	Compare different types of amplifier circuits based on the type of coupling.

b)	Design a class A transformer coupled amplifier using a BJT to deliver 100 mw of 	audio power into 8Ω load.  At the operating point IB=250 µA. Vcc=16V.  The 	collector dissipation should not exceed 200 mw.  RL=1kΩ.  Make reasonable 	approximations wherever necessary.

5.a)	What are the two disadvantages of push pull Amplifiers.

b)	Give schematic of class B pushpull amplifier with complementary symmetry and  	explain the working of it.

6.a) 	Explain about synchronous tuning of Tuned amplifiers with an example of a block

diagram for the system concept.

b) 	Draw the circuit of double-tuned transformer-coupled amplifier. Discuss the 	nature of  responses of the amplifier for different values of KQ=1; KQ>1 and 	KQ<1.

(Contd…2)

Code No.RR210401				-2-			Set No.2

7.a)	What are the limitations of various Linear regulators.  Explain.

b)	In the given Zener diode regulator circuit, Zener diode conducts if the supply 	voltage Vi is greater than 20V.  Zener diode voltage is 16V.  The meter resistance 	is 500 ohms.  Find R1 and R2 where R1 + R2 = 99.5K.

8.a) 	Draw the circuit diagram of half wave voltage doubler circuit and explain its operation.  Sketch the input and output waveforms.  Mention what is the PIV of diode.  What is its output voltage under no load conditions?

b)	Why regulator is necessary for power supplies – Give reasons.

***     ***     ***

Code No.RR210401

Set No.

3

II B.Tech. I-Semester Regular Examinations, November-2003

ELECTRONIC CIRCUIT ANALYSIS

(Electronics and Communication Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Derive the expressions for Ai, AV of CE amplifier circuit. Explain how Ai and AV are effected by RS.

b)	Consider a single stage CE amplifier with RS = 1K, Ri = 50 K, R2 = 2K, RC = 1K, RL = 1.2K, hfe = 50, hie = 1.1K, hre = hoe = 0. Find AI, Ri, Ro, AV and power gain.

2.a)	Derive the expressions for feedback conductance and base spreading resistance of CE amplifier using hybrid- model.

b)	Given the following transistor measurements made at Ic=5mA; VCE=10V and at 	room temperature hfe=100; hie=600; [Aie]= 10 at 10 mHz, Cc=3pf.  Find f, fT, ce, 	rb'e and r bb'.

3.	A three stage CE amplifier using silicon BJT as shown has RF=50k, RF2=10k,

=50, RL1=15k and Vcc=20 V.  Calculate the value of Vc3. Also show that the amplifier is dc stabilized for large values of .

(Contd…2)

Code No.RR210401				-2-			Set No.3

4.a)	What are the advantages of push-pull connection in AF power amplifiers?

b)	Design a push-pull class B power amplifier to achieve maximum output power to 	a 10 load.  The two transistors have the following ratings Pc(Max)=4W; 	VCE(Max)=40V.  IC(Max)=1 A.  Assume reasonable values of data wherever 	necessary.

5.a)	What is thermal resistance?  What is the unit of thermal resistance.

b)	Explain the commonly available heat sinks.

6.a) 	Draw the circuit of FET tuned Voltage amplifier. Derive the necessary expression

to draw   the universal resonance curve with all necessary details.

b) 	Design the single stage FET tuned amplifier for the following specifications, fo=12 M Hz. Bandwidth B=10 KHZ and midband voltage gain Avm =  -15. The FET parameters are gm=4 ms rd=25K ohms; C GS= 30pf; C GD=C DS=5pf.

7.a)	 Compare the merits and demerits of different voltage regulator circuits.

b)	Define the following terms

(i) Line Regulation		(ii) Load Regulation

c)	In the given regulator circuit, the supply voltage varies from 15 to 20V.  10 Volts, 	2 Watts Zener is used.  Find the value of Rs and its power dissipations.  (Assume 	Zener breakdown occurs at 5 ma.)

8.a) 	Draw the circuit diagrams of voltage doubler and voltage tripler and explain their working.

b)	What are the limitations of 3 terminal regulator?

***     ***     ***

Code No.RR210401

Set No.

4

II B.Tech. I-Semester Regular Examinations, November-2003

ELECTRONIC CIRCUIT ANALYSIS

(Electronics and Communication Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	What is square law distortion? What is its effect in FET amplifiers? Compare important characteristics of CD, CS, CG  FET amplifier.

b)	A self-biased CE amplifier circuit has R1 = 100K , R2 = 10K, Rc = 5K,

RE = 1K, RS = 10K. Compute AI, AV, AVS and Ri. The h-parameters of the 	transistor are hie = 1.1k, hfe = 50, hre  = 2.5 x 10-4 , hoe = 25A/V.

2.a)	Derive the expression for output conductance and diffusion capacitance of hybrid- CE amplifier.

b)	A single-stage CE amplifier is measured to have a voltage gain bandwidth fH of 	5MHz with RL = 500 . Assume hfe = 100, gm= 100mA/V, rbb' = 100, Cc= 1PF, 	and fT = 400 MHz (i) Find the value of the source resistance that will give the 	required bandwidth.  (ii) with the value of Rs, determined in part (i) find the mid band 	voltage gain Vo / VS.

3.a)	How is the High frequency gain of a single stage amplifier dependent on frequencies f1 and f2.

b)	In an RC-coupled BJT amplifier, we have RL=6.8k, effective ac load after Cc is 	Rac=1k, Cc=1  f, CE=24F, RE=2.2k, hfe =49, RS=5k and hie=1k, find the low 	frequency cut off  point.

4.a)	Explain about heat sinks.  Explain the term Thermal Resistance.  Give the 	sketches of heat sinks.

b)	What is the Junction to ambient Thermal Resistance for a device dissipating      	600 mw into an ambient temperature of 50oC and operating at a junction 	temperature of 110oC?

5.a)	Explain why the complimentary symmetry power amplifier has become more popular in modern circuits.

b)	Draw a practical circuit of a complimentary symmetry push pull amplifier circuit?

Explain its function.

(Contd…2)

Code No.RR210401				-2-			Set No.4

6.a) 	Explain the principle of stabilizing the double-tuned transformer coupled

amplifier response against the internal feedback

b) 	Tuned amplifier has an internal feedback capacitance of 10 Pf, which has to be neutralized. It operates at 10MHZ and the output transformer is connected with a tapped primary at Na: Nb=1:4. Its primary inductance is 5 H. If, Hazeltine neutralization is to be used, calculate the size of the neutralizing capacitor needed. Draw the Hazeltine neutralization circuit also.

7.a)	 Design a series regulated power supply with following specifications.

Unregulated input voltage Vi = 30V and r0 = 8 ohms

Regulated output voltage = 12 V

Maximum load current = 200 ma.

Control transistor hfe = 100, hie = 200 ohms

Amplifier transistor hfe= 200, hie = 1000 ohms

Reference voltage VR = 6V

Zener resistance Rz = 10 ohms

Zener current = 20 mA

b) 	Also calculate its stability factor and output resistance.

c) 	Also draw the complete circuit diagram with the designed component values.

8.a) 	What are the important features of three terminal regulators?  Explain. Draw the circuit diagram of a three terminal regulator as a current source and explain its operation.

b)	Draw the circuit diagram of a voltage quadrupler and explain its working.

***     ***     ***