Code No: RR-210402/NR-210403

Set No.

1

II B.Tech. I-Semester Regular Examinations, November-2003.

SIGNALS AND SYSTEMS

(Common to Electronics and Computer Engineering, Electronics and Instrumentation Engineering, Instrumentation and Control Engineering, Electronics and Control Engineering and Electronics and Telematics)

Time: 3 hours							   Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.	Define vector space.  Establish the following identity:

a) 	||u+v||2 + ||u-v||2=2||u||2+2||v||2

b) 	u.v=1/4||u+v||2-1/4||u-v||2

2.a)	Find out the exponential Fourier series & plot the magnitude and phase spectrum for the rectangular pulse train shown in the following figure.

b)	Write short notes on exponential Fourier spectrum and the concept of negative frequency.

3.a)	State and prove convolution and differentiation properties of F.T.

b)	A signal x(t) is given as x(t)=6 cos 10 t.  This signal is sampled by an impulse train.  The sampling frequency of the impulse train are 7 Hz and 14 Hz.  Draw the spectra of the original signal.  Draw the spectra of the sampled signal with sampling frequency 7 Hz and 14 Hz.

4.	There are several possible ways of estimating an essential bandwidth of non-band limited signal. For a low pass signal, for example, the essential bandwidth may be chosen as a frequency where the amplitude spectrum of the signal decays to k percent of its peak value. The choice of k depends on the nature of application. Choosing k= 5 determine the essential bandwidth of g(t)=exp(-at) u(t).

Contd…..2.

Code No: RR-210402/NR-210403		:: 2 ::				Set No:1

5.a)	Explain the physical significance of Power Spectral density and Energy Spectral density. Explain in detail how bandwidth and rise time are related and derive the relation.

b)	Discuss about the conditions for physical reliability of an LTI system.

6.a)	Determine the auto correlation function of a sinc pulse ASinc(2wt).

b)	List out the properties of auto correlation function.

7.a)	The signal x(t) = sin 2t oscillates between +1 and  -1  as t      ∞ . So it does not have a final value. Show that the application of the final value theorem gives an incorrect result for the signal.

b)	State and prove the time-differentiation and time-integration properties.

8.a)	What are the methods by which inverse Z- transformation can be found out?

b)	Given X (z)= 11−az−1,za . Find x (n) using long division method.

@@@@@

Code No: RR-210402/NR-210403

Set No.

2

II B.Tech. I-Semester Regular Examinations, November-2003.

SIGNALS AND SYSTEMS

(Common to Electronics and Computer Engineering, Electronics and Instrumentation Engineering, Instrumentation and Control Engineering, Electronics and Control Engineering and Electronics and Telematics)

Time: 3 hours							   Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Prove that (i) A.(B*C)=(A*B).C	(ii) A.(A*C)=0.

b)	Determine whether the following vectors are linearly independent or not.

A=2i+j-3k, B=i-4k, C=4i+3j-k.

2.a)	Find the exponential Fourier series and plot the magnitude and phase spectrum of the following triangular wave form.

b)	State different properties of Fourier series.

3.a)	Distinguish between Fourier series and Fourier transform.

b)	State the conditions for the existence of Fourier transform of a signal.

c)	Find the Fourier transform of the signum function and plot it’s amplitude and phase spectrum.

4.a)	Explain the difference between a time invariant system and time variant system? Write some practical cases where you can find the systems. What do you understand by the filter characteristics of a linear system. Explain the condition of causality?

b)	What is the effect of under sampling.

5.a)	State and prove Parseval’s power Theorem.

b)	Compute the signal energy for x(t) = e –4 t u(t)

u(t) = 1 for  t  0

=  0 for  t 0.

Contd…..2.

Code No: RR-210402/NR-210403	             :: 2 ::				   Set No:2

6.a)	Specify two distinctly different pulse signals that have exactly the same auto correlation function.

b)	Consider a signal g(t) given by g(t)=A0+ A1Cos(2∏f1t+θ)+A2Cos(2∏f2t+θ).

i)   Determine the auto correlation function R(τ) of this signal.

ii)  What is the value of R(0).

iii) Has any information about g(t) been lost in obtaining auto correlation

function?

7.a)	Find the Laplace transform of [4e-2tcos 5t-3e-2tsin 5t]u(t) and its ROC.

b)	Find the signal x(t) that corresponds to the Laplace transform

X(s)=3s2+22s+27(s2+3s+2)(s2+2s+5)

8.a)	Explain the properties of the region of convergence of X (z).

b)	Discuss in detail about the double sided and single sided Z- transform. Correlate Laplace transform and Z-transform in their end use.

@@@@@

Code No: RR-210402/NR-210403

Set No.

3

II B.Tech. I-Semester Regular Examinations, November-2003.

SIGNALS AND SYSTEMS

(Common to Electronics and Computer Engineering, Electronics and Instrumentation Engineering, Instrumentation and Control Engineering, Electronics and Control Engineering and Electronics and Telematics)

Time: 3 hours							   Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.	Explain how the basis signals are formed from the given signals.  Use Gram-Schmidt procedure.

2.a)	Find the exponential Fourier series and plot the magnitude and phase spectrums for the full wave rectified sine wave.

b)	Derive polar Fourier series from Exponential Fourier series representation and hence prove that Dn=2|Cn|.

3.a)	Prove that Sinc(0)=1 and plot Sinc function.

b)	State and prove Parseval’s power theorem.

4.a)	The transfer function of an ideal bandpass filter is given by

H(jw)= k[Gw(w-wo)+Gw(w+wo)]exp(-jwto).

Sketch the magnitude and phase function of this transfer function. Evaluate the impulse response of this filter. Sketch this response and state whether the filter is physically realizable

b)	Derive an expression for the transfer function of an LTI system.

5.a)	State and prove Rayleigh energy theorem.

b)	Determine the signal energy and power

for  i) f(t) = e –3  t    and  ii)   f(t)  = e – 3t

Contd…..2.

Code No: RR-210402/NR-210403		:: 2 ::				Set No:3

6.a)	Let G(f) denote the Fourier Transform of real valued signal g(t) and R(τ) is its auto correlate function. Show that

∞                                   ∞

∫-∞ [dR(τ)/dτ]2dτ =4∏2 ∫-∞f2 |G(f)|4d(f).

b)	Consider an energy signal g(t) whose auto correlation function is given by R(τ) .

Show that |R(τ)| <=R(0) using Schwarz’s inequality.

7.a)	The Laplace Transform of a signal x(t) is given by X(s)=ωn2s2+2ζωns+ωn2) where n and  are the natural frequency and the damping factors respectively.  For what values of  , -1   1, can the Fourier transform be computed from the Laplace transform?

b)	Find the inverse Laplace transform of X(s)=1+e−2s3s2+2s.

8.a)	State and prove the convolution theorem of z transform.

b)	Find the first 4 terms of causal signal whose z transform is as under                       X (z) = 4−z−12−2z−1+z−2.

@@@@@

Code No: RR-210402/NR-210403

Set No.

4

II B.Tech. I-Semester Regular Examinations, November-2003.

SIGNALS AND SYSTEMS

(Common to Electronics and Computer Engineering, Electronics and Instrumentation Engineering, Instrumentation and Control Engineering, Electronics and Control Engineering and Electronics and Telematics)

Time: 3 hours							   Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Define: (i) Basis function:   (ii) ‘Norm’.

b)	What is Hilbert space?  Explain its properties in detail.

2.a)	For the following wave form find the Complex exponential Fourier series.

b)	Explain various type of Fourier series representation and compare them.

3.a)	Obtain the Fourier transform of the following functions.:

(i) Impulse function f(t)	(ii) DC Signal.		(iii) 	Unit step function.

b)	State and prove differentiation property of Fourier.

4.	Assume a signal x ( t )= 6 cos 2 (5) t sampled at 7 Hz and 14 Hz. Illustrate the effect of sampling a signal  at both frequency less than and greater than twice the highest frequency. Plot the output of the reconstruction filter.

5.a)	State and prove Rayleigh’s energy theorem.

b)	Find the total energy of the Sinc pulse(A Sinc(2wt)).

6.a)	Let R12(τ) and R21(τ) denote the cross correlate function of two energy signals 	g1(t) and g2(t) .    Show that the total area under R12(τ) is defined by

∞                                  ∞                      ∞

∫-∞ R12(τ) dτ =[∫-∞g1(t)dt][∫-∞g2(t)dt]*.

b)	Show that R12(τ)=R21*(-τ).

Contd…..2.

Code No: RR-210402/NR-210403		:: 2 ::				Set No:4

7.a)	Show the Laplace transform of                                                                                         x(t)=e-atAcosbt+B−Aabsinbt.u(t)isX(s)=As+B(s2+2as+c)                    where c = b2 + a2 and a>0.

b)	State and prove time-differentiation property of one-sided Laplace transform.

8.a)	Cash flow between 2 group concerns occurs once each day. For a 10 day Period, with t = 0 as day one, cash flow in lakhs of rupees is represented by the discrete time signal XT(n) = {[1.6,2.3, -3.7,0.5, -1.2,5.2,1.5, -2.3, -4.3,1.0], 1 day}

Write the expression for the sequence x(n) corresponding to cash flow signal     XT (n)

b)	Explain with a simple example how convolution of 2 sequences is performed.

@@@@@