Code No:RR210503

Set No.

1

NR210502

II B.Tech., I-Semester Supplementary Examinations, November-2003

LINEAR AND DIGITAL I.C APPLICATIONS

(Common to Computer Science and Engineering, Computer Science and Information Technology, Computer Science and Systems Engineering)

Time: 3hours								Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

----

1.a)	List out the ideal characteristics of an OP-AMP.

b)	With neat block diagram explain the function of various building blocks of an

OP-AMP

c)	Draw the equivalent circuit of an OP-AMP

2.a)	List and explain the two special cases of inverting amplifiers.

b)	What is a voltage follower? What are its features and applications?

c)	Derive the expression for the output voltage of a non inverting amplifier

3.a)	Draw the schematic diagram of Wien Bridge Oscillator and derive the expression for frequency of oscillation.

b)	What are the conditions to be satisfied by a circuit to produce oscillations?

4.a)	Draw and explain the functional diagram of a 555 Timer

b)	 Explain the function of ‘reset’ pin.

5.a)	Give the block diagram of  PLL and explain about each block in detail.

b)	Define the following terms with reference to PLL

(i)	Lock range	(ii)	Capture range	(iii)	Pull-in time.

6.a)	Explain the term “ Frequency Sealing” with suitable example.

b)	Design a wide band-pass filter with fL=200 HZ, fH=1 kHZ and a passband gain=4.  Draw the frequency response and calculate ‘Q’ factor for the filter.

7.a)	What is meant by AOI Logic?  Explain with the help of example.

b)	Draw the circuit of an improved version of D.T.L  3-input Nand gate and explain its operations with the help of Truth Table.

If hFE of each transistor is 40, find FAN-OUT of the circuit.

8.a)	Draw the circuit of a Weighted Resistor DAC and obtain expression for n-bits.

b)	Sketch the Analog output voltage for the given digital input code.

c)	What are the major disadvantages in this type?

!!!!!

Code No:RR210503

Set No.

2

NR210502

II B.Tech., I-Semester Supplementary Examinations, November-2003

LINEAR AND DIGITAL I.C APPLICATIONS

(Common to Computer Science and Engineering, Computer Science and Information Technology, Computer Science and Systems Engineering)

Time: 3hours								Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

----

1.a)	What is an OP-AMP? Why it is called so?

b)	Explain the parameters that should be considered for ac and dc applications of an        OP-AMP

c)	Draw and explain the three open loop OP-AMP configurations with neat circuit diagram

2.a)	Draw the circuit diagram of a two input non inverting type summing amplifier and derive the expression for output voltage.

b)	Briefly explain why negative feedback is desirable in amplifier applications

c)	How does negative feedback affect the performance of an inverting amplifier?

3.a)	Design a saw tooth wave form generator using OP-AMP and plot the waveforms for the given specifications frequency: 5KHz, Vsat= ± 15V.

(Assume necessary data)

b)	What is the difference between a basic comparator and the Schmitt trigger?  Construct a Schmitt trigger circuit using OP-AMP and derive the threshold voltages.

4.a)	Explain the role of the basic building blocks of PLL.

- Determine the DC control voltage vc at lock if signal frequency fs = 10 KHz, VCO free running frequency is 10.66 KHz and the voltage to frequency transfer co-efficient of VCO is 6600 Hz / v.

5.a)	Draw the schematic  circuit diagram of the following and explain their working.

(i)	Analog phase detector		(ii)	VCO

Derive necessary expressions.

b)	What is their role in PLL? Explain.

6.a)	Explain the term “VSVS configuration” .Design a VCVS low pass Butterworth second order filter with a cutoff frequency of 4 kHz.  Assume necessary data in the design process.

(Contd..2)

Code No:RR210503/NR-210502			-2-				Set No:2

b)	Design a second order IGMF  bandpass filter with the following specifications: fo=500 Hz;  Gain at resonance= -5 and bandwidth=50 Hz.  Use the circuit shown below.  Assume necessary data.

7.a)	Define the terms :	(i)	positive logic	(ii)	negative logic.

b)	Show that positive logic EX-OR operation is equivalent to negative EX-NOR operations.

c)	Draw the circuit 3 input D.T.L. NAND gate and explain its operation with the help of truth-table.  How can you improve the Fan-out of the circuit?  Explain with the help of modified circuit.

8.a)	Draw the circuit of Ladder type DAC for 4 bits and derive expression for output

voltage.

- Sketch the Analog output voltage for the given digital code.

- Compare R-2R and Weight Resistor types of DAC.

!!!!!

Code No:RR210503

Set No.

3

NR210502

II B.Tech., I-Semester Supplementary Examinations, November-2003

LINEAR AND DIGITAL I.C APPLICATIONS

(Common to Computer Science and Engineering, Computer Science and Information Technology, Computer Science and Systems Engineering)

Time: 3hours								Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

----

1.a)	Why is emitter resistor RE replaced by a constant current bias circuit in differential amplifier stage of an OP-AMP?

b)	Explain why open loop configurations are not used in linear applications

c)	For an OP-AMP, PSRR=70dB(min),CMRR=105,differential mode gainAd=105. The output voltage changes by 20V in 4 microseconds. Calculate

(i)numerical value of PSRR (ii) Common mode gain (iii) Slew rate of the OP-AMP

2.a)	Define slew rate and derive the expression for it. List causes of the slew rate and explain its significance in applications

b)	Explain the difference between slew rate and transient response.

3.a)	What is a switching regulator? Draw the block diagram  of a typical switching regulator and explain its operation.

b)	What are the four types of voltage regulators? Compare the performance of these regulators.

4.a)	Draw the circuit of a PLL AM detector and explain its operation.

b)	What is the major difference between digital and analog PLLs?

5.	Explain the operation of the following blocks of PLL in detail:

(i)	Analog and Digital phase detectors

(ii)	Voltage Controlled Oscillator

(iii)	Low Pass-Filter.

Explain their role in the operation of PLL.

6.a)	Explain the design procedure (with suitable circuit diagrams) of a fourth order Butterworth lowpass filter.

b)	A certain narrow band-pass filter has been designed to meet the following specifications. fc=2kHz, Q=20 and AF=10.  What modifications are necessary in the filter circuit to change the center frequency ‘fc’ to 1 kHz, keeping the gain and bandwidth constant?

(Contd..2)

Code No: RR210503/NR-210502			-2-				  Set No:3

7.a)	Draw the schematic circuit of TTL active pull-up NAND gate and explain its operation with the help of Truth-Table.

b)	Draw the schematic circuit of CMOS NOR gate and explain its operation with the help of Truth Table.

8.a)	Give the working principle of Analog-multiplexer.  Give block diagram of a 16 input analog multiplexer using CMOS gates and explain how it works.

b)	Draw the circuit diagram of sample and hold circuit and explain its working.

!!!!!

Code No:RR210503

Set No.

4

NR210502

II B.Tech., I-Semester Supplementary Examinations, November-2003

LINEAR AND DIGITAL I.C APPLICATIONS

(Common to Computer Science and Engineering, Computer Science and Information Technology, Computer Science and Systems Engineering)

Time: 3hours								Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

----

1.a)	What are the three differential amplifier configurations?  Compare and contrast these configurations.

b)	what is a level translator circuit?  Why is it used with the cascaded differential amplifier used in OP-AMPS?

c)	Explain the term “Slew Rate” and how it affects the frequency response of

an OP-AMP?

2.a)	Explain the differences between ac and dc amplifiers

b)  	What is instrumentation amplifier? What are its features? List any three applications of instrumentation amplifier.

3 .a)	What are the advantages of adjustable voltage regulator over the fixed regulator?

b)	Explain the importance of voltage references and voltage inverters.

c)	Explain the operation of a phase detector.

4.a)	Explain the functions of each of pins in 555 timer.

- List the important features of 555 timer.

5.a)	Explain in detail the following terms with reference to PLL

(i)	Lock range	(ii)	Capture range	(iii)	Capture transient

(iv)	Pull-in time

b)	(i)Draw the internal functional diagram of NE 566 VCO and derive an expression for free running frequency.

(ii)For the given component values find the free running frequency

Control voltage Vc=10.9V, Vce=12V, R1=4.7 K and e1=1.1 nf.

6.a)	Explain the operation a delay equalizer circuit with neat sketches.  Derive an expression relating input and output voltages of the equalizer.

b)	For the all pass filter, determine the phase shift between input and output at f=2 kHZ.  To obtain a positive phase shift, what modifications are necessary in the circuit?

(Contd..2)

Code No: RR210503/NR-210502			-2-				Set No:4

7.a)	What are the desirable features of CMOS gates ?

b)	Sketch the circuit of CMOS NAND gate and verify that it satisfies the Boolean NAND equation.

c)	Define the following terms:

(i)Fan-in (ii)	Fan-out   (iii)	Standard load	 (iv) Current sink (v) Current source.

8.a)	What are the basic blocks of analog multiplexer?  Explain how the data selection

process is performed in it.

b)	Draw a sample and hold circuit and explain its operation with necessary input and output waveforms and indicate its uses.

!!!!!