Code No:RR-210803

Set No.

1

II B.Tech. I-Semester Regular Examinations November, 2003

MATERIALS AND ENERGY BALANCE

(Chemical Engineering)

Time:3hours							 	Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.	A solution with volume V(L) contains n(mol) of a solute A with a molecular weight of MA(g A/mol). In terms of V, n, and MA:

a)	What is the molar concentration of A?

b)	What is the mass concentration of A?

In terms of CA (mol A/L) and cA (g A/L):

c)	What volume of the solution contains 20 mol of A?

d)	What is the mass flow rate of A in a stream whose volumetric flow rate is 120L/h?

2.a) 	When heated to 100C and 720 mm Hg pressure 17.2 grams of N2O4 gas occupy a volume of 11450 cc. Assuming that the ideal gas law applies, calculate the percentage dissociation of N2O4 to NO2.

b) 	Calculate the density in kg/m3 at 29 in.Hg and 30C of a mixture of hydrogen and oxygen that contains 11.1% H2 by weight.

3.a)	Write short notes on

- Critical properties

- Effect of temperature on vapor pressure.

b)	What are reference substance plots? Discuss about equal-pressure

reference-substance plots and equal-temperature reference-substance plots giving examples.

4.a) 	Define saturation, partial saturation, relative saturation and percentage saturation.

b) 	A mixture of acetone vapor and nitrogen contains 14.8% acetone by volume. Calculate the relative saturation and percentage saturation of the mixture at a temperature of 20°C, and the pressure of 745 mm Hg. Vapour pressure of acetone at 20 °C is 184.8 mm Hg.

5.	CCl4 is made as follows

CS2 + 3Cl2  CCl4 + S2Cl2

The product gases are found to contain CCl4 23.3 %, S2Cl2 23.3%, CS2 21.4% and Cl2 32 % (All percentages are by volume). Calculate

a) The percentage of excess reactant used

b) The percentage of conversion

c) The weight of CCl4 produced per 100 kg of Cl2 converted

Contd..2

Code No:RR-210803				-2-				Set No.1

6.a)	Write a short note on crystal growth.

b)	A batch crystallizer holds 10000 kg of saturated solution of NaHCO3 at 60C. It is required to crystallize 500 kg of NaHCO3 from this solution. To what temperature must the solution be cooled? Solubility data:

| Temperature C | 10 | 20 | 30 | 40 | 50 | 60 |
| --- | --- | --- | --- | --- | --- | --- |
| Solubilityg NaHCO3 per 100 g water | 8.15 | 9.60 | 11.10 | 12.70 | 14.45 | 16.40 |

7.a)	How is the enthalpy of humid air calculated?

b)	Calculate the enthalpy, per kg of dry air, of air at a pressure of 1 atm and a temperature of 37.8oC with a percentage humidity of 50. Air under these conditions contains 0.0215 kg of water per kg of dry air. This corresponds to dew point of 26oC. The mean molal heat capacity of water vapor between 26 and 37.8oC is 33.56 J/g mol K and that of air between 0 and 37.8oC is 29.08J/g mol K. The heat of vaporization at 26oC is 432996J/kg.

8.a)	What is the principle formulated by Lavoisier and Laplace regarding the energy required to decompose a chemical compound into its elements?

b)	State the law of Hess. Explain its use.

c)	Calculate the heat of formation of methane from the following:

CH4 (g) + 2O2 (g) →  CO2 (g) + 2H2O (l)          ΔH = -8,90,346.832 J

C (s) + O2 (g) →  CO2 (g)                                   ΔH = -3,93,512.73 J

2H2 (g) + O2 (g) →  2H2O (l)                              ΔH = -5,71,676.65 J

&&&

Code No:RR-210803

Set No.

2

II B.Tech. I-Semester Regular Examinations November, 2003

MATERIALS AND ENERGY BALANCE

(Chemical Engineering)

Time:3hours							 	Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.	A liquid that is almost entirely water is reported to contain 125 ppb phenol (mass basis).

a)	What is the mass fraction of phenol in the liquid?

b)	How many milligrams of phenol are contained in one kilogram of the liquid?

c)	What is the approximate concentration of phenol in the liquid in g/L?

d)	What is the mole fraction of phenol in the liquid.

2.	Combustion gases having the following molal composition are passed into an evaporator at a temperature of 200C and a pressure of 743mm Hg.

Nitrogen 79.2%, Oxygen 7.2%, Carbon dioxide 13.6%.

Water is evaporated, the gases leaving at a temperature of 85C and a pressure of 740 mm Hg with the following molal composition.

Nitrogen  48.3%, Oxygen 4.4%, Carbon dioxide 8.3%, Water 39.0%.

Calculate:

a)	Volume of gases leaving the evaporator per 100 cu m entering.

b)	Weight of water evaporated per 100 cu m of gas entering.

3.a)	A solution contains 50% benzene, 30% toluene and 20% xylene by weight at a temperature of 100C. The vapors are in contact with the solution. Calculate the total pressure and the molar percentage compositions of the liquid and vapor.

| Components | VP in mm Hg at 100C | Molecular weight |
| --- | --- | --- |
| Benzene | 1340 | 78 |
| Toluene | 560 | 92 |
| Xylene | 210 | 106 |

b)	Explain Clausius-Clapeyron equation. Ether boils at 35C at one atmosphere pressure. Calculate its boiling point at 750 mm Hg of pressure, given that latent heat of vaporization is 88.4 cal/g. (R = 1.987 cal/gmol.K ; Mol.wt. of ether = 74)

4.	Ethyl ether at a temperature of 20°C exerts a vapour pressure of 442 mm Hg. Calculate the composition of a saturated mixture of nitrogen and ether vapour at a temperature of 20°C and a pressure of 745 mm Hg expressed in the following terms:

a)	percentage composition by volume.

b)  	percentage composition by weight.

c) 	kg of vapour per cubic meter of mixture.

d) 	kg of vapour per kg of vapor free gas.

e) 	kg mole of vapour per kg mole of vapour free gas.		Contd..2

Code No:RR-210803				-2-				Set No.2

5.	A batch of leather after drying weighs 500kg and contains 6% moisture. During the process of drying the leather lost 60% of its original weight. Calculate

a) The moisture content in the leather before drying expressed as kg of water per

kg of bone dry moisture free leather

b) kg of water removed per kg of bone dry leather

c) water removed expressed as percentage of the water originally present

6.a)	Explain the working of a Swenson Walker Crystalliser.

b)	A continuous vacuum crystallizer is fed with 25000 kg/hr of 35% MgSO4 solution at 80C. The crystalliser is operated at a pressure of 25 mm Hg absolute .The solution has a boiling point of 50C. Determine the yield of MgSO4.7H2O crystals. Specific heat of solution is 3 kJ/kgC. Heat of crystallization is 50 kJ/kg of MgSO4 7H2O. Solubility of MgSO4.7H2O at the crystallization temperature is 25 kg/100 kg of water.

7.	Write short notes on the following:

- Humid heat capacity of air.

b)   Latent heat of fusion and latent heat of transition.

c)   Heat capacities of solids, liquids and solutions.

8.a)	Differentiate between heat of solution and heat of solvation.

b)	Define standard integral heat of solution. How is it determined?

c)	Calculate the heat of formation of H2SO4 to form an aqueous solution containing 5 moles of water per mole of H2SO4.

ΔHf of H2SO4 (l) at 25oC and 1 atm = -8,11,319.44 J

ΔHs of H2SO4 (l) (n1 = 5.0) at 25oC = -56,902.4 J

&&&

Code No:RR-210803

Set No.

3

II B.Tech. I-Semester Regular Examinations November, 2003

MATERIALS AND ENERGY BALANCE

(Chemical Engineering)

Time:3hours							 	Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	The feed to an ammonia synthesis reactor contains 25 mole % N2 and the balance hydrogen. The flow rate of the stream is 3000 kg/h. Calculate the rate of flow of nitrogen into the reactor in kg/h?

b)	Liquid toluene is flowing through a pipe at a rate of 175 m3/h. What is the mass flow rate of this stream in kg/min? What is the molar flow rate in mol/s?

2.	In the manufacture of hydrochloric acid, a gas is obtained that contains 25% HCl and 75% air by volume. This gas is passed through an absorption system in which 98% of the HCl is removed. The gas enters the system at a temperature of 49C and a pressure of 743 mm Hg and leaves at a temperature of 27C and a pressure of 738 mm Hg.

- Calculate the volume of gas leaving per 100 cu m entering the absorption apparatus.

- Calculate the percentage composition by volume of the gases leaving the absorption apparatus.

Calculate the weight of HCl removed per 100 cu m of gas entering the absorption apparatus.

3.	Write short notes on  the following :

- Kopp’s Rule  		b)	Raoult’s and Henry’s laws

- Critical properties	d)	Reference substance plots.

4.a) 	Write the principle of wet and dry thermometry.

b) 	A telescope gas holder contain 300 m3 of gas saturated with water vapor at         26. 5°C and  a pressure of 11.2 mm Hg above atmosphere. The barometer reads 720 mm Hg.

Calculate the weight of water vapour in the gas.

Vapour pressure of H2O at 26.5 0C is 26.2 mm Hg.

5.a) 	Define limiting reactant and excess reactant

b)	A common salt solution contains 24 % NaCl and 76% water by weight. To recover 80% of the dissolved salt a part of the water is evaporated and the remaining solution is sent to a crystallizer maintained at 200C. At this temperature the solubility of NaCl in water is 36g/100 g water. Calculate

i) The weight of water water evaporated per 1000 kg of original solution		ii) The weight of the mother liquor left after crystallization

Contd..2

Code No:RR-210803				-2-				Set No.3

6.a)	Explain the drying rate curve.

b)		Two tons of bone dry granular solids are to be dried under constant drying conditions from a moisture content of 0.2 kg/kg of dry solid to a find moisture content of 0.02 kg/kg of dry solid. The material has an effective area of 0.0615 m2/kg of dry solid under the same conditions. The following rates were previously known. Calculate the time required for drying.

| MOISTURE CONTENT X KG WATER/KG DRY SOLID | 0.3 | 0.2 | 0.14 | 0.096 | 0.056 | 0.046 | 0.026 | 0.016 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RATE OF DRYING N KG/M2 HR | 1.71 | 1.71 | 1.71 | 1.46 | 1.29 | 0.88 | 0.54 | 0.376 |

7.	What is the enthalpy change of 1 kg of water from ice at 0oC to vapor at 150oC at 1 atm pressure?

Data:

Latent heat of fusion at 0oC and 1 atm = 335 J/kg

Latent heat of vaporization at 100oC and 1 atm = 2256 J/kg

Data on heat capacity:

Liquid (J/g mol K): Cp = 18.296 + 47.212 x 10-2 T – 133.88 x 10-5 T2 +

1314.2 x 10-9 T3 (where T is in K)

Vapor (J/g mol K): Cp = 33.46 + 0.6880 x10-2T + 0.7604 x 10-5 T2 – 3.593

x 10-9T3 (where T is in K)

8.a) 	If a solute forms a hydrate, how is the standard heat of solution of hydrates determined?

b)	What is the heat of hydration?

c)	Calculate the standard heat of solution of CaCl2.6H2O to form a solution containing 10 moles of water per mole of CaCl2.

ΔHf at 25oC and 1 atm are as follows:

CaCl2 = -794960 J

H2O = -285840 J

CaCl2. 6H2O = -2607259.6 J

ΔHs at 25oC, for CaCl2 (n1 = 10) = -64852 J

&&&

Code No:RR-210803

Set No.

4

II B.Tech. I-Semester Regular Examinations November, 2003

MATERIALS AND ENERGY BALANCE

(Chemical Engineering)

Time:3hours							 	Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.	A mixture is 10.0 mole% ethyl alcohol, 75.0 mole % ethyl acetate (C4H8O2) and 15.0 mole% acetic acid. Calculate the mass fractions of each compound. What is the average molecular weight of the mixture? What would be the mass (kg) of a sample containing 25.0 kmol of ethyl acetate?

2.	Nitric acid is produced in the Ostwald process by the oxidation of ammonia with air. In the first step of the process ammonia and air are mixed together and passed over a catalyst at a temperature of 700C. The following reaction takes place:

4 NH3  + 5O2  6 H2O  + 4 NO

The gases from this process are passed into towers where they are cooled, and the oxidation is completed according to the following theoretical reactions:

2 NO   + O2  2 NO2

3 NO2  + H2O  2 HNO3  + NO

a)	Calculate the volume of air to be used per 100 cu m of ammonia entering the process.

b)	Calculate the percentage composition by volume of the gases entering the catalyzer.

c)	Calculate the percentage composition by volume of the gases leaving the catalyzer, assuming that the degree of completion of the reaction is 85% and that no other decompositions takes place.

3.a) 	Write the Clausius-Clapeyron equation. State the assumptions made in its development.

b) 	Write a brief note on vapor pressure plots.

c) 	The vapor pressure of ethyl ether (molecular weight = 74) is 185 mm Hg at 0C. The latent heat of vaporization is 92.5 cal/gm at 0C. Calculate the vapor pressure at 20 and 35C.

4.a) 	Write the expressions with notation for humidity, relative humidity, molar humidity, saturation humidity and percentage humidity.

b) 	Helium contains 12% by volume of ethyl acetate. Vapor pressure of ethyl acetate at 30°C is 15.9 kPa. Calculate

(i)  The percent relative saturation.

(ii) The percent absolute saturation of the mixture at a temperature of 30°C and

pressure of 98 kPa.

Contd..2

Code No:RR-210803				-2-				Set No.4

5.a) 	Define the terms Recycle ratio and Reflux ratio

b) 	1000 kg of a sodium carbonate solution containing 25% Na2CO3 by weight is subjected to evaporative cooling. During this process 15% of the water present in the solution is evaporated. From the concentrated solution Na2CO3.10 H2O crystallizes out. Calculate how much crystals that would be produced if the solubility of Na2CO3. 10 H2O be 21.5 g/100 g water.

6.a)	Write briefly on infrared drying.

b)		A rotary dryer using counter-current flow is to be used to dry 10000 kg/hr of wet salt containing 5% water to a water content of 0.1%. The wet salt entry temperature is 27C. Ambient air at 27C DB and 17C WB will be heated to 130C in a finned tube heater using 6 bar steam as the heating medium. The heated air at 130C is then passed into the drier. The specific heat dry solid is 0.88 kJ/kg.K. Estimate the air and steam flow rates of the drier. Assume no losses to surrounding. Data: BPR = 6K. Specific heat of water = 4.2 kJ/kg.K. Specific heat of water vapor = 2.0 kJ/kg.K. Specific heat of dry air = 1.0 kJ/kg K. Latent heat of vaporization = 2400 kJ/kg. Temperature of air leaving the drier = 50C. Temperature of product leaving the drier = 70C.

7.	If air at 1649oC is to be cooled to 538oC by passing it counter currently through a moving bed of pebbles, compute how many kg of air can be cooled per kg of pebbles. The pebble temperature rises from 260 to 1093oC. The mean specific heat of the pebbles is 0.30 cal/g oC. The heat capacity of air can be expressed as

Cp = 6.386 + 1.762 x 10-3 T –0.2656 x 10-6 T2

Where T is in K and  Cp is in cal/g mol K.

8.a)	Discuss about incomplete and successive reactions.

b)	In the production of metallic manganese, 10 kg of manganese oxide, Mn3O4, are heated in an electric furnace with 3.0 kg of amorphous carbon (coke). The resulting products are found to contain 4.8 kg of manganese metal and 2.6 kg of manganous oxide, MnO, as slag. The remainder of the products consists of unconverted charge and carbon monoxide gas. Calculate the standard heat of reaction of this process for the entire furnace charge.

&&&