Code No: RR-211907/NR-211901.

Set No:

1

II B.Tech. I-Semester Regular Examination November 2003

SIGNALS AND MODULATION THEORY

( Electronics and Computer Engineering  )

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	For the half rectified wave determine the trigonometric Fourier series.

b)	State and prove time-shifting and frequency shifting properties of Fourier transform.

2.a)	Prove that the correlation and convolution functions are identical for even signals.

b)	Show that the auto-correlation function at the origin is equal to the energy of the function.

3.a)  	What is LTI system? Explain its properties.

b)  	A signal with a psd shown in fig. is applied to an ideal Low pass filter with a    bandwidth of  5 KHz . Find the psd of the filter output and the average normalized power content of the output signal.

Gx(f)

10-6 f2

-10 KHz                     10KHz

4.a)	An AM amplifier has RF output of 30 W at 80% modulation . The internal loss in the 	modulator is 5Watts. What is the unmodulated carrier power? If the percentage 	modulation is is reduced to 60% , How much output is needed from the modulator?

b) 	Explain the coherent detection of DSB-SC signal.

(Contd…2)

Code No: RR-211907/NR-211901		.. 2..				Set No: 1

5.a)	When the modulating frequency in an FM system is 400Hz and the modulating volatage is 2.4V the modulation index is 60.  Calculate the maximum deviation.  What is the modulation index when, the modulating frequency is reduced to 250Hz and the modulating voltage is simultaneously raised to 3.2V.

b)	Prove that the phase discriminator is an FM demodulator.

6.a)	Explain with block diagram the delta Modulator and Demodulator.

b)	An audio signal em(t)=3Cos(1000πt)is quantized using DM. Find the signal to quantization noise ratio assuming the sampling rate to be 8000 samples/sec.

Repeat the above problem for PCM and compare the results.

7.	Write short notes on

a)	M-ary signaling schemes

b)	DPSK.

8.a)	Briefly explain return to zero and Non return to zero transmissions.

b)	Contrast the bandwidth considerations of return to zero and Non-return to zero 	transmissions.

c)	Contrast the error detection and decoding capabilities of RZ and NRZ 	transmissions.

~ ~ ~ ~

Code No: RR-211907/NR-211901

Set No:

2

II B.Tech. I-Semester Regular Examination November 2003

SIGNALS AND MODULATION THEORY

( Electronics and Computer Engineering  )

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.	Find the trigonometric fourier series for the fully redified cosine wave of amplitude A given and hence from the trigonometric fourier series derive the exponential Fourier series.

2.a)	State and prove the relation between auto correlation and energy density spectrum.

b)	Determine and plot the auto-correlation function of    e-at.u(t).

3.a)  	Consider a LTI system with impulse function  f(t)= e-4|t| . Find the Fourier Series

representation of the output  y(t) for the following input.

x(t)= sin (4t) + cos (6t + /4)

b) 	For the system shown in find

f(t) = e-at          for       t 0;     |a| 1

=  0            other wise

Y(w) = 1/( + jw)       for     - <w<

=  0                    other wise

Find the transfer function and impulse response   of the filter

f(t)                                                                 y(t)

Filter

4.a)	Draw the spectrum of the wave V(t) = 10 Cos (2.103 t) + Cos [(2t)105]

b)	Give the effects of frequency & phase errors in synchronous detection 	corresponding to DSB-SC .

5.a)	Derive the expressions for FM and PM signals.

b)	Explain how NBFM is generated using Balanced modulator.

(Contd…2)

Code No: RR-211907/NR-211901		.. 2..				Set No: 2

6.a)	A signal to be transmitted is of the form s(t) = 10Cos(1000t) + 5Cos(1500t). The signal is quantized using Delta Modulation (i) Select an appropriate sampling rate and step-size to accomplish the transmission. (ii)  Find the signal to quantization noise ratio for your design.

b)	Explain slope over load distortion and Granular noise in DM system with appropriate waveforms.

7.a)  	What is the difference between bit rate and baud rate?  Derive the relation 	between the two for QPSK signal.

b)  	Draw QPSK transmitter circuit and explain its operation.  What is the band width 	required for transmission?

8.a)	What are the different coding techniques?

b)	Explain the Manchester coding with a neat diagram.

~ ~ ~ ~

Code No: RR-211907/NR-211901

Set No:

3

II B.Tech. I-Semester Regular Examination November 2003

SIGNALS AND MODULATION THEORY

( Electronics and Computer Engineering  )

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Derive the expressions for trigonometric Fourier series.

b)	Find the trigonometric Fourier series for the given wave form.

f(t)

A

-3     -2      -1       0       1       2       3        t

2.a)	Determine and plot the auto-correlation function of e-a|t| .

b)	Prove that auto correlation function of f(t) is maximum at the origin.

3.a)  	A square pulse is applied to a  simple RC  low pass filter network as shown. Find

the output voltage of this circuit.

R=1

C

=1f

f(t)

1                                                                         v(t)

4

b)  	Define bandwidth of a system. What is the bandwidth for distortion less 	Transmission?

4.a)	Discuss the method of generating  AM wave with square law modulator.

b) 	A DSB-SC signal (Cos m t + Cos 2 m t) Cos c t is applied to a receiver    	consisting of a  BPF, a mixer and a  LPF cascaded  in that sequence . The other

input to the mixer is  2 Cos it  & BPF is centered about i. Find the signals at

the input and also at the output of the mixer & LPF if  i = c – 3 m.

5.a)	Derive the expression for NBFM.  Compare the spectrum of this NBFM with an wave.

b)	Give the power contents of the carrier and the sidebands in angle modulated carriers.

(Contd…2)

Code No: RR-211907/NR-211901		.. 2..				Set No: 3

6.a)	Compare DPCM and DM with standard PCM.

b)	A ramp signal em(t)=αt , Constant is applied to a Delta modulator operating at a sampling rate of fs and having representation levels at 

Find the condition to be satisfied to avoid slope overload distortion. And sketch the modulator output for =aTs.

7.	Write short notes on the following

a)	8 QAM

b)	MSK

8.	What are the different waveform patterns of digital signals, explain them.

~ ~ ~ ~

Code No: RR-211907/NR-211901

Set No:

4

II B.Tech. I-Semester Regular Examination November 2003

SIGNALS AND MODULATION THEORY

( Electronics and Computer Engineering  )

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Prove that sinusoidal functions and complex exponential functions are orthogonal functions.

b)	Determine the Fourier series for the half-wave rectified give wave shown below and plot its spectrum

2.a)	Find the Fourier transform of cross-correlation of f1(t) and f2(t).

b)	Find the auto correlation function of f(t)=A cos (ct+), where A and c are constants and  is the phase (over 0 to 2).

3.a)  	What is the principle of Linear System?

b) 	The input to a Linear System along with the impulse response of the system are                                                           shown in fig. Find the output y(t) by convolution.

h(t)                                                            x(t)

1/1                                                                                    1/2

-1/1          1/1                                                                  -1/2         1/2

4.a)	Explain  Balance Modulator for AM.

b) 	Determine the maximum upper , lower & total side band power for an 	unmodulated carrier power Pc= 2000W .Determine the maximum total power (Pt) 	for an AM system.

5.a)	Describe the Indirect method of generation of FM with neat block diagram.

b)	A carrier is frequently modulated by a sinusoidal signal f(t)=12cos 2500t. Find the power carried by the carrier and the sidebands if Kf=30,000.

(Contd…2)

Code No: RR-211907/NR-211901		.. 2..				Set No: 4

6.a)	Draw the block diagram of Adaptive Delta Modulator system and explain.

b)	(i)     Plot  law compression characteristics for =255

(ii)  If mp=20Vand256 quantizing levels are employed what is the voltage between levels when there is no compression? For =255, what is the smallest and largest effective separation between levels.

7.a)  	Compare ASK and FSK binary digital modulation schemes.

b)	Write short notes on optimum receiver filter.

8.	Write short notes on

a)	Bipolar coding

b)	Manchester coding.

~ ~ ~ ~