Code No: RR-212203

Set No.

1

II B.Tech. I-Semester Regular Examinations, November-2003

TRANSDUCERS AND INSTRUMENTATION COMPONENTS

(Instrumentation and Control Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Define the terms (i) linearity (ii) Hysterisis  (iii) threshold  (iv) Resolution.

b)	A 10 KΩ variable resistance has a linearity of 0.1% and the movement of contact  arm is 320o (i) Determine the maximum position deviation in degrees and the resistance deviation in ohm. If the instrument is to be used as a potentiometer with a linear scale of 0 to 1.6 volt, determine the maximum voltage error.

2.a) 	What is a strain gauge? Name four resistance materials for use in wire and foil gauges. What are the units for strain?

b) 	Discuss the principle of operation of the strain gauge with relevant sketches.

c)	A resistance strain gauge with a gauge factor of 2 is fastened to a member,	which is subjected to a strain of 1 × 10-6. If the original resistance value of the gauge is 130, calculate the change in resistance.

3.a) 	Describe the design features of capacitive pressure transducers.

b)	Obtain expression relating the capacitance variation with pressure for the cases of

i) Thin plate diaphragm and (ii) a metal-coated membrane.

c)	Explain how a capacitive transducer can be used as a microphone and show what are the additional considerations applied while designing the same.

4.a)	Where are bimetallic elements used as temperature sensors?

b)	Explain the construction and working of bimetallic temperature transducer.

5.a)	What is the purpose of Mechanical Linkages?  Discuss its Advantages and disadvantages.  Mention its applications.

b)	Describe in detail the system which utilizer mechanical linkages.

6. a)	What are the basic requirements of control mechanism?

b)	With a block schematic the functioning of a control mechanism.

7.a)	If an additional winding in which DC flows will there be any change in the operation of a transformer-Explain.

b)	Explain the operation of a series connected magnetic amplifier with necessary circuit diagram and waveform.

8.a)	Sketch the block diagram of a servo system using two phase motor and derive its transter function.

b)	What will be the response of the system for step input.

^^^

Code No: RR-212203

Set No.

2

II B.Tech. I-Semester Regular Examinations, November-2003

TRANSDUCERS AND INSTRUMENTATION COMPONENTS

(Instrumentation and Control Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Explain with a block diagram an instrumentation system and explain the function of various blocks.

b)	The dead-zone of a certain plyrometer is 0.125 percent of the span. The calibration is 800oC to 1800oC what temperature change must occur before it is detected?

2.a) 	Describe, in brief, a piezo-resistive type strain gauge, mentioning its merits and demerits.

b) 	Name two materials, which are used to construct piezoelectric transducers.

3.a) 	Explain the phenomenon of magnetostriction and how it is used for the measurement of force.

b)	Describe the constructional features of a magnetostrictive transducer and obtain the input -output relation ship.

4.a) 	Write short notes on thin film platinum RTD.

b) 	List the materials used for resistance thermometer construction.

5.a)	Describe the principle of Rack and Pinion arrangement in a mechanical system.

b)	Describe a mechanical system which utilizes Rackand Pinion principles.

6.a)	Explain the principle under typing in pneumatic amplifier.

b)	How is proportional action achieved through pneumatic control mechanism?

7.a)	Express a relationship between various voltages of a synchro transmitter.

b)	How are synchros useful in error detection and correction in a servo control system.

8.a)	In what way a stepper motor is different from an ordinary motor.

b)	Explain the working of a variable relectance stepper motor with a neat sketch.

c)	What is step angle and what is its significance?

^^^

Code No: RR-212203

Set No.

3

II B.Tech. I-Semester Regular Examinations, November-2003

TRANSDUCERS AND INSTRUMENTATION COMPONENTS

(Instrumentation and Control Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Categorise various errors and explain the causes and suggest remedies.

b)	The true value of voltage across a resistor is 50V. The measurement find a value of 49V. Calculate (i) the absolute error (ii) the percent error (iii) the percent accuracy.

2.a) 	What is the difference between metallic & semiconductor strain gauges? Describe the principle of working & constructional details of the bonded type strain gauge.

b) 	How is the force on cantilever beam measured by using strain gauge elements? Find the output in volts/micro strain.

3.a) 	Describe the moving coil microphone system and identify the primary and secondary transducers of the arrangement.

b)	Explain the basic principles of operation of variable permeability transducers and show how the phenomenon of permeability enables measurement of certain physical quantities.

4.		Compare a total radiation pyrometer with an optical pyrometer in terms  
 	of operation and construction details.

5.a)	Discuss the merits and demerits of Chain and Friction drives.

b)	Mention application of chain and friction drives indicating the preference of one over the other in specific system.

6.a)	How is ON-OFF application of pneumatic amplifier.

b)	Show how ON-OFF control is implemented in pneumatic control scheme.

7.a)	What do you understand by a dynamic error of a synchro system.

b)	Explain how a synchro transmitt-receiver pair is useful as a control component in error correction.

8.a)	What is a multistock stepper moter?

b)	Describe the working of a multistock stepper motor with neat diagram.

c)	Mention two uses of stepper motor.

^^^

Code No: RR-212203

Set No.

4

II B.Tech. I-Semester Regular Examinations, November-2003

TRANSDUCERS AND INSTRUMENTATION COMPONENTS

(Instrumentation and Control Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Define accuracy and precisim of a measuring instruments with suitable examples.

b)	An Instrument reads the values for four different readings 4.0, 4.1, 3.9, 4.2.  What is the absolute error and mean error.

c)	An Instrument measures 4.1V, 4.1V, 4.1V and 4.1V in four readings.  Is the instrument precise explain.  If the true value is 4V what is the error?

d)	Is the precise Instrument is better than the errganeous instrument – Explain.

2.a) 	Describe in brief two types of wire wound strain gauges mentioning their typical size, resistance, maximum excitation voltage, and construction material.

b)	The resistance of a strain gauge is 120 ohm and Gf = 2. It is connected to a current sensitive Wheatstone bridge in which resistance on all arms is 120 ohm. If the input voltage is 4V and the resistance of the galvanometer is 100 ohm, calculate the detector current in μA for μ strain.

3.        	Explain the different principles of working of capacitive transducers.

4.a)	Explain how the variation of ambient temperature causes error in  
filled-in-system thermometers.

b)	Explain the method adopted to minimize them.

5.a)	Describe in detail the Geneva Mechanism.

b)	Explain the basic principle of the instrument used for Condition monitoring of any process.

6.a)	What do you understand the term pneumatic amplification?

b)	Explain the operation of pneumatic amplifier with required sketches?

c)	Mention a few applications of pneumatic amplifier.

7.a)	What does a magnetic amplifier amplify?

b)	Is magnetic amplifier an energy converter if so how?

c)	Draw the circuit diagram of a magnetic amplifier and explain how it exerts controls in a control system.

8.a)	With diagram explain the operation of a permanent magnet stepper motor.

b)	How are stepper motors superior to ordinary motors under specific requirements?

^^^