Code No: RR/NR-210102

Set No.

1

II B.Tech. I-Semester Regular Examinations, November-2003

STRENGTH OF MATERIALS-I

(Civil Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.   	State Hooke’s law. Sketch the stress- strain diagram for a ductile material like mild steel tested under tension up to destruction, marking the salient points on it. Explain the significance of each point.

2.	A flat steel bar 30 mm wide and 5 mm thick is placed between two bars of aluminum, each  30 mm wide and  8 mm thick to form a compound bar at 100C. Calculate the temperature stresses induced at  550 C  taking Es=200GPa,  Ea=67GPa,  S  =  12  10-6/ 0C  and  a  =  24    10-6/ 0C.

3.	Obtain the location & magnitude of the maximum S.F. and B. M and draw the B. M. D for a simply supported beam subjected to uniformly varying load.

w KN/m

L

4.	Define Neutral axis. Sketch the bending stress distribution across the cross section of  a rectangular beam section 230 x 400 mm subjected to 60 KNm moment.

5.	Find the forces in all members of a four-bay simply supported truss which is shown below.

Contd…2

Code No. RR/NR 210102			-2-				       Set No.1

6.	Using Macaulay’s method, calculate the mid-span deflection for a simply supported beam loaded as shown in fig.

7. 	A double riveted double cover butt joint is used to connect plates of 10mm thick. Using Unwin’s formula, determine the diameter of the rivet, rivet value, pitch and efficiency of the joint. Adopt τs=100Nmm2,  	σv=300Nmm2, σt=150Nmm2

8.a)	Define the terms

(i).  Hoop strain   (ii).  Longitudinal strain

b)	A cylindrical air receiver for a compressor is 2 m in internal diameter and made of plates 15mm thick.  If the hoop stress is not to exceed 90 N/mm2 and the longitudinal stress is not to exceed 60 N/mm2, find the maximum safe air pressure.

^^^

Code No: RR/NR-210102

Set No.

2

II B.Tech. I-Semester Regular Examinations, November-2003

STRENGTH OF MATERIALS-I

(Civil Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.  	Sketch the stress – strain diagram for a brittle material marking the salient points on it. How is this curve different from that of a ductile material? Explain the procedure of obtaining the yield stress of materials not having a well defined yield point.

2.	A thin tyre is shrunk on a wheel of 1 m dia. Find the internal dia. of the tyre, if the circumferential stress is limited to  90 N/ mm2. Find also the least temperature to which the tyre must be heated above that of the wheel, before it could be slipped on. For the tyre material   take   E  =  200  GPa  and   =  12  10-6/ 0C.

3.	A simply supported beam with overhanging ends carries transverse loads as 	shown. If wL=P, what is the ratio of a/L for which the B. M . at the middle  of the

beam will be zero.

P	P

w KN / m

2 m	L m	2 m

4.	Obtain the shear stress distribution for a rectangular cross section 230 X 400 mm subjected to a shear force of 40 KN. Calculate maximum and average shear stress.

5.	Find the forces in the members of the tress shown in figure.

Contd…2

Code No. RR/NR 210102			-2-				       Set No.2

6.	Determine the maximum deflection and deflection at C for the simply supported beam as Shown in fig:

7.	a) 	Define the efficiency  of a riveted joint.

b) 	A single riveted lap joint is used to connect plates of 12mm thick, if 20mm    diameter rivets are used at 55mm pitch, determine the strength of joint and its efficiency.

8.a)	Define the terms

(i).  Maximum shear stress    (ii).  Volumetric strain.

b)	A steel penstock of 1m. diameter and 1cm thick is subjected to 100m. head of water.  Calculate the hoop stress and longitudinal stress at the bottom of the penstock.

^^^

Code No: RR/NR-210102

Set No.

3

II B.Tech. I-Semester Regular Examinations, November-2003

STRENGTH OF MATERIALS-I

(Civil Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1. 	Explain the Following: -

(a) Elasticity and Plasticity	     (b) Ductility and Malleability 			     (c) Stress and Strain 		     (d) Limit of proportionality and Elastic Limit

2.	A steel bar ABC is fixed at its ends A and C. It consists of two parts AB and BC of same length. The area of cross – section of AB is twice that of BC. Find the thermal stresses induced if the temperature is raised by  850C. Take

E  =  200 GPa   and   =  12   10-6/ 0C.

3.	Define shear force. Write down the equation for S. F & B. M at any section and hence, prove the relation between S. F & B. M. Calculate the shear force and B.M at the section shown

X	30 KN / m

2 m

X        6 m

4.	Design the cross section for a beam acted upon by a bending moment = 50 KNm.

If width of beam is 200 mm calculate, depth.  f = 9 N/mm2.

5.	Calculate the forces in the members BC, BG, and HG of the truss shown in figure by method of sections.

Contd…2

Code No. RR/NR 210102			-2-				      Set No.3

6.	A cantilever beam as shown in figure has a rectangular cross section of 58mm wide and  d mm deep.  Find  d if the maximum deflection is not to exceed 15mm.  Take G=10 Gpa.

7. 	Define the following i) pitch	 ii) Gauge  iii) edge distance iv) lap joint  v) Butt joint  vi) efficiency of the joint  vii) rivet value   viii) gross diameter.

8.  	Derive the expressions for hoop stress, longitudinal stress, change in diameter, length and capacity of the thin cylindrical shell, when subjected to an internal pressure.

^^^

Code No: RR/NR-210102

Set No.

4

II B.Tech. I-Semester Regular Examinations, November-2003

STRENGTH OF MATERIALS-I

(Civil Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.   	Explain the following : -

(a)  Working stress			(b)  Factor of safety					( c) Volumetric strain 			 (d)  Poisson’s ratio

2.	A steel rail 10 m long is in the unstressed condition at  100 C. If the temperature of the rail is increased to 60 0 C, find the stress induced if there is no provision for the expansion of the rail. If there is a gap of 5 mm what will be the change in stress, given   E = 200 GN/ m2 &    = 12  10 –6/ 0C.

3.	Obtain the S.F.D. & B. M D and find the maximum values for the beam loaded as shown.

5 KN/m	15 KN/m

8m

4.	A beam supports a maximum bending moment of 60 KNm at a section on span.

The cross section dimensions are b = 230 mm, d = 400 mm. Calculate the

maximum bending stresses.

5. 	Determine the forces in all the members of the truss and their nature for the simply supported truss shown in figure.

Contd…2

Code No. RR/NR 210102			-2-				       Set No.4

6.	Determine the deflection of the beam at the point of application of the 300 N.m couple as shown in fig.  Use E=2x105N/sq.mm and I=200 cm4.

7. 	A double cover butt joint is used to connect plates of 16mm thick. Design the riveted joint and determine the efficiency. Sketch the joint

8.  	A cylindrical shell 3m long has 1m. internal diameter and 15mm metal thickness.  Calculate the circumferential and longitudinal stresses induced and find out the changes in the dimensions of the shell, if it is subjected to an internal pressure of 1.5 N/mm2.  Also find out the maximum shear stress. Take E = 2 X 105 N/mm2,      = 0.3.

^^^