Code No: RR/NR-210106

Set No.

1

II B.Tech. I-Semester Regular Examinations, November-2003

FLUID MECHANICS

(Civil Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a) 	Give the dimensions, S.I. units and C.G.S. units of the following fluid properties:

i)   Specific weight 	ii)  Density 	iii)  Relative density

iv)  Dynamic viscosity and                 v)   Kinematic viscosity

b) 	Starting from their definitions, obtain the dimensions of the following derived quantities in MLT system: i) Energy ii) Power iii) Pressure and iv) Surface tension.

2.	A circular plane lamina of diameter 3 m has a square hole of side 2 m at its center. It is immersed vertically in a liquid of relative density of 0.82. such that its centroid is at a depth of 3 m and one of the diagonals of the square hole is parallel to the free surface. Determine the magnitude and location of the force of the liquid on the lamina.

3.a)	What is a potential flow?

b)	Write down the continuity equation for a unidimensional unsteady state incompressible flow.

c)	The stream function for fluid flow is given by  = m tan-1 (y/x).  Sketch the streamlines and equipotential lines.

4.a)	Using momentum, energy and continuity equation for one dimensional flow derive an expression for head loss due to sudden enlargement in a pipe.

b)	The velocity distribution in a pipe is given by u = Vm(1-r/R)2 where R is radius of pipe, Vm is maximum velocity at the axis of pipe, r is any radius at which the velocity is u.  Determine energy correction factor.

5.a)	Explain how will you find the drag on a flat plate due to laminar and turbulent boundary layers.

b)	Water is flowing over a thin smooth plate of length 4.5m and width 2.5m at a velocity of 0.9 m/sec. If the boundary layer flow changes from laminar to turbulent at a Reynold number 5 x 105 find (i) the distance from leading edge upto which boundary layer is laminar   (ii) thickness of the boundary layer at the transition point and  (iii) the drag force on one side of the plate. Take viscosity of water as 0.01 poise.

Contd…2

Code No. RR/NR 210106			-2-				    Set No.1

6.a)	What is Hagen poiseuille’s equation. Derive the same for Laminar flow through pipes.

b)	What power is required per kilometer of a line to overcome the viscous resistance to the flow of glycerin through a horizontal pipe of diameter 10cm at the rate of 10 lit/sec. Take viscosity as 8 poise and kinematic viscosity as 6 stokes.

7.	Water flows through a horizontal pipe of 2m long, which tapers from a diameter of 0.2 m to 0.15 m in the length 2m. A constant discharge of 40 liters per second flows through the pipe.  Starting from the first principles find the loss of head due to friction taking f= 0.04.

8.	A venturimeter having inlet diameter 100 mm and throat diameter 25 mm is fitted in a vertical pipe, throat is 0.3 m below the inlet, for measuring the flow of petrol of specific gravity 0.78.  Pressure gauges are fitted at inlet and throat.  Taking loss of heat between inlet and throat as 30 times the velocity head at inlet, find cd of the meter and the discharge when the inlet gauge reads 274.68 KN/m2.

^^^

Code No: RR/NR-210106

Set No.

2

II B.Tech. I-Semester Regular Examinations, November-2003

FLUID MECHANICS

(Civil Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a) 	What are the density, Specific weight, Specific volume and Kinematic viscosity of a liquid in S.I. units, if its relative density is 0.804 and dynamic viscosity is 9.8 poise. Express your answer in C.G.S. units also.

b) 	A liquid occupying a volume of 0.225 m3 has a weight of 1.89 kN. What are its density, relative density, Specific weight and Specific volume?

2. 	A Trapezoidal plate of parallel sides a and 2a and height h is immersed vertically in water with one of the side lengths as horizontal and topmost. The top edge is at a depth h below the water surface. Determine the total force on one side of the plate and the location of the center of pressure.

3.a)	Find the equation of stream line passing through (1,1) if V = 3xi – 3yj.

b)	The velocity along the centerline of a nozzle of length L is given by V=2t1−x2L2, 	where V = velocity in m/S, t = time in seconds from commencement of flow, x = 	distance from inlet to a nozzle.  Find convective acceleration, local acceleration and 	total acceleration when t = 3 sec, x = 0.5 m and L = 0.8 m.

4.a)	50 /sec of water flows in a horizontal pipe which is 200 m long and the center of which is 3 m above the datum line.  The pipe tapers from 30 cm diameter to        20 cm.  If the pressure at 20 cm dia section of the pipe is 9.81 N/cm2, calculate the pressure at the other end.  Neglect losses.

b)	Define momentum and energy correction factors.

5.a)	Define displacement thickness. Derive an expression for the displacement thickness.

b)	Oil with a free stream velocity of 1.5m/sec flow over a thin plate 1.4m wide and 2.2m long. Calculate the boundary layer thickness and the shear stress at the trailing end point and determine the total surface resistance of the plate. Take specific gravity of oil is 0.80 and kinematic viscosity as 0.1 stoke.

Contd…2

Code No. RR/NR 210106			-2-				    Set No.2

6.a)	Prove that the velocity distribution for viscous flow between two parallel plates when both plates are fixed across a section is parabolic in nature. Also prove that maximum velocity is equal to half the average velocity.

b)	Water is flowing between two large parallel plates which are 2mm apart. Determine maximum velocity, the pressure drop per unit length and shear stress at walls of the plate if the average velocity is 0.4 m/sec. Take viscosity of water      as 0.01 poise.

7.	Two reservoirs whose water surfaces are at an elevation difference of H meters are connected by a pipe line, of diameter D meters and length  L meters, which is uniformly tappered all along its length such that the discharge entering it is halved before it reaches the other end. Derive an expression for the discharge Q in cumecs entering the pipe line.  Use Darcy’s formula hf = ( LQ2/600 D5).  Neglecting minor losses and start from first principles.

8.a) 	A flow from a channel is controlled by a trapezoidal notch so that the full supply discharge of 2m3/s flows over the notch at a head of 1.2m measured over the crest. At half of this head, a discharge of 0.6m3/s passes over the notch.  Assuming Cd=0.62, calculate the base width and side slopes of the notch.

^^^

Code No: RR/NR-210106

Set No.

3

II B.Tech. I-Semester Regular Examinations, November-2003

FLUID MECHANICS

(Civil Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a) 	Define Vapour pressure of a liquid. Describe its engineering significance vis-à-vis Separation and Cavitation.

b) 	Why mercury is used in Barometer to measure atmospheric pressure?

2. 	A 1.8 m diameter cylindrical tank is laid with its axis horizontal on a level ground. Each of its ends are closed by a hemispherical dome. The tank contains oil of relative density 0.9 under pressure. If a pressure guage on the top of the tank reads 22 kPa, calculate the resultant force on the spherical end.

3.a)	Explain one, two and three dimensional flows.

b)	If  = 3xy, find x and y components of velocity at (1,3) and (3,3).  Determine the discharge passing between streamlines passing through these points.

4.	Water flows at the rate of 0.71 m3/s through the horizontal pipe transition from  90 cm dia to 60 cm dia.  If the pressure intensity at the center line of 90 cm section is 9810 N/m2, what will be the center line pressure in the 60 cm section?  What force will be required to produce the change in momentum of water as it passes through the transition?

5.a)	What do you mean by boundary layer. Describe the development of boundary 	layer along a flat plate.

b)	Find the frictional drag on one side of the plate 20cm wide and 50cm long placed longitudinally in a stream of crude oil (specific gravity = 0.925, kinematic viscosity = 0.9 stoke) flowing with undisturbed velocity of 5m/sec. Also find the thickness of boundary layer and the shear stress at the trailing edge of the plate.

6.a)	Describe Reynold’s experiment with a neat sketch.  What are the outcomes of Reynolds experiment?

b)	A crude oil of viscosity 0.97 poise and relative density 0.9 is flowing through a horizontal circular pipe of diameter 10cm and of length 10m.  Calculate the difference of pressure at the two ends of the pipe, if 100 kg of the oil is collected in a tank in 30 seconds.

Contd…2

Code No. RR/NR 210106			-2-				    Set No.3

7.	If two pipes of diameter D and d and equal length L are arranged in parallel the loss of head for a flow of Q is h.  If the same pipes are arranged in series the loss of head for the same flow Q is H.  If d= 0.5D find the percentage of total flow through each pipe when placed in parallel and the ratio (H/h).  Neglect minor losses and assume’ f ’  to be constant.

8.	The discharge in a channel varies from 0.16 m3/s to 0.74m3/s. A Contracted rectangular weir is to be constructed across the channel to measure the discharge. Find the length of the weir such that the measured head will never be less than 0.16 m or greater than one third of the length of the weir.

^^^

Code No: RR/NR-210106

Set No.

4

II B.Tech. I-Semester Regular Examinations, November-2003

FLUID MECHANICS

(Civil Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a) 	Explain what do you understand by Surface tension. Give two practical situations where surface tension plays a significant role.

b) 	2.5 m3 of a liquid weighs 3390 N on Moon, where the acceleration due to gravity is 1.614 m/s2. What are its density, Specific weight and Specific gravity in S.I. units on Moon and on Earth?

2.   	Two pipes at same elevation carry liquids of relative densities 2.5 and 1.5 respectively. An inverted U-tube manometer with a liquid of relative density 0.8 as manometric liquid connected to these pipes records a deflection of 15 cm, with the pressure in the pipe carrying heavier liquid being high. Calculate the pressure difference, if the vertical height between the pipes and the manometric liquid surface in the limb connected to heavier fluid is 0.40 m. If the same pressure difference is measured using an upright U-tube mercury Manometer, What would be the corresponding deflection in the manometer?  Assume that the vertical distance between the pipes and the manometric liquid surface in the limb connected to heavier fluid is 0.50 m.

3.	In a circular pipe the velocity varies as

v=vm1−r2R2 where v is the velocity at a radial distance r from the axis of the pipe and m is the maximum flow at the axis.  Find the ratio of average velocity in the pipe to maximum velocity.

4.	A pipeline is 15 cm in diameter and is at an elevation of 100 m at section A.  At section B, it is at an elevation of 107.0 m and has a diameter of 30 cm.  When a discharge of 50 lit/sec of water is passed through this pipe, the pressure at section A is observed to be 30 KPa.  The energy loss in the pipe is 2 m.  Calculate the pressure at B when the flow is (i) from A to B, (ii) from B to A.

5.a)	Define Laminar boundary layer, turbulent boundary layer, laminar sub-layer and boundary layer thickness.

b)	A thin plate is moving in still atmospheric air at a velocity of 4m/sec. The length of the plate is 0.5m and width 0.4m. Calculate the thickness of boundary layer at the end of the plate and drag force on one side of the plate. Take density of air as 1.25 kg/m3 and Kinematic viscosity is 0.15 stokes.

Contd…2

Code No. RR/NR 210106			-2-				    Set No.4

6.a)	Derive an expression for the difference of pressure head for a given length of the two parallel plates which are fixed and through which viscous fluid is flowing.

b)	Calculate the pressure gradient along the flow, the average velocity and the discharge for an oil of viscosity 0.02 NS/m2 flowing between two stationary plates 1m wide maintained 1cm apart. The velocity midway between the plates is 2.5 m/sec.

7.	A pipe of diameter 0.1 m carries water at 15oc. If sand grains 0.75 mm in diameter are connected on the inner surface of the pipe for test purposes, at what velocity of water will the pipe wall behaves as rough boundary. Kinematic viscosity of water at 15oc is 1.14 x 10-2 stokes.

8.a)	Explain cavitation in venturimeter.

b)	A 150 mm x 75mm venturimeter with cd =0.98 is to be replaced by an orifice meter having cd=0.6.  If both the meters are to give the same differential mercury manometer reading for a discharge of 0.1 m3/second and the inlet diameter is to remain 150mm, what should be the diameter of the orifice

^^^