Code No: RR/NR-210203

Set No.

1

II B.Tech. I-Semester Regular Examinations, November-2003.

SWITCHING THEORY AND LOGIC DESIGN

(Common to Electrical and Electronics Engineering, Computer Science and Engineering, Computer Science and Systems Engineering, Computer Science and Information Technology, Electronics and Telematics, Electronics and Computer Engineering, Electronics and Control Engineering, Electronics and Communication Engineering, Electronics and Instrumentation Engineering, Bio-Medical Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	i)  Express decimal digits 0-9 in BCD code and 2-4-2-1 code.

ii) Determine which of the above codes are self complementing.

b)	i)  Convert the decimal number 96 into binary and convert it to gray code number.

ii) Convert the given gray code number to binary: 1001001011.

2.a)	Obtain the simplified expressions in sum of products for the following Boolean functions using Karnaugh-Map.

i) F(A, B,C,D)  =  (7,13, 14, 15)	ii) F(w,x,y,z) =  (2,3,12,13,14,15)

b)	Minimize the following Boolean expressions to the required no. of literals

i)  BC + AC’ +AB+BCD to four literals

ii) ABC+A’B’C +A’BC+ABC’+A’B’C’ to five literals

c)	Obtain complement and dual for the given expression (AB+BC+AC) (EF)

3.	Using the Quine-Mc Cluskey method of tabular reduction, minimize  the given combinational  single – output function f(w,x,y,z) =  m(0,1,5,7,810,14,15)

4.a)     Design  a   full-adder    with   two   half-adders   and  basic gates.

b)     Convert Excess-3 code to BCD using Full adder circuits.

5.a)	Draw the circuit diagram of J-K flip flop with NAND gates with positive edge triggering and explain its operation with the help of truth table.  How race around condition is eliminated.

b)	Realize D-latch using R-S latch.  How it is different from D-flip flop.  Draw the circuit using NAND gates and explain.

6. a)	Draw the circuit diagram of 4-bit ring counter using D-flip flops and explain its operation with the help of bit pattern.

b)	Draw the circuit diagram of 4-bit Johnson counter using D-flip flop and explain its operation with the help of bit pattern.

Contd….2.

Code No: RR/NR-210203			:: 2 ::				Set No: 1

7.	For the machine given below, find the equivalence partition and a corresponding reduced machine in standard form:

| PS | NS, Z |  |
| --- | --- | --- |
|  | X=0 | X=1 |
| A | D, 0 | H, 1 |
| B | F, 1 | C, 1 |
| C | D, 0 | F, 1 |
| D | C, 0 | E, 1 |
| E | C, 1 | D, 1 |
| F | D, 1 | D, 1 |
| G | D, 1 | C, 1 |
| H | B, 1 | A, 1 |

8.a)	Explain in detail the block diagram of ASM chart.

b)	Draw the portion of an ASM chart that specifies the conditional operation to increment register R during state T1 and transfer to state T2, if control inputs z and y are = 1 and 0 respectively.

@@@@@

Code No: RR/NR-210203

Set No.

2

II B.Tech. I-Semester Regular Examinations, November-2003.

SWITCHING THEORY AND LOGIC DESIGN

(Common to Electrical and Electronics Engineering, Computer Science and Engineering, Computer Science and Systems Engineering, Computer Science and Information Technology, Electronics and Telematics, Electronics and Computer Engineering, Electronics and Control Engineering, Electronics and Communication Engineering, Electronics and Instrumentation Engineering, Bio-Medical Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Show the weights of three different four bit Self-Complementing codes whose only negative weight is -4.

b)	Encode each of the ten decimal digits 0,1,…..9 by means of the weighted binary codes obtained in (a).

2. a)	Obtain the simplified expression in sum of products for the following Boolean functions using Karnaugh-Map.

i)	F(w,x,y,z) =   (1,3,7,11,15) + d (0, 2, 5)

ii)	F(A, B,C,D) 	= ABD+A’C’D’ +A’B+A’CD’+AB’D

b)	Show the truth table for each of the following function and find its simplest product of sums form.(POS)

- f(x,y,z) =  xy+xz

- f(x,y,z) =  x+yz

3.	Using   the    tabular method, obtain the prime implicants   of     a    four- input single-output  function f(w,x,y,z) =  m(0,2,4,5,6,7,8,9,10,11,13). Reduce the prime-implicant table and find the minimal cover of   f.

4.a)	Give   the  schematic  circuit   of    a    2-to-4   binary  decoder with an active –low  enable   input.Give  the   truth –table   for  the same.

b)	Give the   gate-level    realization   for 8:1 mux    with active-low enable input. Show how   several 8:1 muxes can be combined to make a 32-to-1 mux.

5.a)	Draw the circuit diagram of master slave J-K flip flop and explain its operation with the help of truth table.  How it is different from edge triggering.  Explain.

b)	Give the transition table for the following flip flops.

i) RS flip flop	   ii) J-K flip flop    iii) T flip-flop	iv) D flip flop and contrast

Contd….2.

Code No: RR/NR-210203			:: 2 ::				Set No: 2

6.	Design a clocked sequential circuit to detect 1111 or 0000 and it will produce an output z=1 at the end of sequence.  Overlapping is allowed.  Draw the circuit using D flip flops.

7.a) 	Convert the following Mealy machine into a corresponding Moore machine:

| PS | NS, Z |  |
| --- | --- | --- |
|  | X=0 | X=1 |
| A | C, 0 | B, 0 |
| B | A, 1 | D, 0 |
| C | B, 1 | A, 1 |
| D | D, 1 | C, 0 |

b)	Design the circuit for the above table.

8. a)	Explain in detail the mealey state diagram and ASM chart for it with an           example.

- Show that 8 exit paths in an ASM block emanating from the decision boxes that check the eight possible binary values of three control variables x,y,z.

@@@@@

Code No: RR/NR-210203

Set No.

3

II B.Tech. I-Semester Regular Examinations, November-2003.

SWITCHING THEORY AND LOGIC DESIGN

(Common to Electrical and Electronics Engineering, Computer Science and Engineering, Computer Science and Systems Engineering, Computer Science and Information Technology, Electronics and Telematics, Electronics and Computer Engineering, Electronics and Control Engineering, Electronics and Communication Engineering, Electronics and Instrumentation Engineering, Bio-Medical Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Generate 4-bit gray code directly using mirror image property.

b)	Generate Hamming code for the given 11-bit message work. 10001110101 and rewrite the entire message with Hamming code.

2.	Four persons, members of a TV panel game, each have an ON/OFF button,  that is used to record, their opinion of a certain pop record. Instead of recording individual scores, some data processing is required such that the score board shows a HIT when the majority vote is in favour of and a MISS if it is against. Provision must also be made for a TIE. From this verbal statement

- Derive the truth tables separately for HIT, MISS and TIE.

- Extract S-O-P and P-O-S for each of the three outputs.

iii)	Simplify the equation in SOP form.

3.	Use the tabulation procedure to generate the set of   prime implicants and to obtain   all    the  minimal  expressions  for the following  function :

F(w,x,y,z) = m(1,5,6,12,13,14) +  d(2,4) )

4.a)   	Write   a  note  on  ‘display  decoders’

b)   	Design a   four-to-one mux using  a   two-to-four decoder  and basic   gates.

5.a)	Compare combinational Vs sequential logic circuits.

b)	Define the following terms of a flip flop

i)  Hold time	ii)   Set up time	iii) Propagation delay time.

c)	Draw the circuit diagram of a master slave RS flip flop and explain its operation with the help of truth table.

6. a)	Compare merits and demerits of ripple and synchronous counters.

b)	Design a modulo-12 up synchronous counter using T-flip flops and draw the circuit diagram.

Contd….2.

Code No: RR/NR-210203			:: 2 ::				Set No: 3

7. a) 	Distinguish between Mealy and Moore machines

b) 	Convert the following Mealy machine into a corresponding Moore machine:

| PS | NS, Z |  |
| --- | --- | --- |
|  | X=0 | X=1 |
| A | B, 0 | E, 0 |
| B | E, 0 | D, 0 |
| C | D, 1 | A, 0 |
| D | C, 1 | E, 0 |
| E | B, 0 | D, 0 |

8.a)	Explain how the ASM chart differs from a conventional flow chart.  Using an example show the difference in interpretation.

- Obtain the ASM charts for the following state transition

i)	If x = 0, control goes from T1 to state T2, if x = 1, generate the conditional operation and go from T1 to T2.

ii)	If x = 1, control goes from T1 to T2 and then to T3, if x = 0, control goes from T1 to T3.

iii)	Start from state T1, then if xy = 00, go to T2, if xy = 01, then go to T3, if xy = 10, then go to T1, otherwise go to T3.

@@@@@

Code No: RR/NR-210203

Set No.

4

II B.Tech. I-Semester Regular Examinations, November-2003.

SWITCHING THEORY AND LOGIC DESIGN

(Common to Electrical and Electronics Engineering, Computer Science and Engineering, Computer Science and Systems Engineering, Computer Science and Information Technology, Electronics and Telematics, Electronics and Computer Engineering, Electronics and Control Engineering, Electronics and Communication Engineering, Electronics and Instrumentation Engineering, Bio-Medical Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	A person on SATURN possessing 18 fingers has a property worth (1,00,000)18.  He has 3 daughters and two sons.  He wants to distribute half the money equally to his sons and the remaining half to his daughters equally.  How much his each son and each daughter will get in Indian currency?

b)	An Indian started on an expedition to SATURN with Rs.1,00,000.  The expenditure on SATURN will be in the ratio of 1:2:7 for food, clothing and traveling.  How much he will be spending on each item in the currency of SATURN.

2.a)	Minimise the following expression using Karnaugh – map                                       f (A,B,C,D) = m (1,4,7,10, 13) +d (5,14,15)

b)     	Find the complement and dual of the function below and then reduce it to a minimum number of literals in each case. f = [(ab)’a] [(ab)’b].

3.	Use the tabulation  procedure  to generate the set of   prime implicants and to obtain   all    the  minimal  expressions  for the following  function

G(w,x,y,z)= m(0,1,4,5,6,7,9,11,15) +  d(10,14)

4.a)	Show how  a  16-to-1   mux    can be  realized   using a tree –type network  of     4-to-1 muxes.

b)	Implement the  function f(a,b,c)= a.b + b .c    using  the  4-to-1  mux.

5.a)	Define the following terms of a Flip-Flop

i)  Hold time		ii)    Set-up time		iii) Propagation delay time

b)	Draw the circuit diagram of a negative edge trigger J-K Flip-Flop with active high present and clear and explain its operation with the help of truth table.

Contd….2.

Code No: RR/NR-210203			:: 2 ::				Set No: 4

6. a)	What is the maximum clock frequency required for a 10 bit ripple counter and synchronous counter, if propagation delay time of flip flop is 10ns.

b)	When do you prefer synchronous counters.

c)	Draw the circuit diagram of 4-bit ring counter using T-flip flops and draw the corresponding timing diagrams.

7.a) 	Explain the limitations of finite – state machines.

b) 	Find the equivalence partition and a corresponding reduced machine in standard form for the machine given below:

| PS | NS, Z |  |
| --- | --- | --- |
|  | X=0 | X=1 |
| A | E, 0 | D, 1 |
| B | F, 0 | D, 0 |
| C | E, 0 | B, 1 |
| D | F, 0 | B, 0 |
| E | C, 0 | F, 1 |
| F | B, 0 | C, 0 |

8.a)	Design the state table of control and ASM control for an example of multiplexer, by the method of one flip flop per state.

b)	Explain in detail the timing considerations among the various operations with an example.

@@@@@