Code No: RR/NR-210304

Set No.

1

II B.Tech. I-Semester Regular Examinations, November-2003.

THERMODYNAMICS

(Mechanical Engineering)

Time: 3 hours							   Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Explain thermodynamic system, surroundings and universe, illustrate the same with examples.

- Distinguish between closed system, open system and isolated system with suitable examples.

2.	A reciprocating air compressor installed in a fertilizer factory takes in air at    1 bar and 20oC and delivers at 6 bar.  Calculate the work done, heat transfer and change in internal energy per kg of air compressed if the compression follows:

(a) isothermal (b) reversible adiabatic (c) polytrophic with index as 1.25.  The changes in potential and kinetic energies may be neglected.

3.a) 	Prove that  Kelvin-planck statement and Clausius statement of Second law of

thermodynamics are equivalent.

b) 	Two reversible heat engines A and B are arranged in series with A rejecting heat directly to B through an intermediate reservoir. Engine A receives 200 kJ of heat from a reservoir at 421 C, and engine B is in thermal communication with a sink at 4.4 C. If the work out put of A is twice that of B find (a) the intermediate temperature between A and B, (b) the efficiency of each engine and (c) the heat rejected to the cold sink.

4.a)	Deduce an expression for the non-flow availability for a system.

b)	A 2-kg piece of iron is heated from room temperature of 25oC to 400oC by a heat source at 600oC. What is the irreversibility in the process? Assume for iron Cp=0.450kJ/kgK.

5.a)	What is compressibility factor?  Explain the basic construction of generalized compressibility chart.

b)	Derive the equations used for computing the entropy change of an ideal gas.

6.	Two kg mole of Carbon di oxide at a pressure of 1.8 bar, 80C is mixed in a thermally insulated vessel with 3 kg-mole of Nitrogen is at equilibrium, Determine the final temperature and pressure and the change in entropy of the mixture.

Contd….2.

Code No: RR/NR-210304			:: 2 ::			       Set No:1

7.	Benzene C6 H6 burns with atmospheric air and by means of an Orsat apparatus the following percentage composition of the products of combustion is recorded.

CO2 – 10-96% CO – 0.5% O2 – 7.5% N2 – 81.04%.

Calculate on mass basis the percentage of the excess air supplied in the combustion process.

8.	An engine working on an Otto cycle has an air standard efficiency of 50 % and rejects 544 kJ/kg of air. The pressure and temperature of air at beginning of the compression are 0.1 Mpa and 600C respectively. Compute (a) the compression ratio of the engine, (b) work done per kg of air, (c) the pressure and temperature at the end of the compression, (d) maximum pressure in the cycle and (e) mean effective pressure.

@@@@@

Code No: RR/NR-210304

Set No.

2

II B.Tech. I-Semester Regular Examinations, November-2003.

THERMO DYNAMICS

(Mechanical Engineering)

Time: 3 hours							   Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Discuss the concept of continuum in thermodynamics.

b) 	What do you mean by property? Distinguish between intensive and extensive    	properties.

2.	2 m3 of hydrogen at a pressure of 1 bar and 200C is compressed isentropically to 4 bar.  The same gas is expanded isothermally to the original volume.  Finally, the gas pressure is restored to the original volume by a constant volume heat rejection process.  Determine (a) pressure, volume and temperature at each end of operation (b) the heat added during the isothermal process (c) the heat rejected during constant volume process and (d) change in internal energy during each process.  Assume R = 4.206 kJ / kg K  & Cp = 14.25 kJ / kg K.

3.a) 	State and prove Clausius inequality and hence deduce that the property entropy exist.

b) 	A cylinder contains 0.5 m3 of a gas at 0.1 MPa and 90 C. The gas is compressed to a volume of 0.125 m3. The final pressure is 800 kPa. Determine the heat transferred and the change in entropy of the gas during the process. Assume R = 0.287 kJ/kgK and Cv = 0.713 kJ/kgK.

4.a)	Derive  an expression for the Gibbs Function of a mixture of inert ideal gases.

b)	Show that on a Mollier diagram (h-s diagram) the slope of a constant pressure line increases with temperature in the superheat region.

5.a)	Steam expands isentropically in a nozzle from 10 bar, 250oC to 0.1 bar.  The steam flow rate is 1 kg/s.  Find the velocity of steam at the exit from the nozzle and the exit area of the nozzle.  Neglect the velocity of steam at the inlet to the nozzle.

b)	Explain with a diagram, the working procedure of separating and throttling calorimeter to obtain the dryness fraction.

6.	A mixture of 25% Nitrogen and 75% Hydrogen by volume is compressed  isentropically from 300 K and 100 kPa to 500 kPa in the first stage of multistage compressor in a fertilizer plant.  The compression to still higher pressure is achieved in subsequent stages after the gas mixture is passed through the intercooler. Find the temperature of the gas mixture after compression as well as the work required per unit mass of the mixture.  Also evaluate the entropy change for each gas.  Assume that the mixture behaves like an ideal gas.

Contd….2.

Code No: RR/NR-210304			:: 2 ::			Set No:2

7.	A spark ignition engine uses a hydrocarbon fuel Cm Hn. The volumetric analysis of the exhaust gas gives the following values. CO2-14.4% CO-0.4% O2-5.4%

N2-79.7% (by difference)

Calculate:

a)	The percentage of theoretical air and

b)	The composition of the fuel on mass basis.

8.a)	Draw p-v diagram and T-s diagram of an Otto cycle and derive the cycle 	efficiency and mean effective pressure?

b)	Calculate the thermal efficiency of an Otto cycle working with maximum temperature of 14000C and minimum temperature of 420C. If the compression ratio is 10 and pressure of air the beginning of the compression is 1.5 bar, then estimate mean effective pressure.

@@@@@

Code No: RR/NR-210304

Set No.

3

II B.Tech. I-Semester Regular Examinations, November-2003.

THERMO DYNAMICS

(Mechanical Engineering)

Time: 3 hours							   Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Explain the terms state, path, process and cyclic process.

b)	Discuss the macroscopic and microscopic point of view of thermodynamics.

2.	A cylinder closed at both ends contains a free piston, on one side of which is nitrogen and other side air. The initial pressure and volume of each being 1.03 bar and 0.5 m3 respectively. Both the piston and cylinder are perfectly insulated. In the cylinder on the air side of the piston there is an electric heater which is used to heat the air.  Heat is added to the air in this manner until the volume occupied by the nitrogen is 0.3 m3. The initial temperature of each gas is 50oC. Determine (a) the final temperature of air and (b) the heat supplied to air.  Assume Cp for air as 1.005 and R for air as 0.287 kj / kg K and  = 1.4 for nitrogen. Also draw the PV diagram.

3. a) 	Prove the statement “Of all engines which operate between given two thermal reservoirs the reversible engine possesses  the  maximum thermal efficiency”.

b) 	A heat engine is used to drive a heat pump. The heat transfer from the heat engine and from the heat pump are used to heat the water circulating through the radiators of a building. The efficiency of the heat engine is 27% and C.O.P. of the heat pump is 4. Evaluate the ratio of heat transfer to the circulating water to the heat transfer to the heat engine.

4.a)	Show that available energy decreases during heat transfer through finite temperature difference.

b)	A mass of 6.98 kg of air is in a vessel at 200 kPa and 27C . Heat is transferred to the air from a reservoir at 727C until the temperature of the air rises to 327C. The environment is at 100kPa, 17C . Determine (i) the initial and final availability of air and (ii) the maximum useful work associated with the process. Assume for air Cp= 1.005 kJ/kg.K and R = 0.287kJ/kg.K.

5.a)	Determine the density of steam at 100 bar and 600oC by using (i) steam table, (ii) the perfect gas equation, (iii) Van der Wall’s equation and (iv) compressibility chart.  Van der Wall’s constant a and b are 550130 Nm4/(kg-mol)2 and 0.03 m3/kg-mol.  Compressibility factor Z is 0.95.

b)	Derive the equation for work transfer for an ideal gas in a reversible isothermal process.

Contd….2.

Code No: RR/NR-210304			:: 2 ::			          Set No:3

6.a)	A Particular gas mixture has the following analysis by volume.  O2 = 20%;     N2 = 50%; H2O = 10% and CO2 = 20%.  If the gas mixture is available at     300 K and 100 KPa, calculate the partial pressures of each consituent and express the composition in mass fraction.

b)	If the gas mixture is treated with diethylamine solution, which absorbs the

CO2 and the gases are maintained at constant temperature, calculate the final pressure of the gas and express its composition by volume as well as mass fraction.

7.	Write notes an adiabatic flame temperature and explain the procedure for finding the same.

8.	The compression ratio of Diesel cycle is 16, the temperature and pressure at the beginning of the compression are 150C and 0.1 Mpa respectively. Heat is added until the temperature at the end of the constant pressure process is 14800C. Calculate (a) the cut off ratio, (b) heat supplied per kg of air, (c) the cycle efficiency and (d) mean effective pressure.

@@@@@

Code No: RR/NR-210304

Set No.

4

II B.Tech. I-Semester Regular Examinations, November-2003.

THERMO DYNAMICS

(Mechanical Engineering)

Time: 3 hours							   Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	What do you mean by reversible and irreversible processes? Give examples.

- What do you understand by cyclic and quasi static process?

2.a)	Derive an expression for heat transfer in polytropic process.

b)	A household refrigerator contains fresh food and it is closed one kwh of electric energy is consumed in cooling the food and internal energy of the system decreases by 500kJ as the temperature drops. Find the magnitude and direction of heat transfer for the process. Assume the entire refrigerator and its contents as a system.

3.a)	Prove that the change in entropy during a polytropic process is given by

s2-s1  = Cv (n- / n-1) loge (T2 / T1)

where  is ratio of specific heats and n- index of compression or expansion.

b)  	A closed system consists of 1kg of air which is initially at 1.5 bar and 67C. The volume doubles as the system undergoes a process according to the law pV 1.2 = Constant. Find the work done, Heat transfer and change in entropy.

4.a)	Deduce an expression for the availability for flow process.

b)	Nitrogen gas flows in a pipe with a velocity of 300m/s at 500kPa, 300C. what is the availability with respect to an ambient at 100 kPa, 20C ? Assume for N2 Cp= 1.042 kJ/kg.K and R = 0.297 kJ/kg.K

5. 	Assume that 20 kg of steam are required at a pressure of 600 bar and a temperature of 750oC in order to conduct a particular experiment.  Find the volume of tank by (a) ideal gas theory, (b) the compressibility chart, (c) Van der Walls equation if a = 550130 Nm4/(kg-mol)2 and b = 0.03m3/kg-mol, (d) the Mollier chart and (v) the steam tables.

6.	Atmospheric air at 20C and 40% Relative humidity enters a heating coil

whose temperature is 38C. The by – pass factor of the heating coil is 0.25. Compute the dry bulb temperature, the relative humidity and the wet bulb temperature of the air leaving the heating coil. The atmospheric air is at 1 bar.

7.	Determine the adiabatic flame temperature when gaseous n-octane C8 H18 reacts with 100% excess air at constant pressure in a steady flow process. The reactants are at the standard state.

Contd….2.

Code No: RR/NR-210304			:: 2 ::			Set No:4

8.	At the beginning of the compression process of Diesel cycle operating with compression ratio of 18, the temperature is 300 K and the pressure of 0.1 Mpa. If the cut off ratio is 2, calculate (a) the temperature and pressure at the end of each process of the cycle (b) thermal efficiency and (c) mean effective pressure.

@@@@@