Code No.RR/NR-211401

Set No.

1

II B.Tech. I-Semester Regular Examinations, November-2003

THERMODYNAMICS AND FLUID MECHANICS

(Common to Production Engineering and Mechatronics)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Define Zeroth law of thermodynamics. Explain how it is basis for the 	measurement of temperature.

b)	A mass of 8 kg gass expands with in a flexible container so that the law of 	expansion is PV1.2 = const. The initial pressure is 1000 kpa and the initial volume 	is 1m3. The final pressure is 5 kpa. If the specific internal energy of the gas 	decreases by 40 kJ/kg. Find the heat transfer in magnitude and direction.

2.a)	State and prove Carnot theorem.

b)	A reversible heat engine receives equal quantity of heat from two thermal 	reservoirs at temperatures T1 and T2, and rejects heat to a sink at T3. Presuming 	that efficiency this engine is times the efficiency of a reversible engine 	absorbing the same amount of heat only from reservoir at T1 and rejecting heat to 	sink T3, show that

α=12T3−T2T1−T3+T2T3XT1T2

3.a)	With the help of simple line diagram explain the working of Rankine cycle. Also 	give reasons why modification is required.

b)	The compression curve on the pressure-volume diagram of a gas engine working 	on ottocycle follows the polytropic law PV1.3=constant. At two points on this 	curve at 1/4th stroke and 3/4th stroke the pressures are measured to be 140 and 360 	kN/m2 respectively. Determine the efficiency of the engine.

4.a)	Show that the two statements of second law of thermo-dynamics are equilent to 	each other.

b)	A source at 1000 K is available for transfer of heat at the rate of 100kW to a 	system at 500 K. If these temperatures remain constant; determine the entropy 	production accompanying the heat transfer and the increase of available energy. 	Take ambient temperature as 300 K.

(Contd…2)

Code No. RR/NR-211401			-2-				Set No.1

5.a)	Write down SFEE and assumptions you make for the following cases.

(i) Boiler 	(ii) Compressor 	(iii) Nozzle   and 	(iv) Turbine.

b)	In a gas turbine 17 kg/s flow through and developed 14000kw of power. The 	enthalpies of gas at inlet and outlet are 1200 kJ/kg and 360kJ/kg respectively and 	velocity of gases at inlet and outlet are 60m/s and 150 m/s respectively. Calculate 	the rate at which the heat is rejected from the turbine.

6.	In a thermal power plant operating on an ideal Rankine cycle, superheated steam 	produced at 5Mpa and 500oC is fed to a turbine where it expands to the condenser 	pressure of 10 kpa. If the net power out put of the plant is to be 20 MW. 	Determine, (a) Heat added in the boiler pr kg of water   (b) Thermal efficiency of 	the cycle (c) Massflow rate of steam in kg/s and 	(d) Mass flow rate of 	cooling 	water in the condenser if the cooling water enters the condenser at 25oC and 	leaves at 35oC.

7.a)	Define the following with neat sketches.

(i) stream line	(ii) streak line	(iii) flow net.

b)	A circular plate of 4m diameter is immersed in water with its plane making an 	angle of 30 degrees with water surface and top edge is also touching water 	surface. Determine the center of pressure and tolal force.

8.a)	Explain boundary layer separation with a neat sketch.

b)	A jet of water issues from 20mm diameter fire hose at the end of which a 5mm 	diameter nozzle is fixed. If pressure at 20mm diameter section is 200kpa gauge. 	Determine force exerted by the nozzle on the flow. Assume that the flow is 	discharging freely at the nozzle end.

***     ***     ***

Code No. RR/NR-211401

Set No.

2

II B.Tech. I-Semester Regular Examinations, November-2003

THERMODYNAMICS AND FLUID MECHANICS

(Common to Production Engineering and Mechatronics)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	What do you mean by the system? Discuss types of systems with one example 	each.

b)	One kg of gas a 25oC and occupying a volume of 3m3 is heated at constant 	pressure to 200oC. Find heat added, work done and change in internal energy of 	the system.

2.a)	Explain how PMM1 is different than PMM2.

b)	A reversible engine works between three heat reservoirs A, B and C. The receives 	equal amount of heat from reservoir A and B at temperature TA and TB 	respectively and rejects heat to a reservoir C at a temperature TC. If the efficiency 	of the engine is  times the efficiency of a reversible engine operating between A 	and C only, show that TATB=21−αTATC+2α−1.

3.a)	Explain that the working of simple vapour compression refrigeration cycle with 	the help of a line diagram.

b)	The following data pertains to a compression ignition engine working an 	airstandard diesel cycle

Cylinder bore = 15 cm, 	Stroke length = 25 cm

Clearance volume = 400 cm3

Calculate the air standard efficiency of the engine if fuel injection takes place at 	constant pressure for 5 percent of the stroke.

4.a)	For a limited temperature range the specific heats of an ideal gas vary linearly 	with temperature as follows.

CP = a + kT and C = b + kT

Where a,b and k are constants. Derive an expression for the change of specific 	entropy of this gas in terms of initial and final pressures and temperatures.

b)	A temp of steel mass 8 kg at 1000 K is droped in 80 kg of oil at 300K. The 	specific heats of steel and oil are 0.5 kJ/kg K and 3.5 kJ/kg K respectively. 	Determine the change in entropy of steel, oil and universe.

5.	Air at 1.06 bar and 7oC is heated at constant volume to a temperature of 847oC. It 	is then expanded adiabatically until the pressure falls to 1.06 bar, following which 	heat is rejected at constant pressure until the temperature is brought back to 7oC. 	Show that the cycle follows the first law of thermodynamics.

(Contd…2)

Code No. RR/NR-211401			-2-				Set No.2

6.a)	With the help of p-v diagram, explain how diesel cycle is more efficient than otto 	cycle.

b)	A brayton cycle operates with air entering the compressor at 1 bar and 25oC. The 	pressure ratio across the compressor is 3 to 1, and the maximum temperature in 	the cycle is 650oC. Determine the efficiency of the cycle.

7.a)	Derive an equation of continuity for three dimensional steady flow.

b)	If the velocity field is given by u = x2-y2+x and  = -(2xy+y). Determine velocity 	potential and steam function.

8.a)	Derive Bernouilles equation and state the assumptions required.

b)	A venturi meter is to be fitted in a pipe 0.25 m diameter where the pressure head 	is 7.6m of flowing liquid and the maximum flow is 8.1m3 per minute. Find the 	least diameter of the throat to ensure that the pressure head does not become 	negative. Take k = 0.96.

***     ***     ***

Code No. RR/NR-211401

Set No.

3

II B.Tech. I-Semester Regular Examinations, November-2003

THERMODYNAMICS AND FLUID MECHANICS

(Common to Production Engineering and Mechatronics)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	State and explain first law of thermodynamics.

b)	A cylinder contains 0.5m3 of air at 10N/m2 and 90oC. The air is compressed to a 	volume of 0.125m3 the final pressure being 60 N/cm2. Determine (i) the mass of 	air   (ii) the value of index for compression  (iii) the heat rejected or rejected by 	the air during the compression.

2.a)	State the Kelvin Planck and Clausius statements of second law of thermodynamics and prove that if a system violates Kelvin Planck statement, it also violates Clausius statement.

b)	Two reversible heat engines A and B are arranged in series. A rejecting heat 	directly to B. A receives 220 kJ of energy at 427oC from a hot source while B 	rejecting energy to a sink at 5oC, of the work output of A is twice that of B find (i) 	the intermediate temperature between A and B  (ii) efficiency of each engine and   	(iii) heat rejected to the sink.

3.a)	Prove that entropy is a property.

b)	The working fluid of a Carnot engine experienced an entropy change of 1.35 kJ/K 	during the isothermal heat rejection process. If the temperature of the thermal 	energy sink is 20oC. Determine (i) the amount of heat rejected to sink (ii) the 	change in entropy of the  sink (iii) the total energy change for this process.

4.a)	Compare air standard efficiency of Otto, Diesel and Dual cycles for same 	maximum pressure and for the same heat input.

b)	An engine working on otto cycle has an air standard efficiency of 55% and rejects 	540 kJ/kg of heat. The pressure and temperature of air at the beginning of 	compression are 0.110 Mpa and 37oC respectively. Compute (i) compression ratio 	(ii) work done and (iii) mean effective pressure. Assume Cp = 1.005 kJ/kg K and

r = 1.4 for air.

5.a)	Draw a neat diagram of vapour compression refrigeration system and explain its 	working.

b)	Find the percentage decrease in ideal thermal efficiency of a Diesel engine, with a 	compression ratio of 16 by delaying the cut-off from 5 to 8.5% of the stroke. Take 	ratio of specific heats as 1.4 for air.

(Contd…2)

Code No. RR/NR-211401			-2-				Set No.3

6.a)	Differentiate between (i) liquids and gases	(ii) real fluids and ideal fluids

(iii) specific weight and specific volume of a fluid.

b)	An oil of viscosity 5 poise is used for lubrication between a shaft and sleeve. The 	diameter of the shaft is 0.5m and it rotates at 200 rpm. Calculate the power lost in 	the oil for a sleeve length of 100mm. The thickness of the oil film is 1.0mm.

c)	Distinguish between (i) Uniform and non uniform flow

(ii) Rotational and irrotational flow.

7.a)	Describe with the help of sketch the construction, operation and use of pitot static 	tube.

b)	Find the velocity of flow of an oil through a pipe when the difference of mercury 	level in a differential U-tube mamometer connected to the two tappings of pitot 	tube is 15cm. Take specific gravity of oil is 0.8 and coefficient of pitot tube as 	0.98.

8.a)	What is Eulers equation of motion? How will you obtain Bernoulli’s equation 	from it?

b)	What do you mean by separation of boundary layer? What is the effect of pressure 	gradient on boundary layer separation?

***     ***     ***

Code No. RR/NR-211401

Set No.

4

II B.Tech. I-Semester Regular Examinations, November-2003

THERMODYNAMICS AND FLUID MECHANICS

(Common to Production Engineering and Mechatronics)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Prove that the heat transferred during a polytropic process is equal to

r−nr−1 x work done.

b)	Define the following (i) State    (ii) Quasi-static process	(iii) Cycle

(iv) Thermodynamic equilibrium.

2.a)	A quantity of air undergoes a thermodynamic cycle consisting of 3 processes in 	series process 1-2: isothermal compression from 1 bar to 4.75 bar. Process 2-3: 	isobaric expansion to 390 K. Process 3-1: Adiabatic expansion to the initial state. 	(i) sketch the cycle P-V and T-S coordinates (ii) if the cycle is a power cycle 	determine its thermal efficiency.

b)	Define the internal energy of the system and prove that it is a property of the 	system.

3.a)	Show that the efficiency of a reversible heat engine is more than that of an 	irreversible engine, both operated between the same temperature limits.

b)	0.5 kg of air initially at 25oC is heated at constant pressure until the volume is 	doubled and is then heated at constant volume until the pressure is doubled. For 	the total path find the work transfer, heat transfer and change in entropy.

4.a)	Derive an expression to find out the mean effective pressure of the Diesel cycle.

b)	The compression ratio in an air-standard otto cycle is 8. At the beginning of the 	compression the pressure is 1 bar and temperature is 300 k. The transfer of heat to 	the air per cycle is 1900 kJ/kg of air calculate (i) the pressure and temperature at 	the end of each process of the cycle (ii) thermal efficieny.

5.a)	Describe the different operations of Rankine cycle. Derive also the expression for 	its efficiency.

b)	An ideal diesel engine has a diameter 150 mm and stroke 200 mm. The clearance 	volume is 10 percent of the swept volume. Determine the compression ratio and 	the air standard efficiency of the engine if the cut-off takes place at 6 percent of 	the stroke.

(Contd…2)

Code No. RR/NR-211401			-2-				Set No.4

6.a)	Define and explain Newton’s law of viscosity.

b)	State and prove the Pascal’s law.

c)	Two plates are placed at a distance of 0.15mm apart. The lower plate is fixed 	while the upper plate having surface area 1.0m2 is pulled at 0.3m/sec. Find the 	force and power required to maintain this speed if the fluid separating them is 	having viscosity 1.5 poise.

7.a)	Distinguish between (i) steady and unsteady flow	(ii) laminar and turbulent 	flow.

b)	Define the equation of continuity. Obtain an expression for continuity equation 	for a three dimensional flow.

c)	Define the terms (i) velocity potential function   (ii) stream function.

8.a)	What is a Venturi meter? Derive an expression for the discharge through a 	Venturi meter.

b)	A horizontal Venturi meter with inlet diameter 30cm and throat diameter 15cm is 	used to measure the flow of oil of specific gravity 0.8. The discharge of oil 	through Venturi meter is 50 lt/sec. Find the reading of the oil mercury differential 	manometer. Take Cd = 0.98.

c)	Define the terms drag and lift.

***     ***     ***