Code No:RR211002/NR211003

Set No.

1

II B.Tech. I-Semester Regular Examinations, November-2003

ELECTROMAGNETIC THEORY

(Electronics and Instrumentation Engineering, Electronics and Control Engineering.)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	By applying Gauss’s law to an isolated point charge q, show that Coulomb’s law  can be deduced from Guass’ law.

b)	Charge is uniformly distributed in the region –2<y<2. Use Gauss law suitably and find E at all points for which y<-2, -2<y<2, y>2.

2.a)	Discuss the various methods of solving the static field boundary value problems.

b)	Obtain by means of Laplace’s equation the potential distribution between two concentric spherical conductors separated by a single dielectric. The inner conductor of radius ‘a’ is at potential V0 and the other conductor of radius’b’ is at zero potential.

3.a)	State and explain Biot-Savart law and relate it to Amperes law. Show that the divergence  magnetic induction is always  zero.

b)	If H=x2yax+0.1xazA/m, find the current density.

4.a)	Explain  the inconsistency   of  Ampere’s   law   in time-varying  fields.

b)      The  electric vector E of  an electromagnetic  wave in free  space  is  given by  the

expressions  Ex  = Ez = 0  ;  Ey =  A cos w(t – z / c).  Using  Maxwell’s  equations

for  free-space  conditions,  determine expressions for the  components  of the

magnetic  vector H.

5.a)	Starting from the Maxwell's curl equations , derive the wave equation in Electric

field for free space.

b)	A 9 GHz plane wave is propagating in a medium with r = 2.5.  If  E = 20 V/m and the material is assumed to be loss less, find the phase constant, wave length, phase velocity, propagation constant, intrinsic impedance and the magnitude of the H field.

6.a)	Show that expression for the attenuation of a lossy dielectric medium is

( / 2) (  / )0.5

b) 	A plane wave of frequency 1 GHz is travelling in a large block of Teflon with

r =  1, r = 2.1,  = 0  mhos/m. Determine  , ,  and 

Contd…..2

Code No:RR211002/NR211003			::2::				Set.No.1

7.a)	Define and distinguish between horizontal and veritical polarizations with neat sketches.

b)	Explain the reflection of plane waves by a perfect conductor for oblique incidence, under parallel polarization.

8.a)	Define perfect dielectric, imperfect dielectric, good conductors and perfect            	conductors. Give one example for each.

b)	The magnetic field associated with a uniform plane wave propagating in the +z-

direction in a nonmagnetic (=o) material medium is given by

H = Ho e-z cos (6 x 107t - 3 z) ay A/m. Find the following:

- the instantaneous power flow across a surface of area 1 m2 in the z = 0 plane at t = 0.

- the time-average power flow across a surface of area 1 m2 in the z = 0 plane; and

- the time-average power flow across a surface of area 1 m2 in the z = 1m plane.

$$$

Code No:RR211002/NR211003

Set No.

2

II B.Tech. I-Semester Regular Examinations, November-2003

ELECTROMAGNETIC THEORY

(Electronics and Instrumentation Engineering, Electronics and Control Engineering.)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Derive an expression for the electrostatic energy due to a continuous distribution of charge.

b)	A point charge Q is situated at the center of a hollow spherical conductor of internal and external radii ‘a’ and ‘b’. What is the potential at r = 2b? What are the charge densities at r = a and at  r = b?

2.a)	State and explain the concept of uniqueness theorem.

b)	An uncharged infinite sheet of metal of thickness “t” is inserted between two          infinite conducting plates separated by a distance of “d”. Find the capacitance of  the condenser per unit area with and without metal sheet.

3.	Find the magnetic field intensity due to the presence of  a finite straight  filament conductor carrying current I using Ampere’s Law for current element: Hence establish the relations for semi-infinite and infinite wires.

4.a)	State  and prove the boundary   conditions  on the tangential   component of             time varying  E- field   and the normal  component  of time varying  B-field            at  the  surface of   discontinuity.

b)	An  a.c.  voltage source  v = V0 sin wt  is connected   across   a  parallel plate             capacitor  C.  Verify   that the displacement   current   in the capacitor   is the             same  as the  conduction   current   in the connecting  wires.

5.a)	Starting from the Maxwell's curl equations , derive the wave equation in Magnetic  field for free space.

b) 	A plane wave with frequency f = 16 GHz travels in a distilled water with                r = 1,r = 50,  = 20 mhos/m. Calculate the propagation constant, attenuation, phase factor and  intrinsic impedance.

6.a)	What is the loss tangent of a medium? Derive an expression for the same in terms

of medium constants.

b)	Compute the phase velocity, attenuation, propagation constant and intrinsic impedance for a 1 MHz plane wave in a large copper with  r = 1, r = 1,  = 5.87 x 10 7 mhos/m.

Contd…..2

Code No:RR211002/NR211003			::2::				Set.No.2

7.a)	State and explain Poynting theorem, and the significance of Poynting vector.

b)	What is the role of conductor in power flow of a transmission line.

8.a)	A uniform plane wave is normally incident from air on to a perfect dielectric with      εr = 3. If εi =10 Cos (wt – z) ay y(S)m is the incident field, find the time average power in both regians.

b)	In free space, H = 0.2 cos (t– z) az A/m. Find the total power passing through :

a circular disc of radius 5 cm on plane x = 1

$$$

Code No:RR211002/NR211003

Set No.

3

II B.Tech. I-Semester Regular Examinations, November-2003

ELECTROMAGNETIC THEORY

(Electronics and Instrumentation Engineering, Electronics and Control Engineering.)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	If the point charge Q is at the origin, find an expression for flux which crosses the portion of a sphere centered at the origin described by      .

b)	Given D = 5x2 ax+10z az  C/m2. Find the net outward flux crossing the surface of a cube 2m on an edge centered at the origin. The edges of the cube are parallel to the axis.

2.a)	Derive Poisson’s and Laplace’s equations from fundamentals.

b)	 Given the potential field V=5x2yz + ky3z.

  - Determine k so that Laplace’s equation is satisfied.

  - For this value of k, specify the direction of E at (2,1, -1) by a nitvector.8m

3.a)	Develop an expression for the magnetic field at any point on the axis  at a distance ‘h’ from the center of a circular loop of radius ‘a’ and carrying current I

b)	Define and distinguish between magnetic field intensing and fluxdensity

4.a)	State  and prove the boundary   conditions  on the tangential   component of  time varying   H-field   and the normal  component  of time varying   D-field at the  surface of   discontinuity.

b)	A square  loop   of  wire  20 cm.  by  20cm.  has  a  voltmeter (of  infinite impedance)  connected in series with one side.  Determine  the voltage indicated by the meter  when the loop  is  placed in an alternating  magnetic field,  the   maximum  intensity of which is 1  ampere per meter.  The  plane of  the  loop is              perpendicular   to the  magnetic   field ;  the frequency  is  10MHz.

5.a)	Starting from the Maxwell's curl equations , derive the wave equation in Electric          	field for free space.

b)	Given a non-magnetic material having r = 2.35,  = 10- 4  mhos/m., find the loss      	 tangent , attenuation, phase factor and intrinsic impedance at 2.5 MHz.

Contd…..2

Code No:RR211002/NR211003			::2::				Set.No.3

6.a)	What is the relation between 1/e depth of penetration and 1% depth of penetration.

b)	 For what frequency range may sea water with r = 1, r = 81,  = 4 mhos/m. be       	 considered as a good conductor. Assume that  the loss tangent > 100 for good            	 conductor.

7.a)	Give and explain a proper interpretation of the poynting vector.

b)	In free space (z<=0), a plane wave with Hi = 10 cos ( 108t - z) ax mA/m is incident

normally on a lossless medium ( = 2 o ,  = 8o) in region z>=0. Determine the expression for reflected and transmitted Hi  electric and magnetic fields

8.a)	Derive the  pointing theorem from Maxwell’s equations and explain its physical significance

b) 	A plane wave is traveling in a medium for which σ=0, εr=3,µr=1 if Epeak=5v/m find

    - Peak poynting vector

    - Average poynting vector

    - Peak value of H

    - Impedance of medium

$$$

Code No:RR211002/NR211003

Set No.

4

II B.Tech. I-Semester Regular Examinations, November-2003

ELECTROMAGNETIC THEORY

(Electronics and Instrumentation Engineering, Electronics and Control Engineering.)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Show that .E = 0 for the field of an uniform sheet charge in all the three coordinate systems.

b) 	Define divergence theorem and verify both sides of this theorem  for the volume enclosed by r = 2, z = 0, and z = 10. if D = (10 r3 / 4) ar.

2.a)	Explain clearly experimental mapping methods for electric fields.

b)	Find the voltage across each dielectric in a capacitor with an area of 1 square meter for the plates. The thickness of one dielectric is 1mm with relative permittivity of 4.The thickness of the second dielectric is 2mm with relative permittivity of 2 and is attached to the first dielectric. Applied voltage is 200V and the distance between the plates is 3mm.

3.a)	Define and establish Ampere’s circuital Law for magnetostatic fields.

b)	State and prove Stoke’s theorem.

4.a)	Derive the  equation of   continuity for  time-varying fields.

b)     Express the Maxwell’s equations in point form   and in integral form  for  sinusoidal time  variations.  Use   phasor  representation.

- Within  a  certain region  e = 10-11  F/m,  and  m = 10-5 H/m.  If Bx = 2x10-4cos105 t

sin10-3 y (Tesla)   (i) find  E,  (ii) find the total magnetic flux passing through the			surface  x = 0,  0 < y < 40m, 0 < z < 2m,  at  t =  1  ms.

5.a)	Starting from the Maxwell's curl equations , derive the wave equation in magnetic

field for free space.

b) 	Consider a material for which r = 1,r = 4, and loss tangent is 0.1 at frequency 50

MHz. Calculate conductivity, wavelength, phase velocity and intrinsic impedance.

6.a)	Define the term intrinsic impedance of the medium, hence derive an expression for the same for a conducting medium, in terms of medium constants.

b)	Determine the loss per kilometer for a plane wave propagating in certain medium

at a frequency 2.5 MHz , if r = 1, r = 12,  =  4.5 x 10-5 mhos/m for the medium.

Contd…..2

Code No:RR211002/NR211003			::2::				Set.No.4

7.a)	Explain the significance of  Poynting vector .

b)	An incident plane electromagnetic wave of 10 MHz and having Ex=3.0 V/m and traveling along positive Y-axis strikes an aluminium sheet at normal incidence. Calculate the average power densities of incident and reflected waves.

8.a)	Define and explain the significance of the terms:  Polyting vector.  Instantomeors, Average and complex pointing vectors.

b)	Obtain an expression for the power loss in a plane conductor.

$$$