Code No.RR212305

Set No.

1

II B.Tech. I-Semester Regular Examinations, November-2003

CHEMICAL AND BIOCHEMICAL THERMODYNAMICS

(Bio-Technology)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Draw a neat figure of gas thermometer and explain its working.

b) 	Explain the relations among temperature scales.

2.	Explain the Carnot refrigeration cycle.

3.	A pure fluid is described by the canonical equation of state: G=T(T) + RT Inp, where T(T) is a substance specific function of temperature. Determine, for such a fluid, expressions for V,S,H,U,Cp and Cv. These results are consistent with those for an important model of liquid-phase behavior. What is the model?

4.	Determine the fugacity coefficients for nitrogen and methane in a N2 (i)CH4

(ii) mixture at 200 K and 30 bar if the mixture contains 40 mol % N2.  Experimental virial coefficient data are as follows:

B11= -35.2; B22=  -105.0;  B12= -59.8 cm3 mol-1

5.a) 	Describe the criteria for phase equilibrium.

b) 	What is an azeotrope? How azeotropes can be separated.

6.a) 	Describe the thermodynamic criteria for a reaction to proceed in a homogenius 	system.

b)  	Derive the relationship between the equilibrium constant and the standard Gibb’s 	free energy change.

7.a) 	Describe the inter – relationship amongst the metabolism, energy and redox

processes.

b) 	Explain the concept that “ATP is the energy shuttle” in the cell.

8.a) 	Define the “ Yeild Coeffecienct “ and explain its significance in the growth of 	cells by substrate utilization.

b) 	The following is the stoichiometric equation for combustion of the cell

Pseudomonas fluorescens growing in glucose medium.

CH1.66N0.2O0.27  + 1.28  O2    CO2 + 0.1 N2 + 0.83 H2O

Assuming the heat of combustion of 104 kcal per mole O2 , calculate the heat

released by combustion of bacteria in k calculate/gram.

***     ***     ***

Code No.RR212305

Set No.

2

II B.Tech. I-Semester Regular Examinations, November-2003

CHEMICAL AND BIOCHEMICAL THERMODYNAMICS

(Bio-Technology)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a) 	Write about the thermodynamic state and state functions.

b) 	Write about the steady – state flow process.

2.	Write about the air refrigeration cycle.

3.	Estimate the change in enthalpy and entropy when liquid ammonia at 270 K is 	compressed from its saturation pressure of 381 kPa to 1,200 kPa. For saturated 	liquid ammonia at 270 K, V1 = 1.551*10-3m3kg-1, and =2.095*10-3K-1.

4.	A vessel divided into two parts by a partition, contains 4 mol of nitrogen gas at 750 C and 30 bar on one side and 2.5 mol of argon gas at 1300C and 20 bar on the other. If the partition is removed and the gases mix adiabatically and completely, what is the change in entropy?  Assume nitrogen to be an ideal gas with Cv=(5/2)R and argon to be an ideal gas with Cv=(3/2)R.

5.a)  	Explain the V-L-E concept with a T-x-y diagram

b)  	How the VLE data of a binary mixture can be calculated using equation of

state?

6.a)  	Describe  the effect of temperature on reaction equilibrium constant.

b)  	Is the following reaction promising as a method of manufacture for sodium

formate.

NaOH(s) + CO(g)           HCOONa (s).

The following are the data on  Fo

(Contd…2)

Code No.RR212305				-2-			Set No.2

7.a)  	Explain the Gaden classification from stoichiometric point of view, the

product formation in fermentation processes.

b)  	The following stoichiometric equation describes penicillin systhesis :

1.5 Glucose  +  H2SO4  + 2NH3 + phenyl acetate  Penicillium G + CO2 + 8H2O

The theoretical yield of penicillium is 1.2 g \ (gram of glucose ).  Find out the 	molecular weight of penicillium G.

8.      	Assuming that cells can convert two – thirds (wt / wt ) of the substrate carbon    	(alkane) to biomass, calculate the “ stoichiometric “ coefficients for hexadecane

utilization.

C16H34 + 1O2 + 2 NH3    1 (C4.4H7.3N0.86O1.2)  +  2 CO2 +  3 H2O

***     ***     ***

Code No.RR212305

Set No.

3

II B.Tech. I-Semester Regular Examinations, November-2003

CHEMICAL AND BIOCHEMICAL THERMODYNAMICS

(Bio-Technology)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a) 	Write about the phase rule for homogeneous substances.

b)	Write about heat capacity and specific heat.

2.	Compare the refrigeration cycles.

3.	Liquid isobutene is throttled through a valve from an initial state of 360 K and 	4000 kPa to a final pressure of 2,000 kPa. Estimate the temperature change and 	the entropy change of the isobutene. The specific heat of liquid isobutene at 360 	K is 2.78J.g-1.0C-1.

4.	What is the ideal work for separation of an equimolar mixture of  methane and ehtane at 1750C and 3 bar in a steady-flow process in to product streams of the pure gases at 350C and bar if T=300K?

5.a)  	Explain the V-L-E concept with a T-x-y diagram.

b)  	How the VLE data of a binary mixture can be calculated using equation of  state.

6.a) 	Describe the thermodynamic criteria for a reaction to proceed in a homogenius 	system.

b)  	Derive the relationship between the equilibrium constant and the standard Gibb’s 	free energy change.

7.a) 	Describe the inter – relationship amongst the metabolism, energy and redox

processes.

- Explain the concept that “ATP is the energy shuttle” in the cell.

(Contd…2)

Code No.RR212305				-2-			Set No.3

8.		Some microorganisms exhibit growth inhibition in the presence of excess

oxygen. Assuming that the growth dependence on oxygen can be represented by

Where  KO2 is oxygen saturation constant

KI is inhibition constant

CO2, L is the dissolved oxygen concentration

Show that the specific growth rate () reaches a maximum value ( m ) at a

dissolved oxygen concentration of

CO2,L  = [K02.KI]1/2

***     ***     ***

Code No.RR212305

Set No.

4

II B.Tech. I-Semester Regular Examinations, November-2003

CHEMICAL AND BIOCHEMICAL THERMODYNAMICS

(Bio-Technology)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.	A 1 – lb mass initially at rest falls in a vacuum under a constant gravitational force.

a)	What is its downward velocity as a function of distance from the starting point?

b)	Explain how the law of conservation of energy is involved in this problem.

2.	Write about the choice of refrigerant.

3.	One kilogram of water (V1=1,003 cm3 kg-1) in a piston/cylinder device at 250C 	and 1 bar is compressed in a mechanically reversible, isothermal process to 1,500 	bar. Determine Q,W,U,H and S given that  = 250*10-6K-1 and k = 45*10-6 	bar.

4.	What is the work required for separation of air (21 mol % oxygen and 79 mol % nitrogen) at 250C and 1 bar in a steady-flow process into product streams of pure oxygen and nitrogen, also at 250C and 1 bar, if the thermodynamic efficiency of the process is 5% and if T=300K?

5.a)  	Describe Classius – Clapeyron Equation and its importance in phase            	transition.

b)  	Describe P – X – Y  diagram for Liquid – Liquid – equilibrum.

(Contd…2)

Code No.RR212305				-2-			Set No.4

6.a)  	Describe  the effect of temperature on reaction equilibrium constant.

b)  	Is the following reaction promising as a method of manufacture for sodium

formate.

NaOH(s) + CO(g)           HCOONa (s).

The following are the data on  Fo

7.a)  	Explain the Gaden classification from stoichiometric point of view the

product formation in fermentation processes.

b)  	The following stoichiometric equation describes penicillin systhesis :

1.5 Glucose  +  H2SO4  + 2NH3 + phenyl acetate  Penicillium G + CO2 + 8H2O

The theoretical yield of penicillium is 1.2 g \ (gram of glucose ).  Find out the 	molecular weight of penicillium G.

8.a)	The growth of the yeast, Saccharomyces cerevisiae can be given by the following 	overall stoichiometry equation

Go = -167.9 MJ/kmol.

Give a schematic representation of the fermentation process.

- Humid air enriched with oxygen is prepared for gluconic acid fermentation. The air is prepared in a special humidifying chamber. 1.5 lit/h liquid water enters the chamber at the same time as dry air, and 15 g-moles/min dry oxygen gas. All the water is evaporated. The out flowing gas is found to contain 1 % (w / w ) of water. Draw and label the flow sheet for the process.

***     ***     ***