Code No.220303

Set No.

1

II B.Tech. II-Semester Supplementary Examinations, November-2003

THERMAL ENGINEERING-I

(Mechanical Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Give complete classification of i.c. engines.

b)	Explain with a neat sketch the construction details of i.c. engine mechanism and name the principal components.

2.a)	Discuss the basic requirements of combustion.

b)	What is the ignitability range of air-fuel ratios for gasoline. Describe the situation 	when the mixture is too rich or too lean.

c)	What is flame front in S.I. engine combustion?

3.a)	Discuss the characteristics of fuels desirable for the SI engines as against CI 	engines.

b)	Define (i) flash point		(ii) fire point 	(iii) cloud point and explain their 	significance.

4.	A six cylinder single acting internal combustion engine has the piston speed of 540 m / min, piston diameter 12 cm and stroke length 18 cm. While developing 60 kw BP, it gave mechanical efficiency equal to 80%. Mean effective pressure acting on the piston face is 4.603 bars.  The specific fuel consumption per BP hour is 0.3 kg.  If the calorific value of the fuel used is 42000 kj / kg, determine

(a) whether it is a two-stroke or four stroke engine (b) Thermal efficiency of the engine based on brake power.

5.a)	State the uses of compressed air in engineering

b) 	Working from first principles, derive an expression for work done on air in a 	reciprocating compressor in terms of the pressure ratio.

6.a) 	Define mechanical efficiency of a compressor.

b)	A centrifugal compressor receives air at the rate of 1400m3/min at 100kPa and 	35oC and delivers at 350kPa.  It has an isentropic efficiency of 82%.  Mechanical 	losses amounts to 2.5% of the shaft power.  Determine the power required and 	exit air temperature.

7.a)	Draw the flow diagram of vapour compression refrigeration system and analyse the thermodynamic cycle applied for it?

b)	Explain the difference between the Bell Coleman and Joule cycle of refrigeration.

8.a)	Discuss the essential properties of an ideal refrigerant?

b)	Name various psychrometric processes and show each of them on psychrometric

chart? Which of these properties is most suitable in summer?

***     ***     ***

Code No.220303

Set No.

2

II B.Tech. II-Semester Supplementary Examinations, November-2003

THERMAL ENGINEERING-I

(Mechanical Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Name the important systems required for i.c.  engine and explain any one of them.

b)	What is the need of valves in i.c. engines?

2.a)	Explain normal combustion process in a SI engine with the help of p- diagram.

b)	Discuss the effect of rate of pressure rise on engine operation.

3.a)	Explain knock in CI engines and in which stage of the combustion process does 	it occur.

b)	Explain the effect of the following operating parameters on delay period in CI	engines.

(i)  Speed 	(ii)  Air-fuel ratio	(iii)  Injection timing.

4.		An engine is required to develop 100 KW, the mechanical efficiency of the engine 	is 86% and the engine uses 55 kg/h of fuel. Due to improvement in the design and 	operating conditions there is reduction in engine friction to the extent of 4.8 kw. If 	indicated thermal efficiency remains the same, determine the saving in fuel in 	kg/h.

5.a) 	State the various methods of classification of compressors.

b) 	Working from first principles, derive an expression for work done on air in a 	reciprocating compressor in terms of the temperature difference.

6.a)	Derive an expression for the work done in two stage compressor with perfect intercooler in terms of the pressure ratio.

b)	An 8-stage axial flow compressor takes in air at 20oC at the rate of 180kg/min.  	The pressure ratio is 6 and isentripic efficiency is 0.9.  Determine the power 	required.

7.a) 	Derive the expression for COP of vapour compression refrigeration system interms of enthalpies?

b)	Explain effects of under cooling and super heating.

8.a) 	Define wet bulb and dry bulb temperature and illustrate its application with respect to air conditioning applications?

b)	How the refrigerants are named according their composition? Discuss the 	important properties of refrigerants?

***     ***     ***

Code No.220303

Set No.

3

II B.Tech. II-Semester Supplementary Examinations, November-2003

THERMAL ENGINEERING-I

(Mechanical Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Explain how an i.c. engine qualifies to be a heat engine?

b)	Why is ignition required in an i.c. engine and how does it take place in a diesel engine.

2.a)	What is flame speed in the normal combustion of S.i.engines and discuss its 	influence on combustion phenomenon.

b)	What is the effect of     (i)  Air-fuel ratio     (ii)  Engine speed and

(iii)  Compression ratio on the flame velocity.

3.a)	What is meant by surface-volume ratio of combustion chambers in CI engines 	and how does it influence the starting capacity of the engine.

b)	List out the advantages and disadvantages of turbulent combustion chambers over 	non-turbulent type.

4.		A four cylinder two stroke cycle, petrol engine, 75 mm bore by 90 mm stroke 	operates on the constant volume cycle and has a compression ratio of 6 to 1, the 	efficiency ratio being 55 %.  Calculate the indicated thermal efficiency.  When 	running at 40 rps engine develops a bmep of 360 kPa and uses 9.2 kg of fuel per 	hour of CV 44000 kj / kg.  Calculate the brake thermal efficiency, the mechanical 	efficiency and the indicated specific fuel consumption in kg / kWh.

5.a)	 Explain the terms free air delivery and clearance ratio.

b) 	Compute the work required to compress 1kg of air at 25oC and 100kPa to 10MPa 	pressure if the compression process is (i) isotheral (ii) adiabatic and

(iii) polytropic with n=1.35.

6.a) 	Draw the T_S diagram for a multistage compressor.

b) 	Discuss the merits and demerits of rotary compressors over reciprocating 	compressor.

7.a)	What are the basic components of a vapour compression refrigeration system and discuss their properties?

b)	Why is an expansion valve preferred to expansion cylinder in vapour 	compression  refrigeration  cycle.

8.a) 	A Sling psychrometer reads 450C DBT and 380C WBT. Find the humidity ratio, relative humidity, dew point temperature and enthalpy of moist air?

b)	What is evaporative cooling? Why is it used for summer comfort?

***     ***     ***

Code No.220303

Set No.

4

II B.Tech. II-Semester Supplementary Examinations, November-2003

THERMAL ENGINEERING-I

(Mechanical Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Differentiate between s.i and c.i engines.

b)	Differentiate between internal combustion and external combustion engines.

2.a)	What is ignition lag in SI engines and how does it affect performance.

b)	How does engine speed influence flame velocity and explain it with proper 	reasoning.

c)	What two rates combinedly advance the normal flame front? Explain their roles.

3.a)	Does a definite flame front develop during combustion in CI engines. If so where 	from.

b)	Why an objectionable smoke zone is encountered in CI engine operation in 	certain conditions. Explain.

c)	What is the role of ignition accelerator in CI engines. Name one ignition 	accelerator.

4.		The output of a single cylinder four stroke IC engine is measured by a rope brake 	dynamometer.  The diameter of the brake pulley is 750 mm and rope 	diameter 50mm.  The dead load on the tight side of the rope is 400 N and the 	spring balance reading is 50 N.  The bore is 150 mm and stroke is 190 mm.  The 	engine consumes 4 kg / h of fuel at rated speed of 1000 rpm.  The calorific value 	of the fuel is 44 Mj / kg.  Calculate the brake specific fuel consumption, bmep and 	the brake thermal efficiency.  If the mechanical efficiency is 80 %, calculate the 	IP, imep indicated specific fuel consumption and indicated thermal efficiency.

5.a)	 Describe with a neat sketch the working of a single acting, single stage compressor.

b)	If the air temperature at inlet to an air compressor is increased from 25oC to 45oC, 	find the percentage increase in power required for each m3 of air compressed per 	minute from 105kPa to 630kPa.

6.a)	What do you understand by polytropic efficiency in axial flow compressors.

b)	List out the advantages and disadvantages of dynamic compressions as against the positive displacement compressors.

7.a)	Compare the air refrigeration system with vapour compression refrigeration system?

b)	What are the types of mechanical refrigeration and explain them in brief.

(Contd…2)

Code No.220303				-2-			Set No.4

8.a)	Define the terms relative humidity and specific humidity and illustrate its application with respect to air conditioning?

b)	What do you understand by airconditioning and what are its applications in 	modern day living.

***     ***     ***