Code No: 221201

Set No.

1

II B.Tech. II-Semester Supplementary Examinations, November-2003

COMMUNICATION THEORY

(Computer Science and Information Technology)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a) 	Explain how a periodic function of time v(t) having a fundamental period To can be represented as an infinite sum of sinusoidal waveforms.

b) 	Find the complex fourier series expansion for the given periodic wave form.

f(t)

2

------

-----				     -------

-8

t

-4

0        4       8    sec

2.a) 	What is meant by ‘coherence’ and ‘non-coherence’? Give an example of non coherent system.

b) 	Define cross correlation. Obtain a relation between coherence and cross-correlation of two signals.

3.a) 	What is the need for modulation?  Derive the relation between the output power of an AM transmitter and depth of modulation.  Give output power in case of critical modulation.

b) 	The output RMS current of 60% modulated AM generator is 1.5 A.  To what values will this current rise if the generator is modulated additionally, by another audio wave whose modulation index is 0.7?  What will be the percentage power saving if the carrier and one of the side bands are now suppressed?

4.a)	Explain the working of Foster-Seely discriminator for FM demodulation.

b) 	Explain with the help of block diagram, the working of Armstrong method for FM generation.

5.a) 	What are the various types of noise that effect the digital communication?  Explain in detail about each of them and techniques to minimize them.

b) 	A signal has peak-to-peak variation of 8V and a bandwidth equal 5 KHz.  If the signal is converted into PCM with 32 levels, find the bit rate, bandwidth and the quantization noise power.

6.a) 	Define nyquist rate of sampling and derive the expression for it.

b) 	A bandpass signal has a center frequency fo and extends from (fo-5) KHz to   (fo+5) KHz.  If the sampling rate 25 KHz, find the range of center frequency for which the sampling satisfies nyquist rate.

(Contd…2)

Code No:221201				:: 2 ::				Set No.1

7.a)   	Define and explain the significance of the terms: i) amount of information,        (ii) average information, (iii) entropy  and (iv) information rate.  List out their units.

b)	For a binary memoryless source, emitting symbols  0  and  1  with probabilities of  p0    and    p1     respectively, evaluate the entropy and sketch the entropy function.  Explain the properties of the entropy function.

8.	A convolutional encoder has a single shift register with two stages (i.e. constraint length K = 3), three modulo 2 adders and an output multiplexer. The generator sequences of the encoder are as follows:

g(1) = (1,0,1)

g(2) = (1,1,0)

g(3) = (1,1,1)

Draw the block diagram of the encoder and find the tree diagram and the encoder output produced by the message sequence 10111….

-*-*-*-

Code No: 221201

Set No.

2

II B.Tech. II-Semester Supplementary Examinations, November-2003

COMMUNICATION THEORY

(Computer Science and Information Technology)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a) 	Discuss the correspondence between signals and vectors and hence obtain Fourier series representation of signals over a finite time duration.

b) 	Obtain the Fourier transform of given function.

f(t)

10

-T/2

0

T/2

t

2.a) 	Evaluate the auto correlation of the delta function by considering it as the            limiting form of a rectangular pulse of unit area.

b) 	Consider a signal g(t) defined by:

g(t)=Ao+A1cos(2f1t+) + A2cos(2f2t+). Determine the autocorrelation function Rg() of this signal.

3.a) 	Explain the operation of square law modulator.

b) 	The message signal is given by m(t) = 20cos 2t – volts and the carrier wave is c(t) = 50 cos 100 volts.  Find the power developed across a load of 100-ohms due to the AM wave with 75% modulation.

4.a) 	Explain with a diagram the operation of limiter circuit for FM demodulation.

b) 	Explain analytically how do the AM and narrow-band FM differ from each other. Show that the average power of FM is constant.

5.a) 	Give the block diagram of DM and explain clearly the procedure for generation and demodulation of delta modulated signals.

b) 	What are the various types of errors occur in delta modulation?  Explain how they can be minimized.

6.a) 	What is cross talk?  Explain the reasons for cross talk in sampled signals and suggest methods to minimize cross talk.

b) 	Explain clearly ideal sampling and natural sampling and derive expression for the spectrum of naturally sampled signal with pulse width z.

(Contd…2)

Code No:221201				:: 2 ::				Set No.2

7.a) 	Estimate the communication rate of a binary symmetric system in which the two symbols A and B were transmit at a rate of 1000 symbols / sec, if the system noise cause 1 % of the symbols to be received incorrectly.

b) 	The terminal of a computer is used to enter alpha – numeric data is connected to the computer through a voice grade telephone having a usable bandwidth of          3 KHz and an output S/N of 10 dB. Determine:

    - the capacity of the channel,

    - The maximum rate at which data can be transmitted from the terminal to the computer without error.

Assume that the terminal has 128 characters and that the data sent from terminal consist of independent sequences of characters with equal probability.

8.a) 	With reference to coding techniques, define and explain the following terms:      (i) systematic code (ii) parity check matrix  (iii) syndrome .

b) 	Classify the different types of error control coding techniques, and distinguish between them.

-*-*-*-

Code No: 221201

Set No.

3

II B.Tech. II-Semester Supplementary Examinations, November-2003

COMMUNICATION THEORY

(Computer Science and Information Technology)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.	Draw the transfer characteristics of an idealized low pass filter and an idealized band pass filter with pass band from f1 to f2.    Explain why these filters cannot be realized physically.

2.a) 	Determine an expression for the correlation function of a square wave having values 1 and  0 and a period T.

b) 	Obtain the power spectral density function for the given waveform.  And show that it is discrete and symmetric in frequency domain.

3.a) 	Describe principle of VSB transmission.  What are its advantages over SSB.

b) 	Explain the working of phase-shift method of suppressing the unwanted side band.

4.a) 	Explain the principle of direct method of FM generation with necessary diagrams.

b) 	Explain the balanced slope detector method for FM demodulation.

5.a) 	Define the terms: time division multiplexing and demultiflexing, synchronization and explain their importance in digital signalling.

b) 	What are the limitation of delta modulation and how they can be rectified in ADM.

6.a) 	Explain the principle of pulse time modulation and obtain relation between the magnitude of low pass signal and the pulse repetition time T and pulse width  of pulse carrier.

b) 	Give the block diagram to generate pulse position modulation and explain its working.

7.a) 	The voice frequency-modulating signal of a PCM system is quantized in 16 levels with the following probabilities,

P1 = P2 = P3 = P4 = 0.1

P5 = P6 = P7 = P8 = 0.05

P9 = P10 = P11 = P12 = 0.075

P13 = P14 = P15 = P16 = 0.025

Find the information rate taking the band limiting frequency of the modulating signal at 3 KHz.

b) 	Derive the capacity for binary symmetric channel.

(Contd…2)

Code No:221201				:: 2 ::				Set No.3

8.a)  	Write short notes on  :  Binary Symmetric Channels.

b) 	What are the essential features of error control coding schemes?   Give a comparison of error correcting and error detecting codes.

-*-*-*-

Code No: 221201

Set No.

4

II B.Tech. II-Semester Supplementary Examinations, November-2003

COMMUNICATION THEORY

(Computer Science and Information Technology)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.	Prove the following properties of the Fourier series:

a)	If the periodic function gp(t)  is  even, that is, gp(-t) = gp(t) then the Fourier co-efficient Cn is purely real and an even function of n.

b)	If gp(t)  is  odd, that is , gp(-t) = - gp(t) then the Fourier co-efficient Cn is purely imaginary and an odd function of n.				(5)

c)	If gp(t)  has half wave symmetry, that is gp(t + 12  To) = - gp(t) , where to is the period of  gp(t), then the Fourier series of such a signal consists of only odd-order terms.						(6)

2.a)	Prove that R21()=R12(-)

b)	If  v(t)= cos ot, find its Fourier tranform.

c)	Prove that  G(f)=F[R()]

3.a) 	Compare various methods of SSB.

b) 	A 400-W carrier is modulated on a depth of 75%.  Calculate the total power in the modulated wave in (i) AM (ii) DSB-SC (iii) SSB-SC.

4.a) 	Compare AM and FM and bring out the advantages of FM over AM.

b) 	When the modulating frequency in an FM system is 400 Hz and the modulating voltage in 2.4V, the modulation index is 60, calculate the maximum deviation.  What is the modulation index when the modulation frequency is reduced to       250 Hz and modulation voltage is simultaneously raised to 3.2 volts?

5.a) 	What is intersymbol interference? What are the reasons for its existence?  How it can be minimized in communication system?

b) 	Explain various ways of quantization and bring out the optimum quantization procedure for minimum quantization noise.  Derive the expressions.

6.a) 	Distinguish clearly between natural sampling.  flat top sampling and give the schematic diagrams to generate them.

b) 	A signal (t) = cos 2 fo(t) + cos 4 fo(t) + cos 6 fot is sampled with a sampling frequency of 5 fo .  Give the expression for sampled signal fs(t) and obtain the output of an ideal low pass filter with cut off frequency 2fo.  What should be the sampling frequency, low pass filter cut off frequency to restore the signal completely?

(Contd…2)

Code No:221201				:: 2 ::				Set No.4

7.a) 	Determine the capacity of a band limited Gaussian channel. Using this explain in detail, the trade off between bandwidth and signal to noise ratio.

b) 	Find the capacity of the channel described by the following noise matrix.

Y1	Y2	Y3	Y4

X1		3/8	3/8	1/8	1/8

X2		1/8	1/8	3/8	3/8

8.a) 	Distinguish between block codes and convolutional codes, and compare their performance characteristics.

b) 	Give G(P) = p3 + 0+ P + 1, obtain the parity sub matrix p of 74 Hamming-code.

-*-*-*-