Code No: 220203

Set No.

1

II B.Tech. II-Semester Supplementary Examinations, November-2003

LINEAR AND DIGITAL IC APPLICATIONS

(Common to Electrical and Electronics Engineering and

Electronics and Computer Engineering)

Time: 3 hours							Max.Marks: 80

Answer any FIVE questions

All questions carry equal marks

----

1.a)	Explain the operation of zero crossing detector using Op-Amps.

b)	Design a differentiator using Op-Amp to differentiate an input signal that varies in frequency from 1 KHz to 10 KHz.

2.a)	With the help of block diagram explain Instrumentation Amplifier in detail.

b)	Design a subtractor circuit whose output is equal to the difference between two inputs.  Use a basic differential Op-Amp configuration.

3.a)	With the help of block diagram explain 556 function generator IC.

b)	Design an astable multivibrator having an output frequency of 10 KHz with a duty cycle of 50% using 555 timer.

4.a)	For a second order low pass Butterworth filter show that fH=12πR2R3C2C3.

b)	Design a wide band-pass filter with fL=400 Hz, fH=2 KHz and pass band gain=4.  Also draw an approximate frequency response plot of the filter.

5.a)	Explain the operation of diode transistor logic NAND gate.

b)	Explain the operation of emitter coupled logic OR gate.

6.a)	Give the classification of analog multipliers.  Explain the low level and high level multiplexing.

b)	Explain in detail the succession approximation type ADC?

7.a)	Explain the application of three terminal adjustable voltage regulator.

b)	With the help of block diagram explain LF398 sample and hold IC along with wave forms.

8.	Write short notes on:

a)	Triangular wave generator using Op-Amp.

b)	Monostable multi vibrator application.

&&&

Code No: 220203

Set No.

2

II B.Tech. II-Semester Supplementary Examinations, November-2003

LINEAR AND DIGITAL IC APPLICATIONS

(Common to Electrical and Electronics Engineering and

Electronics and Computer Engineering)

Time: 3 hours							Max.Marks: 80

Answer any FIVE questions

All questions carry equal marks

----

1.a)	Explain the block diagram of Op-Amp.

b)	Explain LM370 voltage regulator with diagram.

2.a)	Explain how 555 timer is used as multivibrator.

b)	What is PLL?  Explain IC565 PLL in detail.

3.a)	Explain the operation of TTL NAND gate.

b)	What is race around condition?  How this race around condition is overcome in master slave flip-flop.

4.a)	Explain how Op-Amp used as a integrator and differentiator.

b)	Explain instrument amplifier in detail.

5.a)	Design a few multi-vibrator using 555 timer to give a 25% duty cycle waveform at 10 KHz and 5 V amplitude.  Draw the diagram with waveforms.

b)	Draw a circuit using Op-Amp which can work as adder (inverted and non-inverted) and explain how it works.

6.a)	Design a second order Active Band pass filter to transmit frequencies in the range of 2 KHZ to 12 KHz, given 2K=1.414

b)	Give the classification of filters.  Explain the frequency response of all filters.

7.a)	Explain ADC and DACS.

b)	Explain with a neat diagram the principle of operation of sample and hold circuit.

8.	Write short notes on:

a)	Voltage regulator ICs.

b)	Analog MUX.

c)	Successive approximation ADC.

&&&

Code No: 220203

Set No.

3

II B.Tech. II-Semester Supplementary Examinations, November-2003

LINEAR AND DIGITAL IC APPLICATIONS

(Common to Electrical and Electronics Engineering and

Electronics and Computer Engineering)

Time: 3 hours							Max.Marks: 80

Answer any FIVE questions

All questions carry equal marks

----

1.a)	Define parameters of Op-Amp.

(i)  CMRR    (ii)  Sleeve rate	(iii)  Off-set voltage   (iv) Large signal voltage gain.

b)	Explain the frequency response of Op-Amp.

2.a)	Explain HWR using inverting and non-inverting configuration.

b)	Explain the operation of astable multivibrator using Op-Amp.

3.a)	Explain PLL using block diagram.

b)	Explain pin configuration of 555 Timer.

4.a)	Explain 556 function generator.

b)	Explain ideal characteristics of Op-Amp.

5.a)	Explain the basic operation of latch.  Explain the SR flip-flops with Truth Tables.

b)	Convert the SR-flip flop to JK flip flop using excitation tables.

6.a)	Explain the frequency responses of all filters.

b)	Figure shows the first order Butterworth LPF that uses RC network, calculate the gain if the filter as a function of frequency.  Give gain magnitude and phase angle equations.

Contd..2

Code No:220203			-2-					Set No.3

7.a)	Compare TTL, IIL and ECL families.

b)	Explain Ramp type counter.

8.	Write short notes on:

a)	Peak – detector

b)	Applications of timer.

c)	Digital filters.

d)	Sample and hold circuit.

&&&

Code No: 220203

Set No.

4

II B.Tech. II-Semester Supplementary Examinations, November-2003

LINEAR AND DIGITAL IC APPLICATIONS

(Common to Electrical and Electronics Engineering and

Electronics and Computer Engineering)

Time: 3 hours							Max.Marks: 80

Answer any FIVE questions

All questions carry equal marks

----

1.a)	Explain the non-linear application of Op-Amp as logarthmic and anti logarthimic amplifier.

b)	Design a Integrator to integrate an I/P signal that varies in frequency from 1 KHz to 10 KHz and plot the O/P wave forms if the I/P is a sine wave of 1V peak at

1 KHz.

2.a)	Explain the terms:

(i) CMRR (ii) PSRR (iii) Thirmal drift (iv) Inverting configuration of Op-Amp.

b)	The 74IC Op-Amp having the following amplifier with R1=1K, and RF=10k, A=200000, Ri=6 M, Ro=150 , fo=5 H.  Supply voltages = 15 V

O/P Voltage Swing = 13V.

Computer the values of (a) AF closed loop voltage gain (b) fF bandwidth with feedback.

3.a)	Explain in detail the operation of IC1496 balanced modulator.

b)	Using the 555 timer, design a monostable multivibrator having an output pulse width of 100 ms.

4.a)	Explain the application of PLL in detail.

b)	Design a first order low pass Butter width filter at a cut off frequency of 1 MHz with a passband gain of 2.  Further using frequency scaling technique convert the 1 KHz cut off frequency of the low pass filter to a cut off frequency of 1.6 KHz.

5.a)	Explain in detail all the parameters of logic families.

b)	Explain the operation of basic NAND and NOR latches.

6.a)	Give the classification of ADCs?  Explain in detail basic block diagram of ADC.

b)	What is quantization error?  Explain R-ZR type DAC.

7.a)	Explain Triangular wave generator.

b)	What do you mean by sampling?

c)	Explain basic circuit for sample and hold circuit.

8.a)	Write short notes on any THREE:

a)	IC voltage regulator.		b)	Analog multiplexor.

c)	TTL Vs CMOS		d)	PLL.

&&&