Code No. 220356

# OR

II B.Tech. II Semester Supplementary Examinations, November-2003

KINEMATICS OF MACHINERY

(Common to Mechanical Engineering, Production Engineering and Mechatronics)

Time: 3 hours							     Max. Marks:70

Answer any Five questions

All questions carry equal marks

- - -

1.a) 	List different inversions of double slider brank mechanism.  Explain the function of any of them with a neat sketch.

b)	The distance between the axes of the two parallel shafts is 10-2 m. They are connected by Oldham coupling having flanges each 0.2 m diameter. Find the maximum kinetic energy of the intermediate piece when the shafts revolve at 250

rpm. The intermediate piece is 0.025 m thick and has tongues each 0.01 m wide X 0.005 m deep. The specific weight of C.I., of which the coupling is made, is 72,000 N/m3.

2.	Classify the straight-line motion mechanisms. Describe “a copied straight-line mechanism”.

3.	A quick return mechanism is shown in figure, Link 2 rotates at 20 rod/S and has a 	length of 0.15m. Draw the velocity and acceleration diagram.  Determine the 	magnitude of corili’s component of acceleration.

Contd…2.

Code No. 220253				-2-				OR

4.	A mechanism as shown in figures has the following dimensions: O1A = 60 mm; 	AB 180 mm; O2B = 100 mm; O2C = 180 mm and CD = 270 mm.  The crank      	O1C 	= 180 mm and CD = 270 mm.  The crank O1A rotates clockwise at a 	uniform speed of 120 r.p.m.  The block D moves in vertical guides.  Find by 	instantaneous 	centre method, the velocity of D and the angular velocity of CD.

5.a)	Describe the working of Davis steering gear mechanism giving neat sketch.  Derive the condition for correct steering of the above mechanism.

b)	The Distance between the steering pivots of a Davis Steering gear is 1.3m. The wheel base is 2.75 metres. What will be the inclination of the track arms to the longitudinal axis of the vehicle if it is moving in a straight path.

6.	A cam with 30mm as minimum diameter is rotating closewise at a uniform speed of 1200 r.p.m. and has to give the following motion to a roller follower 10 mm in diameter.

- Follower to complete outward stroke of 25mm during 120 of cam rotation with equal uniform acceleration and retardation.

- Follower to dwell for 60 of cam rotation.

- Follower to return to its initial position during 90 of cam rotation with equal uniform acceleration and retardation.

- Follower to dwell for the remaining 90 of cam rotation

Draw the cam profile if the axis of the roller follower passes through the axis of the cam.

Determine the maximum velocity of the follower during the outstroke and return stroke and also the uniform acceleration of the follower on the outstroke and the return stroke.

Contd…3.

Code No.220253				-3-				OR

7.	Two mating involute spur gears of 200 pressure angle have a gear ratio of 2.   	The number of teeth on the pinion is 20 and its speed is 250 rpm.   The module 	pitch of the teeth is 12 mm.   If the addendum on each wheel is such that the 	path of approach and the path of recess on each side are half the maximum 	possible length each, find (i) the addendum for pinion and gear wheel (ii) the 	length of arc of contact (iii) the maximum velocity of sliding during 	approach and recess.   Assume pinion to be driver.

8.     	In an epicyclic train an annular wheel A having 54 teeth meshes with a planet 	wheel B which gears with a sun wheel C, the wheels A and C being coaxial.  The 	wheel B is carried on a pin fixed on one end of arm P which rotates about the axis 	of the wheels A and C.  If the wheel A makes 20 rpm in a clockwise direction and 	the arm P rotates at 100 rpm in the anticlockwise direction and the wheel C has 24 	teeth, determine the speed and sense of rotation of arm P.

-x-