Code No: 221852

# OR

II B.Tech. II-Semester Supplementary Examinations November-2003.

THERMODYNAMICS AND KINEMATICS

(Metallurgy and Material Technology)

Time: 3 hours							Max. Marks: 70

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Briefly explain intensive extensive properties.

b)	Define thermodynamic equilibrium.

c)	Consider a gas enclosed in a piston-cylinder occupies a volume of 0.2 m3 at a pressure of 500 kPa.  The gas is taken to a final state where P2=100 kPa by the following two different processes. Calculate work done in each case.

(i) The volume of the gas is inversely proportional to the pressure.

(ii) The process follows the path Pvr=C.   r=1.4.

2.a)	What is the difference between an open and a flow system?

b)	Obtain an expression for the flow work.

c)	In a study flow system 50 kJ of work is done per kg of the fluid.  The values of specific volume, pressure and velocity at the inlet and exit sections are 0.4m3/kg, 6 bar, 15 m/sec and 0.6 m3/kg 1 bar, 250 m/sec respectively.  The inlet is 3m above the exit.  The heat loss from the system is 8 kJ/kg.  Calculate change in internal energy.

3.a)	Define Entropy.

b)	Briefly explain Clausius inequality.

c)	An insulated chamber of volume 2V1 is divided by a thin rigid partition into two halves.  One chamber contains an ideal gas at a temperature T1.  The other chamber is evacuated.  The partition is suddenly removed and the equilibrium is reestablished.  Determine (i) the final temperature  (ii) change in entropy.

4.a)	Define second law of thermodynamics.

b)	A mass m of liquid at temperature T1 is mixed with an equal amount of the same liquid at the temperature T2.  If the specific heat of the liquid is C show that the total change in entropy is

ΔS=2mCln(T1+T2)2T1T2

c)	One kg of super heated steam at 0.2 MPa and 200oC contained in a piston-cylinder assembly is kept at ambient conditions of 300K till the steam is condensed to saturated liquid at constant pressure.  Calculate the change in the entropy of the universe associated with this process.

Contd…..2

Code No:221852					-2-			OR

5.a)	Briefly explain the need for statistical thermodynamics.

b)	Define micro and macro state.

c)	A system consisting of a large number of distinguishable particles, has available to it for non degenerate energy levels with energies 1, 2, 3, 4 units.  Determine the fractional distribution of particles into the energy level if the average energy per particle is 1.5 units.

6.a)	Show ∂u∂TP=Cp−Pνβ and ∂u∂νP=Cpνβ−P.

b)	Derive a relation Cp-Cv=Tνβ2K.

c)	Estimate the rise in temperature if liquid water at 25oC is compressed isentropically from 0.1 MPa.  For liquid water Cp=4.2 kJ/kgoK and        =2.07x10-4K-1.

7.a)	Briefly explain fugacity and activity.

b)	A mixture of A and B with ratio of 1:1 is fed in to a reactor which is maintained at temperature T and 0.1 MPa pressure.  The following two independent reactions occur in the mixture.

A+BC+D	K1=0.4

A+2BC+E	K2=0.5067

Determine the equilibrium composition of the mixture.

8.a)	Briefly explain and derive an expression for Clapeyron equation.

b)	Carbon tetra chloride boils at 76.75oC at 101 kPa.  The latent heat of vaporization of carbon tetra chloride is 194.8 kJ/kg.  Determine the boiling point at 200 kPa.

- - -