Code No. 220152

OR

II B.Tech. II-Semester Supplementary Examinations, November-2003

ELECTRICAL SCIENCE

(Common to Civil Engineering, Mechanical Engineering Mechatronics and Production Engineering)

Time: 3 hours							       Max. Marks:70

Answer any Five questions

All questions carry equal marks

- - -

1.a)	Explain the following terms as applied to magnetic circuits:

(i) Magneto motive force  (ii)  Permeability  (iii) Reluctance  (iv) flux.

And also derive the relation between them.

- A cast iron ring with a mean diameter of 10cm and cross-sectional area of

3 sq.cm. has a radial air of 0.15cm. Find the current through the winding of 1000 turns wound uniformly over turning to produce a flux density of 0.6 wb/m2  in the air gap. Take relative permeability of iron as 238.

2.a)	State and explain Kirchhoff’s laws.

b)	Find the value of R and the current flowing in it is the network shown below if the current flowing in branch OA  is zero.

3.a)	With a neat circuit and phasor diagrams derive the relation between phase and line voltages in a 3-phase star connected system.

b)	A balanced 3-phase, star connected load of 100 kW takes a leading current of 80A, when connected across a 1100V, 50 Hz supply. Find the circuit constants of the load per phase.

4.a)	Derive the torque developed by a D.C. motor from fundamentals .

b)	A 100V series motor takes 45A when running at 750 r.p.m.  its armature resistance is 0.22 ohm. While the series field resistance is 0.13 ohm. Iron and friction losses amount to 750 W. Find shaft power and total torque.

(Contd…2)

Code No.220152			-2-					OR

5.a)	With neat circuit diagrams explain how can you determine regulation of an alternator by synchronous impedance method.

b)	A 550V, 55KVA single phase alternator has an effective resistance of 0.2 ohm. A field current of 10A produces an armature current of 200A on short circuit and an e.m.f. of 450V on open circuit. Calculate the synchronous reactance and the bulb load regulation with power factor 0.8 lagging.

6.a)	Explain the construction and principle of operation of single phase transformer.

b)	Explain the principle and operation of a three phase induction motor.

7.a)	Describe with a neat sketch, the construction and working of an induction type energy meter.

b)	Make a wiring diagram showing how two  wattmeters should be connected into a 3-phase system to measure the total power. Obtain the expressions for the meter indications from the vector diagram.

8.	Write a short notes on any THREE of the following:

a)	Star – delta transformation

b)	Representation of alternating quantity by phasors

c)	Swinburne’s test

d)	Dynamometer type wattmeters.

-x-