Code No. 220451

OR

II B.Tech II-Semester Supplementary Examinations November-2003

SWITCHING THEORY AND LOGIC DESIGN

(Common to Electronics and Communication Engineering, Electronics and Instrumentation Engineering, Electronics and Control Engineering and Electronics and Telematics)

Time: 3 hours							       Max. Marks:70

Answer any Five questions

All questions carry equal marks

- - -

1.a)	Express the following numbers in decimal : (10110.0101)2 , (16.5)16  and  (26.24)8

b)	The solution to the quadratic equation x2 – 11x + 22 = 0 is x=3 and x=6. What is the base of the numbers.

2.a)	Convert the following to binary

(i)  1011111    101	(ii)	AF3B

b)	For the decimal number from 0 to 15 write the Gray code.

3.a)	Draw a logic diagram using only two-input NAND gates to implement the following expression:

(AB + A'B') (CD' + C'D)

- Derive the circuits for a three-bit parity generator and four-bit parity checker using odd parity bit.

4.a)	Implement the following Boolean function with a multiplexer:

F(A,B,C,D) = (0,1,3,4,8,9,15)

- Construct a 4-16 line decoder with file 2-4 line decoders with enable.

5.a)	Draw the block diagram of BCD Adder and explain its operation.

b)	Explain the construction of a Johnson counter.

6.a)	The 32x6 ROM together with the 2o line is as shown in figure converts a 6-bit  binary number to its corresponding 2-digit BCD number. For example, binary 100001 converts to BCD 0110011 (decimal 33). Specify the truth table for the ROM	.

2o

I1                    D1

I2                 D2

I3    32x6     D3

I4     ROM   D4

I15               D5

D6

21                                                                                                10o

22

23

24

25

101

b)	How many  32K x 8 RAM chips are needed to provide a memory capacity of 256 bytes.

Contd…2.

Code No.220451			-2-					OR

7.	Design a sequential circuit with two D-flip flops A and B, and one input x. When x=0, the state of the circuit  remains the same. When x=1, the circuit goes through the state transitions from 00 to 01 to 11 to 10 back to 00 and repeats.

8.	Write short notes on :

a)	ASM	 chart

b)	Finite state machines

c)	Asynchronous Vs Synchronous circuits.

-x-