Code No. 220255

OR

II B.Tech. II Semester Supplementary Examinations, November- 2003

ELECTROMECHANICS-II

(Electrical and Electronics Engineering)

Time: 3 hours							        Max. Marks:70

Answer any Five questions

All question carry equal marks

- - -

1.a)	Derive an approximate regulation expression when the transformer is supplying lagging 	power factor load.

b)	A 20 KVA, 2000/200 V, 50 Hz single phase transformer gave the following test results.

O.C test:	200V,		1A,		100 watts.

S.C. test:	70V,		70V,		200 watts.

Calculate all the parameters and draw-equivalent circuit referred to HV and LV side.

2.a)	Explain in detail the parallel operation of transformer.  How the load is shared by two 	transformers.  Draw relavant circuit and vector diagram.

b)	A 11,500 Volts/2,300 Volts transformer is rated at 100 KVA as a 2-winding transformer.  	If the two windings are connected in series to form an auto-transformer, what will be the 	voltage ratio and the output?

3.a)	Derive an expression for the approximate relative weight of copper in two winding and 	auto-transformer.

b)	The efficiency of a 400 KVA, single phase transformer is 98.77%.  When delivering full 	load at 0.8 p.f. and 99.13% at half full load at UPF.  Calculate iron loss and full load 	copper loss and also calculate full load efficiency at UPF.

4.a)	Explain with a neat circuit diagram, how to obtain 2-phase balanced supply from 3-phase balanced supply using single phase transformers.

b)	Discuss the operation of Buchhobz relay with neat diagram.  Explain the function conservator tank and breather in transformer.

5.a)	Discuss the principle of production of rotating magnetic field in 3-phase induction motor.

b)	A 3-phase induction motor has 4-pole, star connected stator winding and runs on 400V, 50Hz mains. The rotor resistances and standstill reactance/phase are 0.4  and 0.6 .  The rotor to stator effective turns ratio is 0.66 calculate (i) Gross torque at 4% slip and gross mechanical power.  (ii) Maximum torque and speed at maximum torque.

6.a)	Explain how the equivalent circuit of induction motor is determined from no load and 	blocked rotor tests ? And also draw the vector diagram for the same motor.

b)	Determine approximately the starting torque of an induction motor in terms of its full 	load torque when started by means of (i) Star-delta starter and (ii) By auto-transformer 	starter with 50% taps.  The motor draws 5 times full load current when switched directly 	on line has a full load slip of 4%.

(Contd..2)

Code No. 220255					-2-			 	OR

7.a)	Discuss the crawling and cogging phenomena in 3-phase induction motor.

b)	Draw the circle diagram of a 7.46 KW, 200V, 50Hz, 3-phase slipring induction motor 	with a star connected stator and rotor, a winding ratio of unity, a stator resistance of 0.38 	ohm/phase and a rotor resistance of 0.024 ohm/phase, the following are the test results.

No-load test : 	        200V, 7.7A, 0.2 pf

Locked Rotor test : 100V, 4.6 A, 0.5 pf

Find the starting torque and slip for maximum torque.

8.a)	Discuss the construction and principle of operation of 5 charge motor ?  How do we 	control the speed of this motor?

b)	Explain various speed control methods that are used for 3-phase induction motor.  	Discuss their relative merits.

!!!!!