OR

Code No.220254

II B.Tech II-Semester Supplementary Examinations, November-2003

ELECTRICAL CIRCUITS-II

(Electrical and Electronics Engineering)

Time: 3 hours							Max. Marks:70

Answer any Five questions

All questions carry equal marks

- - -

1.a)	A balanced delta connected load of 12+i5 ohm is fed from 3 phase 3 wire 230 V 	system. Find (i) the magnitude of phase and line currents  (ii) total active power.

b)	A 3 phase balanced star connected load is connected to a 240V 3 phase supply . 	The total volt amperes are 6000 VA and the total active power is 4800 W. Find 	(a) power factor (b) line current  (c) phase current  (d) resistance and reactor of 	load in each phase.

2.	Discuss the phenomenon of neutral shift. Derive an expression for calculating the 	neutral shift voltage.

3.	Using graph theory how do you select basic loops of a network. Illustrate with 	suitable example.

4.

For the network shown above find the resistance between (i) A and a (ii) C and a.

5.	State and explain the thevioun’s theorem. Illustrate with suitable example.

Contd 2

Code No:220254					-2-			OR

6.a)	Using superposition theorem fixed the current flowing in 20 ohm resistance of 	the network shown below

fig.

b)	State and explain maximum power transfer theorem.

7.	Two coils are connected in series as shown in figure below (a) find an expression for the equivalent inductance of the circuit (b) If L1 = 10mH, L2=5mH,                                 M12 = 	2mH.  Find the equivalent inductance.

8.	A two port network is terminated by a load impedance ZL  at port 2. Find driving 	point impedance at port 1 in terms of (a) Z parameters  (b) Y parameters and (c) 	ABCD parameters.

-x-