Code No. 221051

OR

II B.Tech. II-Semester Supplementary Examinations, November-2003

MECHANICS OF FLUIDS AND HYDRAULIC MACHINES

(Common to Electronics and Instrumentation Engineering and  Electronics and Control Engineering)

Time: 3 hours							       Max. Marks:70

Answer any Five questions

All questions carry equal marks

- - -

1.a)	Explain Newton’s law of viscosity, stating its applications.

b)	A U-tube is made up of two capillaries of diameters 1 mm and 1.5 mm respectively.  The U-tube is kept vertically and partially filled with water (surface tension of 0.0075 kg/m) and zero contact angle.  Calculate the difference in levels of miniscii caused by the capillary action.

2.a)	Derive a general expression for total hydrostatic pressure and center of pressure on an inclined submerged plane surface in a liquid.

b)	A single tube piezometer installed vertically on a reservoir containing water registers a rise in its level by 50 cm.  Estimate the pressure in the reservoir and what would be the piezometer reading be if the reservoir contain an oil of specific gravity 0.8.

3.a)	Define (i) stream lines  (ii) path lines and   (iii) streak lines in a fluid flow.

b)	Air flows steadily through a horizontal nozzle.  At the nozzle inlet, the velocity is 6 m/s and pressure is 100 KPa.  If the inlet area is 0.1 m2 and the contraction ratio is 5, determine (i) the velocity and pressure at the exit.   (ii) the discharge and mass flow rate through the nozzle.

4.a)	Derive an expression for discharge through Venturiemeter.

b)	A tank has the form of a frustum of a cone with its large diameter D at the top.  The height of water in its frustum is H.  Obtain an expression for time of emptying all the water through an orifice of area ‘a’ located at the bottom of the frustum in terms of H, a and D.

5.a)	Derive an expression for maximum power transmission through a pipe.

b)	A siphon of diameter 20 cm connects two reservoirs having a difference in elevation of 20 m.  The total length of the siphon is 800 m and the summit is 5 m above the water level in the upper reservoir.  If the separation takes place at 2.8 m of water absolute, find the maximum length of the siphon from the water level in upper reservoir to the summit.  Assume friction factor as 0.004 for siphon and atmospheric pressure as 10.3 m of water.

Contd……2

Code No:221051					-2-				OR

6.a)	Derive the expression for work done by a jet of water impinging on a moving curved vane.

b)	A jet of water 4 cm diameter enters a fixed curved vane with a velocity of 50 m/s at an angle 20o to the horizontal.  Find the normal and tangential forces exerted by the jet, if it leaves the vane at angle of 15o to the horizontal.

7.a)	What is the significance of unit power, unit speed and unit discharge?

b)	A Pelton wheel has a tangential velocity of buckets as 15 m/s.  Water is being supplied under a head of 36 m at the rate of 20 H/s.  The buckets deflect the jet through an angle of 160o.  If  the coefficient of velocity for the nozzle is 0.98, find the power produced by the wheel.

8.a)	Define (i) manometric head   (ii) manometric efficiency and  (iii) overall efficiency of a centrifugal pump.

b)	A centrifugal pump has to discharge 2000 H of water per second and develops a total head of 15 m, when the impeller rotates at 240 RPM.  The impeller diameter is 1.5 m and velocity of flow at outlet is 2.5 m/s.  If the vanes are set back at an angle of 30o at the outlet, find manometric efficiency.  If the inner diameter is half of the outer diameter, find the minimum speed to start pumping.

- - -