Code No: 220301

Set No.

1

II B.Tech. II-Semester Supplementary Examinations, November-2003

MECHANICS OF FLUIDS

(Common to Mechanical Engineering and Metallurgy and Material Technology)

Time: 3hours								Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

----

1.a)	The weight of 8 m3 of a  certain oil is 64 KN.  Calculate its specific weight, mass density and specific gravity.

b)	The weight of an object measured on ground level where ge = 9.81 m/sec2 is 35,000 N. Calculate its weight at the following locations (I) Moon gm = 1.62 m/sec2,(ii) Sun , gs=274.68 m/sec2 (iii) Mercury, gme = 3.53 m/sec2 (iv) Jupiter, gj= 26.0 m/sec2 (v) Saturn, gsa = 11.2 m/sec2 and (vi) Venus, gv = 8.54 m/sec2.  Also find the mass density of the object on these planets.

2.a)	Define and distinguish between steady flow and uniform flow.  Give two examples of each flow.

- Define stream line and derive the equation of a stream line.

3.a)	Derive Euler’s equation of motion along a stream line. State assumptions made in the derivation.

In an inclined pipe of uniform diameter 25 cm, a pressure of 50 kPa was observed at section – 1 which was at elevation 10.0 m. At another section – 2 at elevation 12.0 m, the pressure was 20 kPa and the velocity was 1.25 m/s. Determine the direction of flow and the head loss between these two sections. The fluid in the pipe is water.

4.a)	Sketch the growth of boundary layer on a flat plate and explain the different regions of it.

b)	Find the displacement thickness and wall shear stress for the velocity distribution in a boundary layer (u / U) = (y / ) where U is the Velocity and  is the boundary layer thickness.

5.a)	What is the function of wind tunnel?

b)	Dry air flows in a duct 30 cm inside diameter. A pitot tube inserted in the direction of flow measures 15 cm water column in a manometer. The static pressure is 5 cm water gauge above atmospheric pressure and the temperature is   17  C. Determine the velocity of air. Take atmospheric pressure as 10.5 N/cm2.

Contd..2

Code No: 220301				-2-				Set No.1

6.a)	Prove that in a steady uniform laminar flow, the pressure gradient in the direction of flow is equal to the shear stress gradient in the normal direction.

b)	In an experiment, the details of laminar flow of fluid are as follows.  Determine the discharge in the pipe.

Specific gravity	=	1.67

Viscosity		=	1.56 poise

Diameter of pipe	=	15 cm

Length of the pipe	=	2000 m

Loss of head		=	0.45 m

7.a)	Define and explain the terms hydraulic gradient line and total energy line.

b)	A pipe 20cm diameter and 1800 m long connects two reservoirs one being 30m below the other. The pipe line crosses a ridge whose summit is 7.5m above the upper reservoir. What will be the minimum depth of the pipe below the summit of the ridge in order that the pressure at the apex doesn’t fall below 7.5m vacuum? The length of the pipe from the upper reservoir to the apex is 300m. Taking       f= 0.032 determine the rate of flow to the lower reservoir in lit/min.

8.	An orifice meter is to be fitted into a horizontal pipe 20 cm dia, carrying oil of specific gravity 0.85 for the purpose of flow measurement.  The differential head is to be indicated by a U-tube Manometer containing mercury (specific.       Gravity = 13.6).  If the manometer reading is not to exceed 0.2 m when the flow is 15Kg/sec. What should be the diameter of the orifice?  Assume Cd = 0.62.

&&&

Code No: 220301

Set No.

2

II B.Tech. II-Semester Supplementary Examinations, November-2003

MECHANICS OF FLUIDS

(Common to Mechanical Engineering and Metallurgy and Material Technology)

Time: 3hours								Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

----

1.a)	Derive the equation for shear stress produced when a fluid is subjected to angular deformation. Give the units of the fluid property so obtained in the derivation.

b)	Calculate the velocity gradient at distances of 0,10,15 cm from the boundary if the velocity profile is a parabola given by u = Ay2 + By + C and with the vertex 15cm from the boundary where the velocity is 100 cm/sec.  Also calculate the shear stress at these points if the fluid has a viscosity of 8.2 poise.

2.a)	Define and derive the equation of rotation for a fluid particle in a flow field about any  axis.

- A fluid flow field is given by V = xy i+ 2yz j – (yz +z2) k

Determine whether this is a possible steady incompressible fluid flow.  If so, determine the value of rotation at the point P (1,2,3).

3.a)	What are the applications of Bernoillis equation.

b)	A conical pipe has diameter 0.40 m & 0.80 m at its two ends. The smaller end is 2 m above the larger end. For a flow of 0.30 m3 / sec of water the pressure at the lower end is 10 kPa. Assuming a head loss of 2 m and kinetic energy correction factor  = 1.1 and 1.5 at the smaller and larger ends respectively, estimate the pressure at the smaller end.

4.a)	What is meant by smooth boundary and a rough boundary?

b)	Describe briefly the phenomenon of boundary layer separation.

- At what wind speed must a 127 mm diameter sphere travel through water to have a drag of 5 N.

5.a)	Differentiate between compressible and incompressible flows.

b)	A large vessel fitted with a nozzle, contains air at pressure of  2500 KN/m2 and a temperature of 20 C . If the pressure at the outlet of the nozzle is 1750 KN/m2, find the velocity of air flowing at the outlet of the nozzle?

6.a)	Enumerate distinguish characteristics of laminar flow? Give examples where such a flow is encountered.

b)	Oil of absolute viscosity 1.5 poise and relative density 0.85 flows through a 30 cm diameter pipe.  If the headloss in 3000m length of pipe is 20m, estimate the friction factor by assuming the flow to be laminar.

Contd..2

Code No:220301				-2-				Set No.2

7.a)	Obtain an expression for head loss due to sudden expansion in the pipe. List all the assumptions made in the derivation.

b)	If two pipes of diameters D and d and equal length L are arranged in parallel, the loss of head for a flow of Q is h. If the same pipes are arranged in series, the loss of head for the same flow Q is H. If d = 0.5D, find the percentage of total flow through each pipe when placed in parallel and the ratio (H/h). Neglect minor losses and assume f to be constant.

8.	Explain orifice meter in detail with diagram.  Also derive an expression for finding out the actual discharge from a given orifice meter.

&&&

Code No: 220301

Set No.

3

II B.Tech. II-Semester Supplementary Examinations, November-2003

MECHANICS OF FLUIDS

(Common to Mechanical Engineering and Metallurgy and Material Technology)

Time: 3hours								Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

----

1.a)	Explain from microscopic point of view, the concept of viscosity and momentum transfer.  Explain  how viscosity of liquids and gases behave with temperature.

- A thin plate of large area is placed midway in a gap of height h filled with oil of viscosity o  and the plate is pulled at a constant velocity V.  If a lighter oil of viscosity 1 is then substituted in the gap, it is found that for the same velocity V, the drag force will be the same as before if the plate is located unsymmetrically in the gap but parallel to the walls.  Find 1 in terms of o and the distance from the nearer wall to the plate.

2.a)	State the basic principle of continuity equation.  Obtain an expression of continuity equation for a three dimensional – steady – incompressible fluid flow.

- A flow is described by the  stream function  = 2 3 XY.  Locate the point at which the velocity  vector has a magnitude of 4 units and makes an angle of 150o with the x- axis.

3.a)	What are the body forces considered in fluid flow problems?

b)	A 15-cm diameter pipe is reduced to 7.5 cm diameter through a gradual contraction. The difference between the piezometric heads at the main and contracted section is 4 cm of mercury. By neglecting losses calculate the discharge of water.

4.a)	Define coefficients of drag and lift and state factors affecting on which those coefficients depend.

b)	A kite has an effective area of 0.4 m2 and weighs 2.0 N in a wind of 40 km/hr., the drag on the kite is 12 N.  Determine the tension in chord if the chord make an angle of 45o with the horizontal. Also determine lift coefficient

5.a)	How are shocks formed? Give some practical examples.

b)	During a normal shock in a constant area duct containing air, the initial conditions are  P1 = 10 N/m2, T1 = 0  c; U = 1000 m/s Calculate (i) the corresponding trans shock condition  and (ii)  percentage change in density across the shock if R= 287 J/Kgk

Contd..2

Code No:220301				-2-				Set No.3

6.a) 	Derive Hazen-Poiseuille equation for laminar flow in the circular pipes.

b)	Glycerin of viscosity 1.5 pascal-sec and mass density 1200 kg/m3 flows at a velocity of 5 m/sec in a 10 cm diameter pipe.  Check whether the flow is laminar in pipe line.  Find the boundary shear stress in the pipe.

7.a)	Obtain an expression for the optimum exit diameter of a nozzle to be fitted at the service end of a pipe for maximum power transmission.

b)	 Find the loss of head when a pipe of diameter 20cm is suddenly enlarged to a diameter of 40cm. The rate of flow of water through the pipe is 250lit/sec.

8.a)	The rate of flow of water in a 150mm diameter pipe is measured with a venturimeter  with a 50mm dia. throat.  When a mercury manometer is connected across the converging section reads 8mm, the flow rate is 2.7 kg/s.  What is the coefficient of discharge at that flow rate and what is permanent loss of head? Specific gravity of mercury = 13.6

b)	What is the device used for measuring fluid pressure? Explain briefly the principle of an inclined Manometer.

&&&

Code No: 220301

Set No.

4

II B.Tech. II-Semester Supplementary Examinations, November-2003

MECHANICS OF FLUIDS

(Common to Mechanical Engineering and Metallurgy and Material Technology)

Time: 3hours								Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

----

1.a)	Define kinematic viscosity.  How is this name attributed to this property?

b)	Lateral stability of a long shaft 15cm diameter is obtained by means of a 25cm stationary bearing having an internal diameter of 15.025cm.  If the space between bearing and shaft is filled with a lubricant having a viscosity 24 Pa s, what power will be required to overcome the viscous resistance when the shaft is rotated at a constant rate of 180 r.p.m.

2.a)	Define stream function and explain its characteristics.

- If for a two dimensional potential flow, the velocity potential is given by

 = x (2y – 1)

Determine the velocity at the point P(4,5).

Determine also  the value of stream function   at the point P.

3.a)	Define the terms (i) Vortex flow (ii) Forced vertex flow (iii) Free vortex flow. Give suitable examples.

b)	A rectangular duct of width 25 cm has a two dimensional irrotational flow. It has an elbow made up of circular arcs of radius 40 cm and 65 cm for the inner and outer walls respectively. Calculate the discharge per unit width of the duct when the difference in pressure between outer and inner walls in the elbow is 30 kPa

4.a)	State stokes law. Prove that the drag coefficient for sphere for Reynolds number up to 0.2 is given by CD = 24/R

- A spherical sand particle of 0.1-mm diameter falls under the action of gravity in water. Determine its terminal fall velocity.

c)	Determine the bending moment at the base of a 40 m high chimney of cylindrical shape of diameter 2.5 m in a wind of uniform velocity 25 m / s. Take CD = 0.35 and pair = 1.25 kg / m3.

5.a)	What is sonic velocity? On what factor does it depends?

b)	A gas velocity of 300 m/s is flowing through  a horizontal pipe at a  Section where the pressure is 60 KN/m2 and temperature 40  C. The pipe changes in diameter and at this section pressure is  90 KN/m2. If the flow of gas is adiabatic, find the velocity of gas at this section.

Contd..2

Code No:220301				-2-				Set No.4

6.a)	Sketch the velocity distribution of laminar flow in ideal and real fluid flow and

explain it in detail.

b)	A fluid of viscosity 0.883 pascal-sec and specific gravity 1.26 is pumped along a horizontal pipe 65 m long and 10 cm diameter at a flow rate of 0.18 m3/sec. Determine the Reynolds Number and calculate the pressure loss in the pipe of the flow is laminar.

7.a)	What is siphon? On what principle it works? Under what conditions would it stop functioning?

b)	 A horizontal pipe of diameter 50cm is suddenly contracted to a diameter of 25cm. The pressure intensities in the large and smaller pipe are given as 13.734N/cm2 and 11.772 N/cm2 respectively. If the rate of flow of water is 300lit/sec,  find the value of coefficient of contraction.

8.a)	 Derive an expression for discharge of liquids through a rectangular notch.  Explain how it is modified to take into account the effect of end contractions and velocity of approach.

- Explain the working of Viscometers.

&&&