Code No: 220353

OR

II B.Tech. II Semester Supplementary Examinations, November-2003

THERMAL ENGINEERING

(Common to Mechanical Engineering, Production Engineering and Mechatronics)

Time: 3 hours						       	Max. Marks: 70

Answer any Five questions

All question carry equal marks

- - -

1.a)	Give the boilers classification.

b)	Sketch one variety of safety valve and explain its working.

c)	How do you decide the height of the chimney for a given draught and discharge?

2.a)	Compare artificial, induced, forced and chimney draughts.

b)	How is boiler horse power calculated?

c)	Sketch and explain the working of Economizer used in boilers.

3.a)	What are ideal and actual flows through nozzles? Explain.

b)	What is meant by degree of super saturation? Explain.

c)	What is meant by critical pressure ratio? Explain its importance.

4.a)	What is meant by condenser efficiency and vacuume efficiency? Discuss.

b)	Give the classification of condensers.

c)	A simple Delavel steam turbine nozzle ring is supplied with 200 kg/hr of dry steam at 10 kgf/cm2. The exhaust pressure is 0.6 bar, nozzle angle is 200. Nozzle efficiency is 85%. Blade outlet angle is 280. Blade speed is 200 m/sec. Blade velocity coefficient is 0.78 power loss due to fan action and disc fraction is        1.5 kW. The losses reappear as heat.

Calculate:

i)   Net output at the shaft.

ii)  The dryness fraction of steam in exhaust passage when the speed is small and

iii) The dryness of steam just as it leaves the turbine rotor blading.

5.a)	What is the advantage of multistage air compression? Explain with P-V-diagram.

b)	Sketch the Roots blower and explain its working.

c)	A single stage, single acting reciprocating air compressor is compressing 20 kg/min. of air from 110 kPa, 300C to 600 kPa and delivers to a receiver. Law of compression is PV1.25= Constant. Mechanical Efficiency = 80%. Find the power input neglecting losses due to clearance, leakage and cooling.

6.a)	Sketch an impulse turbine and explain its working.

b)	Discuss the condition for maximum efficiency of turbines.

c)	Discuss pressure compounding of a steam turbine. Bring out its salient features.

(Contd…2)

Code No:220353				:: 2 ::					OR

7.	In an air craft cooling system, air enters the compressor at 0.1 MPa, 4oC and is compressed to 0.3 MPa with an isotropic efficiency of 72%. After being cooled at 550 C at constant pressure in a heat exchanger, the air then expands in a turbine to 0.1 MPa with an isentropic efficiency of 78%. The low temperature air absorbs a cooling load of 3 tonnes of refrigeration at constant pressure before re-entering the compressor which is driven by the turbine. Assuming that air to be ideal, determine the COP of the refrigerator, the driving power required and the air mass flow rate.

8.	Write short notes on any FOUR of the following:

a)	Equivalent evaporation.

b)	Super saturated flow.

c)	Velocity compounding.

d)	Polytropic efficiency.

e)	Vapour compression refrigeration.

-*-*-*-