Code No: 220102

Set No.

1

II B.Tech.II-Semester Supplementary Examinations November-2003

STRENGTH OF MATERIALS-II

(Civil Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Explain the construction of Mohr’s circle to obtain principal  stresses.

b)	Direct stresses 120mpa tensile and 90 MPa compressive are applied to an elastic material at a certain point on planes at right angles.   The greater principal stress is limited to 150 MPa.  What shearing stress may be applied to the given planes and what will be the maximum shearing stress at the point?

2.a)	On an element subjected to biaxial stress system given x, y,  and  xy, derive the expression for normal and shear stress on a plane making an angle  with the positive direction of x.

b)	Determine the direction of principal planes and magnitude of the principal stresses, whenever, rectangular block of material is subjected to tensile stress of x, on one plane and another tensile stress of y on the other plane at right angles together with shear stress of  xy on the same plane.

3.	A solid shaft is subjected to a torque of 100 N.M.  Find the necessary shaft diameter, if the allowable shear stress is 100 N/mm2 and the allowable twist is    30  per 10 meter length of the shaft.  Take C=1X105N/mm2.

4.	A helical spring is made of 12 mm diameter steel wire by winding it on a 120 diameter mandrel.  If there are 10 active coils, what is the spring constant?  What force must be applied to the spring to elongate it 10mm?

5.a)	Derive the Eulers formula for a column hinged at both ends.

b)	Compare the crippling loads given by Euler’s and Rankine formula for a tabular steel column 2.5m long having outer and inner diameters 40 mm and 30 mm  respectively loaded through pin joint at each end.  Take y = 330 MPa, =17500 and E-=2.05X105 MPa.

6.	A masonry chimney of a hollow circular section has an external diameter of 1.8m and thickness 0.3m.  Compute the maximum height to which it can be constructed, such that no tension develops at its base when subjected to a horizontal wind pressure of 1.5 KN/m2.  Take density of masonry as 20 KN/m3.

Contd..2

Code No: 220102				-2-			Set No.1

7.a)	Derive Lame’s equations for radial and hoop stresses developed in a thick cylinder which is subjected to fluid pressure.

b)	A thick walled cylindrical pressure vessel has inner radius of 150 mm and outer radius  of 185 mm is subjected to an internal pressure of 10 MPa.  Determine the radial stress and hoop stress induced at the inner and outer surfaces.

8.	Determine the principal moments of Inertia for an unequal angle section 60 mm X 40 mm X 6mm.

&&&

Code No. 220102

Set No.

2

II Year II-Semester Supplementary Examinations November 2003

STRENGTH OF MATERIALS - II

(Civil Engineering)

Time: 3 hours							Max. Marks: 80

Answer any Five questions

All question carry equal marks

***

1.	A rectangular block is subjected to a tensile stress of 80N/mm2 and 35 N/mm2 on another plane which is at right angle to the first. A shear stress of 50 N/mm2 on the same planes. Find the magnitude of principal stresses and the direction of principal planes.

2.	If σx = σ1 = 0 and σy = σ2 =  −20MPa, using Mohr’s circle of stress, find the stresses acting on a plane defined by θ = +300.

3.	A hollow shaft of diameter ratio 3/5 is required to transmit 450 kw at 120 rpm. The shear stress in the short should not exceed 60 MPa and the angle of twist in a length of 2.5m must not exceed 10 using C=8X104 MPa, find the minimum internal diameter.

4.	In a close coiled helical spring, the diameter of each coil is to be 12 times that of the wire of the spring and the maximum shear stress is not to exceed 50 N/mm2. Maximum permissible deflection under a load of 400N is 80mm. Using a shear modulus of 8.4X104 N/mm2 determine: (i) diameter of the coil.(ii) Number of coils.

5.	A hollow cylindrical cast-iron column of 150mm enternal diameter and 15 mm thickness, 3.6m length is hinged at one end and fixed at the other. Find:

i)  The ratio of Euler’s and Rankine’s  loads.

ii) The length for which the critical load by Euler’s and Rankine’s  formula will be equal.

E = 8.4 X 104 N/mm2, σ1 = 525 N/mn, α = 11600.

6.	A masonry retaining wall is 8m high. Its water face is vertical and the cross-section is trapezoidal, 1m wide at top and 4m wide at bottom. Determine the maximum height of water level so that no tensile stress is developed at the base of wall. Also determine the maximum compressive stress produced at that water level. Take the specific weight of masonry 22.8 KN/m3 and that of water            10 KN/m3.

7.	A pressure vessel with outer and inner diameters of 400mm and 320mm respectively is subjected to an enternal pressure 8 MPa. Draw a sketch showing the radial pressure and hoop stress distribution on the section of the cylinder wall.

8.a)	Explain the resolution of bending moment into two rectangular axes through the centroid.

b)	Explain the location of neutral axis.

-*-*-*-

Code No:220102

Set No.

3

II B.Tech.II-Semester Supplementary Examinations November-2003

STRENGTH OF MATERIALS-II

(Civil Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.	At a certain point in a stressed material, the following stress condition exit:

x = +600 MPa;	y=+200Mpa and =400Mpa.  Calculate the principal stresses including their planes and maximum shear stress and its plane.

2.a)	Explain the construction of Mohr’s circle to obtain principal stresses.

b)	Draw Mohr’s stress circle for a stress condition of x=y=+400 MPa and =0

3.a)	Derive the basic torsion equation TJ=τr=Cθl.

b)	Find the power that can be transmitted by a shaft 60 mm in diameter at 200 rpm, if the permissible shear stress is 90N/mm2 and maximum torque exceeds the mean by 30%.

4.	A close coiled helical spring of 20 turns has to be designed such that the maximum shear stress produced does not exceed 150 MPa and the stiffness of the spring does not fall below 20 Kn/cm.  Determine the diameter of spring wire and mean diameter of spring,  Take C=0.8x105 MPa.

5.a)	Derive the Rankines formula.		Pcr=σ c ×A1+α(lk)2

b)	A built-up  I section has an overall depth of 400mm, width of flanges 300 mm thickness of flanges 50mm  and web thickness 20mm.  It is used as a beam with simply supported ends and it deflects by 10mm when subjected to a load of 40KN/m length.  Find the safe load on this I-section used as a column with both ends hinged.  Use Euler’s formula Assume a factor of safety 1.75 and take E=2x105 N/mm2

6.a)	What do you understand by the kern of section.

b)	A square hollow chimmney of outside dimension 1500 mm and thickness          150 mm, is 30 m high.    Dead weight of the material of the chimney is 25kN/m3.  If a wind load of intensity 2.5kN/m2 acts through out the height of the chimney, determine the maximum and minimum stresses induced at the base of the chimney.

Contd..2

Code No:   220102			-2-					Set No. 3

7.a)	Define thick  cylinders.

b)	A cylindrical pressure vessel of 500 mm inner diameter is subjected to an internal pressure of 9MPa.  Taking allowable stress for the material of the cylinder as      40 MPa, determine.

- Wall thickness of cylinder

- Circumferential stress at the outer surface.

8.	Derive centroidal principal axes of a section Iuu and IVV.

&&&

Code No:220102

Set No.

4

II B.Tech.II-Semester Supplementary Examinations November-2003

STRENGTH OF MATERIALS-II

(Civil Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1. 	A rectangular block of material is subjected to a tensile stress of 100MPa on one plane and a tensile stress of 50MPa on a plane at right angles, together with a steam stress of 60MPa on the same planes. Find

a)	The principal stresses and their directions.

b)	The greatest shear stress.

2. 	Construct Mohr’s stress circle for a state of stress condition x = +600MPa;

y = -200MPa and  = 400MPa. Calculate from this diagram principal stresses, 	maximum shear stress and their planes.

3. 	A hollow shaft of diameter ratio 3/8 is required to transmit 600KW at 110rpm, the 	maximum torque being 20% greater than the mean. The shear stress is not to 	exceed 63N/mm2 and the twist in a length of 3m not to exceed 1.4 degrees. 	Calculate the maximum external diameter satisfying these conditions.

4. a)	Derive an expression for the deflection of an axially loaded close coiled helical 	spring.

b)	Explain various theories of failures.

5. a)	Write the assumptions made in Euler’s theory of columns. Also mention its 	limitations.

b)	A 2 metre long column has a square cross-section of side 40mm. Taking the 	factor of safety as 4, determine the safe load for the end conditions.

(i)  Both ends are hinged (ii) One end is fixed and other end is hinged. E=210Gpa.

6.	 A dam section is 8 metres high, the maximum depth of water impounded being 	7.5 metres, The top width of section is 1 metre. The weight of masonry is 	22kN/m3. While that of water is 9.81kN/m3. Find the minimum width required. 	Coefficient of 	friction between masonry is 0.6 The water force of the dam is 	vertical.

7. 	A cylindrical pressure vessel has inner and outer radii of 200mm and 250mm 	respectively. The material of cylinder has an allowable stress of 75Mpa. 	Determine the maximum internal pressure that can be applied and draw a sketch 	of radial pressure and circumferential stress distribution.

8.	Determine the principal moments of Inertia for an unequal angle section 200mm x 	150mm x 10 mm.

~ ~ ~ ~