Code No. 220204

Set No.

1

II Year II-Semester Supplementary Examinations November 2003

POWER SYSTEMS -I

(Electrical and Electronics Engineering)

Time: 3 hours							Max. Marks: 80

Answer any Five questions

All question carry equal marks

---

1.a)	With a neat sketch explain the arrangement of hydraulic power plant.

b)	A hydro-station has to operate with a mean head of 30 meters and is supplied from a reservoir lake at the rate of 6.93m3 per second.  Calculate the power generated in kW.  Assume density of water 1,000 kg/m3, load factor of the station 80%, mechanical efficiency of water turbine 90%, efficiency of generators 90%.  Neglect head loss in pipes,  penstocks etc.

2.	Describe the schematic arrangement of a thermal power station and explain the function of each briefly.

3.a)	Explain with neat diagram the concept of regenerative heating and mention its advantages.

b)	Discuss the natural and forced draughts and list out the difference between them.

4.	What is a Nuclear Reactor? Explain the basic components of Nuclear Reactor.

5.		Explain with neat diagram the function of BWR. Mention the advantages and

disadvantages.

6.a)	What is a sub-station ? Discuss the different ways of classifying the sub-stations.

b)	Write explanatory note about the sub-station equipments

7.a)	What do you understand by the load curve? What information is conveyed by a load curve?

b)	Describe the desirable characteristics of a tariff.

8.a)	Explain the terms load factor and diversity factor. How do these factors influence   the cost of generation?

b)	Describe some of the important types of tariff commonly used .

$$$

Code No. 220204

Set No.

2

II Year II-Semester Supplementary Examinations November 2003

POWER SYSTEMS -I

(Electrical and Electronics Engineering)

Time: 3 hours							Max. Marks: 80

Answer any Five questions

All question carry equal marks

---

1.a)	Explain hydrograph and flow duration curve and mention their use.

b)	A hydro-electric station operates under a mean head of 30 metres.  The reservoir employed has a catchment area of 4 x 108 m2.  The average rainfall is area is 125cm per annum.  Determine the capacity of the station for which it should be designed.  Assume that 30% of the rainfall is lost due to evaporation etc., 5% of the head is lost in penstock, turbine efficiency is 85%, alternator efficiency is 85% and the load factor is 50%.

2.	What are the systems used for firing the boilers? Explain by giving sketches. Also mention the advantages and disadvantages of pulverizing the fuel used in boilers?

3.a)	Explain the function of chimney and precipitator.

b)	Discuss the need of cooling towers and list out various types of cooling towers

4.	Name different nuclear fuels. Explain each of them in detail.

5.	Describe working of PWR (pressurized water reactor). What are its advantages and disadvantages?

6.a)	Give the symbols used for the equipments in sub-station and mention their role.

b)	Define and differentiate indoor and outdoor sub-stations.

7.a)	Define the following terms in connection with power supply systems:

(i) Connected load			(ii) maximum demand

(iii) Demand factor and 		(iv) load factor.

b)	Describe some of the important types of tariff commonly used.

8.a)	Discuss the various methods of determining the depreciation of an equipment.

b)	A 100 MW power station delivers 100 MW for 2 hour, 75 MW for 6 hour and is shut down for the rest of each day. It is also shut down for maintenance for 20 days each year. Calculate its annual load factor.

$$$

Code No. 220204

Set No.

3

II Year II-Semester Supplementary Examinations November 2003

POWER SYSTEMS -I

(Electrical and Electronics Engineering)

Time: 3 hours							Max. Marks: 80

Answer any Five questions

All question carry equal marks

---

1.a)	What are the factors to be considered for selection of site for hydro-electric power station.

b)	A proposed hydro-electric station has a available head of 25 metres, a catchment area of 5 x 107 m2, the rainfall for which is 120 cm per annum. If 75% of the total rain all can be collected, calculate the power that could be generated,  Assume overall efficiency as 65%.

2.	Explain the basic principle of working of pumped storage plants.  Mention the advantages and disadvantages of the scheme.

3.	What are the functions of Economizer and superheater in a thermal power plant?

4.	Explain with neat sketch,  the principle of operation of Nuclear Reactor.

5.	Describe with neat sketch the construction and operation of a Heavy water cooled and Moderate Reactor? What are its advantages and disadvantages?

6.a)	Explain the role of sub-station in a power system.

b)	Draw and explain the layout of 11 KV / 440 KV substation.

7.a)	Write notes on the advantages of high load factor.

b)	The power generation equipment of a power station costs Rs. 60, 75, 000 and has a usefull life of 30 years.  If the salvage value of the equipment is Rs. 3, 00, 000 and annual interest rate is 6%, determine the annual amount to be saved by sinking fund method.

8.a)	Enlist the effects of high load factor on the operation of power plants.

b)	The tariff in force is Rs. 550 per kVA of maximum demand and Rs. 2.5 per unit consumed. If the load factor is 40%, find the overall cost per unit at

(i) unity pf and     (ii) 0.7 pf.

$$$

Code No.220204

Set No.

4

II Year II-Semester Supplementary Examinations November 2003

POWER SYSTEMS -I

(Electrical and Electronics Engineering)

Time: 3 hours							Max. Marks: 80

Answer any Five questions

All question carry equal marks

---

1.a)	With neat diagrams discuss penstocks and water hammer.

b)	A hydro-electric station operates with a mean head of 40 metres and is supplied from a catchment area of 600 km2 over which the average rain fall is 150 cm per annum.  If 65% of the total rainfall can be utilized, calculate the average power that can be generated.  Assume hydraulic efficiency 80% and electrical efficiency 90%.

2.	What do you understand by dry bottom furnace and wet bottom furnace? Explain with relevant diagrams.

3.	Discuss steam station controls with reference to boiler control, turbine control and electrical control.

4.	Name the different nuclear materials. Explain each of them with examples.

5.	Explain with neat sketch the fast breeder reactor. Write its advantages and	disadvantages.

6.a)	Explain the classification of sub-stations.

b)	Explain the advantages of outdoor sub-station as compared to the indoor          sub-station.

7.a)	Explain how a load duration  curve is plotted . What is its use?

b)	Discuss the different classifications of costs of electrical energy.

8.a)	What do you understand by (i) base load and (ii) peak load of a power station?

b)	A power station has a daily load cycle as under: 260 MW for 6 hour; 200 MW for 8 hour; 160 MW for 4 hour; 100 MW for 6 hour. If the power station is equipped with 4 sets of 75 MW each, calculate

i)	daily load factor 		ii)	capacity factor and

iii)	daily requirement of fuel if the calorific value of oil used were of 10,000 kcal/kg and the average heat rate of station were 2860 kcal/kWh.

$$$