Code No. 220205

Set No.

1

II Year II-Semester Supplementary Examinations November 2003

ELECTRO MECHANICS-II

(Electrical and Electronics Engineering)

Time: 3 hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.a)	Discuss the effects of variation of frequency and supply voltage on iron losses of 	transformer.

b)	The flux in a magnetic core is alternating sinusoidally at a frequency of 600 Hz.  	The maximum flux density is 2 tesla.  The eddy current loss is 15 watts.  Find the 	eddy current loss in the core if the frequency is raised to 800 Hz and the 	maximum flux density is reduced to 1.5 tesla.

2.a)	Explain the procedure for conducting OC and SC tests with neat diagrams.

b)	A 20 KVA, 2500/250V, 50Hz, Single phase transformer gave the following test results:OC test(LV side): 250V, 1.6A, 110W; SC test(HV side): 90V, 7A, 300W.  Compute the parameters of the approximate equivalent circuit referred to LV side.

3.a)	Explain the following characteristics of an auto transformer with two winding 	transformer:

(i)	Rating				(ii)	Losses

(iii)	Impedance drop		(iv)	Voltage regulation

b)	The primary and secondary voltages of an auto transformer are 500V and 400V 	respectively.  Show with the aid of a diagram, the current distribution in the 	winding when the secondary current is 100A and calculate the economy of Cu in 	this particular case.

4.a)	Discuss the constructional details of the three phase transformers with necessary 	diagrams. Mention their advantages and disadvantages of different 3 	transformers.

b)	Draw the phasor diagrams and winding connection of a three-phase transformer 	for 	i)	Group 1: phase displacement of zero degrees,

ii)	Group 2: phase displacement of 180 degrees.

5.a)	Does the induction motor have any similarities with the transformer? Compare 		the similarities and differences between them.

b)	Show that a rotating magnetic field is produced in the air-gap, when a balanced 	three-phase ac supply is given to the stator of a 3-phase induction motor. Justify 	your claim with necessary mathematical equations.

Contd….2

Code No. 220205				::2::				Set.No.1

6.a)	Describe the principle of regulating the voltage with the help of tap-changers.

b)	Discuss whether the winding should be tapped on HV side or LV side.

7.a)	Discuss the points of similarities between a  transformer and an induction machine. Hence explain why an induction machine is called a generalized transformer.

b)	Explain why an induction motor at no load operates at a very low power factor.

8.a)	Draw the neat sketch of Scharge motor and explain the principle of operation with 	necessary phasor diagram.

b)	Explain how the speeds above and below synchronous speeds are obtained in   charge motor.

$$$

Code No. 220205

Set No.

2

II Year II-Semester Supplementary Examinations November 2003

ELECTRO MECHANICS-II

(Electrical and Electronics Engineering)

Time: 3 hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.a)	Explain mutual flux, leakage flux, magnetizing reactance and leakage reactance of 	transformer.

b)	A 100 KVA transformer has 400 turns on the primary and 80 turns on the secondary. The primary and secondary resistances are 0.3Ω and 0.01. respectively. The corresponding leakage reactances are 1.Ω and 0.035Ω respectively. Calculate the equivalent impedance referred to primary circuit.

2.a)	Explain the procedure for conducting Sumpner’s test along with all precautions to 	be taken while conducting the test with neat diagram.

b)	The corrected instrument readings obtained from short circuit test on 10 KVA, 	450/125V, 50Hz transformer are 9.65V, 22.2A, 120W, with low voltage winding  	short circuited.  Compute voltage regulation for an 85% lagging load at 50% load.

3.a)	What are the applications of auto transformers?

b)	A 20 KVA, 2400/240V two winding step down transformer is connected as an 	auto transformer with additive polarity. Compute: (i) Original current capacity of 	HV winding (ii) Original current capacity of LV winding (iii) KVA rating of auto 	transformer using current capacity of LV winding as calculated in(ii),  (iv)

percent increase in KVA capacity of auto transformer as compared to 	original two winding transformer (v) percent overload of 2400V winding when 	used as an auto transformer.

4.a)	Discuss in detail the effect of  (i) current harmonics, (ii) voltage harmonics on 	the performance of three phase transformers.

b)	Explain how the harmonics can be suppressed using Star/Delta earthing 	transformer. Draw the relevant connection diagram.

5.a)	Explain how the torque is produced in the rotor of an induction motor?

b)	(i) Show that the relative speed between resultant rotor field and resultant stator 	field of a 3-phase induction motor is zero, (ii) Derive the expression for the 	rotor e.m.f and rotor current of an induction motor.

Contd….2

Code No. 220205				::2::				Set.No.2

6.a)	Describe the principle of regulating the voltage with the help of tap changers.

b)	Explain why the transformer winding should be trapped in the middle.

7.a)	In a polyphase induction motor, show that the rotor must rotate in the direction of rotating magnetic field.

b)	Show that the slip ring and squirrel cage induction motors are identical in respect of their rotor reactions on the stator are concerned.

8.a)	Explain the principle of operation of Induction generator and draw its power flow 	diagram and speed torque characteristics.

b) 	Explain the pole change method speed control with neat sketch by changing the connection of stator coils of 3-phase Induction motor.

$$$

Code No. 220205

Set No.

3

II Year II-Semester Supplementary Examinations November 2003

ELECTRO MECHANICS-II

(Electrical and Electronics Engineering)

Time: 3 hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.a)	Draw the equivalent circuit of a single phase transformer.  Show how the 	equivalent circuit can be simplified without introducing much error.

b)	Prove that the secondary resistance R2 and reactance X2 of the transformer 	referred to primary side are R2/K2 and X2/K2 respectively, where K is 	transformation ratio.

2.a)	Explain the procedure to predetermine the efficiency and regulation of a 	transformer with all necessary equations (need not explain test procedures if any).

b)	The iron losses in a transformer core at normal flux density were measured at 	frequencies of 30 and 50Hz, and the results being 34W and 55W respectively.  	Calculate the hysteresis and eddy current losses at 50Hz.

3.a)	What are the advantages and disadvantages of autotransformers?

b)	Two transformers have the following particulars:

Transformer A	     Transformer B

Rated current				200A			600A

Per unit resistance			0.02			0.025

Per unit reactance			0.05			0.06

No-load emf				245V			240V

Calculate the terminal voltage when they are connected in parallel and supply a 	load impedance of (0.25 + j0.1) ohms.

4.a)	Explain why the magnetizing current in the transformer is non-sinusoidal even 	though the voltage applied to the primary is sinusoidal?

b)	A 3-phase, 3-winding, delta/delta/star, 33000/1100/400 V, 200 kVA transformer 	has a secondary load of 150 kVA at 0.8 pf lagging and a tertiary load of 50 kVA 	at 0.9 pf lagging. The magnetizing current is 4% of rated load, the iron loss 	being 1 kW. Calculate the value of the primary current when the other two 	windings are delivering the above loads.

Contd….2

Code No. 220205				::2::				Set.No.3

5.a)	Explain why an induction motor cannot develop torque when running at synchronous speed. Define the slip speed of an induction motor and deduce how the frequency of rotor currents and magnitude of rotor emf are related to slip.

b)	A 4-pole induction motor is energized from a 50 Hz supply. If the machine runs on full load at 2% slip, determine the

i) 	rotor speed with respect to resultant rotor field,

ii)	stator resultant field speed with respect to rotor structure, and

- frequency of rotor currents.

6.a)	Why should the tap-changer be connected near the neutral? What about delta connected transformed.

b)	Describe the no load tap changer with a suitable diagram.

7.a)	Explain why the rotor of polyphase induction motor can never attain synchronous speed.

b)	The rotor of a slip ring induction motor is connected to an AC source, where as its stator winding is short circuited. If rotating magnetic field produced by rotor winding rotates clock wise, explain the direction in which rotor must revolve.

8.a)	Explain how speed can be controlled by inserting external resistance in rotor winding of 3-phase slip ring Induction motor with the help of slip-torque characteristics.

b) 	A 400V,3-phase,4-pole,50HZ,1440 rpm slip ring induction motor is driving a constant torque load. What is the external resistance to be inserted to run the motor at 1200 rpm if the rotor resistance per phase is 0.3 ohm.

$$$

Code No. 220205

Set No.

4

II Year II-Semester Supplementary Examinations November 2003

ELECTRO MECHANICS-II

(Electrical and Electronics Engineering)

Time: 3 hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.a)	Draw the phasor diagram of a transformer on no load and explain the function of 	active and reactive components of no load current of transformer.

b)	Explain why transformer rating will be given in kVA but not in kW.

2.a)	Two similar 250KVA similar transformers gave the following  results when tested 	by back-to-back method:  Mains wattmeter, W1 = 5.0 KW, Primary series circuit 	wattmeter, W2 = 7.5KW (at full load current).  Find out the individual transformer 	efficiencies at 75% full load and 0.8 power factor lead.

b)	A 10KVA, 440/3300V, 1-phase transformer, when tested on open circuit, gave 	the following figures on the primary side : 440V, 1.3A, 115W.  When tested on 	short circuit with full load current flowing, the power input was 140W.  calculate 	the efficiency of the transformer at (i) full load unity power factor (ii) one quarter 	full load 0.8p.f.

3.a)	Explain, How a two winding transformer can be converted into an auto 	transformer?  What is its new rating?

b)  	Two single phase transformers with equal turns have impedances of (0.5 + j3) 	ohm and (0.6 + j10) ohm with respect to the secondary.  If they operate in 	parallel, determine how they will share a total load of 100KW at p.f. 0.8 lagging?

4.a)	Draw the phasor diagrams corresponding to parallel operation of transformers 	with (i) equal voltage ratio and (ii) unequal voltage ratios.

b)	Two single phase transformers A and B of equal voltage ratio are running  in  	parallel and supplying a load of 1000A at 0.8p.f. lag.  The equivalent 	impedances of the two transformers are (2 + j3) and (2.5 + j5)ohms 	respectively.  Calculate the current supplied by each transformer and the ratio of 	the KW output of the two transformers.

Contd….2

Code No. 220205				::2::				Set.No.4

5.a)	Explain the principle of operation of a three-phase induction motor and the 	meaning of the term slip. How does slip vary with the load?

b)	Most induction motors have a full load slip of 0.05 or less. The measured speed 	of a 60 Hz motor at rated load is 575 rpm. (i) How many poles does the motor 	have and what is its synchronous speed? (ii) what is the full-load slip? (iii) if no-	load slip is 0.01, what is the percentage speed regulation? (iv) what is the 	frequency of rotor voltage at no-load, full-load and the instant of starting?

6.a)	Why should the tap changer be connected near the neutral? What about delta connected transformer.

b)	What is the difference between no load and non load tap changer.

7.a)	Describe the methods to reduce the effect of crawling and cogging in an induction motor.

b)	With the help of generalized theory explain the steady state equivalent circuit of an induction motor.

8.a)	Describe static slip power recovery scheme of speed control with neat sketch.

b) 	What are the merits of this method over classical methods?

$$$