Code No: 220305

Set No.

1

II B.Tech. II-Semester Supplementary Examinations, November-2003

PRODUCTION TECHNOLOGY - I

(Common to Mechanical Engineering and Aeronautical Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Explain using simple sketches the procedure for making green- sand moulds.

b)	How can a sweep-type pattern cast a solid vase shaped casting?

c)	Why are cores seldom necessary in disposable pattern moulding?

2.a)	How are the moisture content and clay content of a molding sand tested?

b)	Explain with a neat sketch the working and use of a sand rammer.

3.a)	How are gates and risers removed?

b)	Discuss the properties that an alloy to be cast by permanent mould should posses.

4.a)	With the help of a schematic diagram explain the constructional details and reactions involved in Oxy-acetylene welding.

b)	Differentiate between “Solid Phase Welding” and “Liquid Phase Welding”.

5.a)	Explain the various types of resistance welding processes. What are the advantages and disadvantages and their limitations?

b)	What is nugget? How is the size of the resistance spot weld determined?

6.	Describe the different nondestructive tests performed on weldments.

7.a)	Classify the plastics with examples.

b)	What are the forms of  raw materials for processing plastics into products?

c)	Justify plastics as engineering materials.

8.a)	What is meant by ‘Power metallurgy’.

b)	Explain clearly the main stages of power metallurgy process.

-*-*-*-

Code No: 220305

Set No.

2

II B.Tech. II-Semester Supplementary Examinations, November-2003

PRODUCTION TECHNOLOGY - I

(Common to Mechanical Engineering and Aeronautical Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Explain the advantages of casting over other manufacturing processes.

b)	How does clay content affect moulding, sand strength and permeabiligy?

c)	Is an allowance made for draft in a disposable mould casting?

2.a)	Describe the process of shell moulding indicating:

(i)   Composition of sand mixture.	(ii) Steps in moulding.

(iii) Advantages and limitations.

3.a)	Why is pouring temp important in casting process? State and explain various pouring equipments.

b)	Explain investment-casting process.

4.a)	Explain the phenomenon of “Residual Stress” and “Distortion”?

b)	What is “Weldability”? Explain how and on what basis weldability can be assessed?

5.a)	Briefly explain “Butt-welding” and :seam welding”?

b)	With the help of a neat sketch explain the principle of Spot welding?

6.a)	What are the various welding defects? Discuss the causes for these defects.

b)	Explain the following metal joining techniques:

(i) Induction brazing.		(ii) Torch brazing.

7.	List various methods of processing of plastics and briefly explain them.

8.	What are the different methods of producing metal powders? Describe any two of them in detail.

-*-*-*-

Code No: 220305

Set No.

3

II B.Tech. II-Semester Supplementary Examinations, November-2003

PRODUCTION TECHNOLOGY - I

(Common to Mechanical Engineering and Aeronautical Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Write a note on types of foundry.

b)	What are the choices  in selecting a core binder?

2.a)	Explain the following:

(i) AFS Clay content test	(ii) AFS Sieve Analysis.

b)	Explain the following machines:

(i) Squeezers			(ii) Jolt-Squeeze machines.

3.a)	Differentiate between chamber and hot chamber die-casting.

b)	Give reason why Permanent moulds are better than sand moulds for mass production.

4.a)	Explain the mechanical effects of Welding Heat.

b)	Explain how Oxy-acetylene torch can be used for both Welding process.

5.a)	Explain the various steps involved in flash bult joint welding process.

b)	Compare upset welding and spot welding process?

6.a)	Explain the causes for different types of cracks in welds.

b)	Describe the following metal joining techniques:

(i) Dip soldering 	(ii) Braze welding.

7.a)	With the help of a neat sketch explain the blow moulding method for plastics.

b)	Write notes on the following:

(i) Slush moulding	(ii) Calendering.

8.a)	Distinguish between shotting and atomization.

b)	Enumerate the main characteristics of metal powders.

-*-*-*-

Code No: 220305

Set No.

4

II B.Tech. II-Semester Supplementary Examinations, November-2003

PRODUCTION TECHNOLOGY - I

(Common to Mechanical Engineering and Aeronautical Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Explain the importance of using metal as pattern material.

b)	Why is coarser sand better for steel castings than fine grained sand? Why is it that as castings increase in size, it is often better to use increasingly coarser sand?

2.a)	Why testing of foundry sand is necessary? What are the common tests performed on Foundry sand?

b)	Explain how you will use a sieve shaker for determining the grain fineness of a foundry sand.

3.a)	What is centrifugal casting? Discuss types of centrifugal casting.

b)	Write short notes on (i) side blind riser (ii) Swril gate.

4.a)	classify the different regions of Oxy-acetylene flame and with the help of neat sketches explain their characteristics.

b)	What is flux generally used in gas welding? What characteristics are required for a flux?

5.a)	Compare TIG welding with MIG welding with regard to equipment, performance, merits and demerits.

b)	What are the advantages and disadvantages of spot welding?

6.	How is Brazing different from soldering? Compare them with regard to methods adopted and their applications.

7.a)	Write notes on the following plastic laminating methods.

(i)    Pressure bag moulding	(ii) Vacuum bag moulding

(iii) Auto clave forming.

b)	With the help of a neat sketch explain the extrusion moulding process of plastics.

8.	Explain clearly the primary and secondary process, used for processing of metal powders.

-*-*-*-