Code No: 220402.

Set No:

1

II B.Tech. II-Semester Supplementary Examinations, November 2003

ELECTRICAL TECHNOLOGY

( Common to Computer Science and Engineering, Computer Science and Information Technology, Electronics and Communications Engineering,

Electronics and Computers Engineering, Computer Science and Systems Engineering, Electronics and Telematics  )

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1. a)	List out applications of D.C. generators and motors.

b)	A 400V shunt generator has full load current of 200A, its armature resistance is 0.06 , field resistance 100 ;the stray losses are 2000 W. Find the horse power of its prime-mover when it is delivering full load and find the load for which the efficiency of the generator is maximum.

2. a)	Why is starter necessary for a DC motor.

b)	Explain the working of a 3-point starter with a circuit diagram for a DC shunt motor.

c)  	A 230 V shunt motor has an armature resistance of 0.2. The starting armature current must not exceed 50A. If the numbers of sections are 5, calculate the values of resistance steps to be used in the starter.

3. a)	Discuss  the constructional features of transformers.  Draw neat diagrams.

b)	Calculate the flux in the core of a single-phase transformer having a primary voltage of 230 V, at 50 Hz and 50 turns.  If the flux density in the core is 1 Tesla, calculate the net cross-sectional area of the core.

4. a) 	Obtain the equivalent circuit of a single-phase transformer. Explain how to evaluate the equivalent circuit of a transformer from open circuit & short circuit tests.

b) 	A 5 kVA, 220 / 110 volts, 1-phase transformer has a maximum efficiency of 96.97 % at 0.8 p.f. lagging. It has a core loss of 50 watts and the full load regulation at 0.8 p.f. lagging is 5 %. Find the efficiency and regulation at full load 0.9 p.f. lagging.

(Contd…2)

Code No: 220402.			.. 2 ..					Set No: 1

5. a) 	With usual notation deduce the expression for starting torque of a 3-phase induction motor.

b) 	The rotor of a 3-phase induction motor has 0.04  resistance per phase and 0.2  standstill reactance per phase. What external resistance is required in the rotor circuit in order to get half of the maximum torque at starting? Neglect stator impedance. By what percentage will this external resistance change the current and pf at starting?

6. a) 	A 6-pole 3-phase, 50Hz alternator has 12 slots per pole and four conductors per slot. The winding is five-sixths pitch. The flux per pole is 1.5wb; the armature coils are all connected in series. The winding is star connected. Calculate the induced e.m.f per phase.

b) 	A 3-phase, 10KVA, 400V, 50Hz alternator has per phase armature resistance and synchronous reactance of 0.5ohm and 2.5 ohm respectively. Calculate the generated voltage corresponding to (i) full load unity power factor (ii) full load 0.85 lagging power factor.

7. a)	 Explain the principle of a  pole synchronous motor.

b) 	A 750KW, 11KV, 3 phase star connected synchronous motor has a synchronous 	reactance of 35 ohm per phase and negligible armature resistance. Determine the 	excitation e.m.f. per phase when the motor is operating on full load at 0.6 p.f 	leading. Its efficiency under this condition is 90%.

8. a) 	How is the direction of rotation reversed for each of the following:

(i) Resistance –start induction run motor and

(ii) Capacitor start induction run motor.

b) 	Explain the construction and operation of split – phase induction motor.

~ ~ ~ ~

Code No: 220402.

Set No:

2

II B.Tech. II-Semester Supplementary Examinations, November 2003

ELECTRICAL TECHNOLOGY

( Common to Computer Science and Engineering, Computer Science and Information Technology, Electronics and Communications Engineering,

Electronics and Computers Engineering, Computer Science and Systems Engineering, Electronics and Telematics  )

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1. a)	Explain how a.c. voltage generated is converted to D.C. voltage in a generator?

b)	What is the main purpose of laminating the armature core of a D.C. Generator.

c)	A 4-pole, long shunt, lap wound generator supplies 25kw at a terminal voltage of 500 V. The armature resistance is 0.03, series field resistance is 0.04 and shunt field resistance is 200. The brush drop may be taken as 1 V. Determine the e.m.f. generated.

2. a)	What are the applications of (i) D.C. Shunt motor (ii) D.C. Series Motor and

(iii) D.C. Cumulative Compound Motor.

b)	Draw typical Load-Speed characteristics of a D.C. Series and Shunt Motors and give difference in them.

c)	A 220V d.c. shunt motor with constant field drives a load whose torque is proportional to the speed. When running at 1000rpm it takes 30A. Find the speed at which it will run if a 10 resistance is connected in series with its armature. The resistance of armature may be neglected.

3. a) 	Explain the principal of operation of transformer.  Derive its e. m. f. equation.

b) 	A 1-phase transformer has 180 turns respectively in its secondary and primary 	windings. The respective resistances are 0.233 and 0.067. Calculate the 	equivalent resistance of (i) the primary in terms of the secondary winding, (ii) the 	secondary in terms of the primary winding, and (c) the total resistance of the 	transformer in terms of the primary.

4. a) 	Obtain an expression for the regulation of a single-phase transformer from its equivalent circuit / phasor diagram.

b) 	A 20 kVA, 2500 / 250 volts, 50 Hz, 1-phase transformer gave the following test results:                O.C. test (L.V. side): 250 V, 1.4 A, 105 W;

S.C. test (H.V. side): 104 V, 8 A, 320 W.

Compute the parameters of the approximate equivalent circuit referred to L.V.

(Contd…2)

Code No: 220402			.. 2 ..					Set No: 2

5. a) 	Derive the condition for maximum torque at starting of a 3-phase induction 	motor.

b) 	A 100 kW, 3-phase, 420 V, 6-pole, 50 Hz wound rotor induction motor, with its 	rotor winding short circuited, has a full-load slip of 0.04 and the slip at maximum 	torque of 0.2 when operating at rated voltage and frequency. Neglect stator 	resistance and rotational losses. Find (i) maximum torque, (ii) starting torque and 	(iii) full-load rotor ohmic loss. The rotor resistance is now doubled by adding 	external series resistance. Determine (iv) slip at full-load output (v) full-load 	torque (vi) slip at maximum torque.

6. a) 	Discuss in details the predetermination of regulation of an alternator from the open circuit and short circuit tests.

b)	A 2000 V, single-phase alternator was tested on open circuit and short circuit. The details of which are as fallows:

A field current of 2.5A produced a short circuit current of 100 A. With open 	circuit, the same field current generated an e.m.f. of 500V. The effective 	resistance of the armature is 0.8 ohm. Calculate the regulation at full load current 	of 100A.

(i) at a power factor of 0.75 lagging (ii) at a power factor of 0.65 leading.

7. a)	A 500 V, 50 Hz, Single-phase synchronous motor takes 50A current at a power factor of 0.8 lagging. The motor has a synchronous reactance of 2ohm and negligible resistance. The armature has 120 full pitch coils in series, with a distribution factor of 0.95. Assuming a sinusoidal variation of flux in the air gap. Calculate the flux per pole.

b)	Derive expressions for distribution factor and pitch factor.

8. a) 	Discuss the function of an a.c. tachometer.  Explain its construction and 	operations.

b)	What are the applications of stepper motor and synchros?

~ ~ ~ ~

Code No: 220402.

Set No:

3

II B.Tech. II-Semester Supplementary Examinations, November 2003

ELECTRICAL TECHNOLOGY

( Common to Computer Science and Engineering, Computer Science and Information Technology, Electronics and Communications Engineering,

Electronics and Computers Engineering, Computer Science and Systems Engineering, Electronics and Telematics  )

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1. a)	With the help of O.C.C. explain how voltage is build up in a D.C. shunt generator.

b)	State the reasons for droop in terminal voltage of a D.C. shunt generator when it is loaded.

c)	The magnetization curve of a d.c. shunt generator running at 1000rpm is as follows :

Field amperes : 	0.25	0.5	1.0	1.5	2.0	2.5	3.0

EMF Volts		36.0	72.0	138.0	188.0	225.0	250.0	270.0

Find (i) the value of field resistance to give 240V on no-load (ii) the speed at 	which 	the generator just fails to build up.

2. a)	State the reasons for drop in speed of a D.C. shunt motor when it is loaded.

b)	Explain why a D.C. series motor is best suited for Electric traction applications.

c)	Explain why a D.C. shunt motor can be referred as Constant Speed Motor.

d)	250 V 	d.c. shunt motor takes 41 A at full load. Resistances of motor armature and 	shunt 	field windings are 0.1  and 250  respectively. Find the back emf on full 	load. What will be its generated emf, if working as generator and supplying?

41 A to load at terminal voltage of 250 V.

3.	Discuss the constructional details of single-phase transformer and hence obtain the expression for induced e. m. f. of transformer.

4. a) 	Develop the equivalent circuit of a single-phase transformer.

b) 	A 200 kVA, 1- phase, 3300 / 400 V transformer gave the following results in the 	short circuit test with 200 V applied to the primary and secondary short circuited, 	the primary current was full load value and the input power was 1650 W.  	Calculate the secondary potential difference and the % of regulation when the full 	load current was passing at a 0.707 p. f. lagging with normal primary voltage.

(Contd…2)

Code No: 220402			.. 2 ..					Set No: 3

5. a) 	Draw the torque-speed characteristics of a 3-phase induction motor and mark 	stable and unstable regions.

b) 	A 3-phase, 4-pole, 60 kW, 50 Hz induction motor connected to rated supply 	voltage and running without load consumes 3 kW. When prevented from rotating 	it draws rated current at 30% rated supply and takes a power input of 4 kW. 	Assuming that under rated load conditions, the stator and rotor copper losses are 	equal and that the mechanical losses are 30% of the no-load losses, determine

(i) slip, at rated load and (ii) starting torque of the motor with rated applied 	voltage.

6. a) 	Explain how open circuit and short circuit tests are conducted on a synchronous machine.

b)	The synchronous reactance per phase of a 3-phase star connected 6600v 	synchronous motor is 10 ohm. For certain load, the input is 100 kW at normal 	voltage and the induced e.m.f is 9000 V. Determine the line current and the power 	factor. Neglect the armature resistance.

7. a)	Explain the working principle of the synchronous motor on no load and on load with the help of phasor diagrams.

b) 	A 3phase, 440V, 50Hz star connected synchronous motor develops 7.4 kW. The effective resistance per phase of the stator winding is 0.5 ohm. The motor operates at power factor of 0.75 lagging. Iron and mechanical losses amount to 500W and the excitation loss is 650W. Calculate (i) armature current

(ii) Efficiency of the motor.

8. a) 	What is a synchro? Draw a connection diagram of a synchro transmitter 	and 	explain its working.

b) 	Explain the working principle of split-phase capacitor-run single phase induction 	motor.

~ ~ ~ ~

Code No: 220402

Set No:

4

II B.Tech. II-Semester Supplementary Examinations, November 2003

ELECTRICAL TECHNOLOGY

( Common to Computer Science and Engineering, Computer Science and Information Technology, Electronics and Communications Engineering,

Electronics and Computers Engineering, Computer Science and Systems Engineering, Electronics and Telematics  )

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1. a)	Why do iron losses occur in a D.C. machine?

b)	Mention the factors on which Iron losses in the armature of a D.C. machine depend upon. How these losses are reduced?

c)	A d.c. machine is rated at 5 kW, 250 V, 2000 rpm. The armature resistance Ra is 1. Driven from the electrical end at 2000 rpm the no-load power input to the armature is Ia = 1.2 A at 250 V with the field winding (Rf = 250 ) excited by               If = 1 A. Estimate the efficiency of the machine as a 5 kW generator.

2. a)  	Derive an expressions for torque of a d.c. motor.

b)	Explain how the torque of a D.C. Shunt and D.C. series motor varies with the speed of the motor.

c)	The armature resistance of a 220 V d.c. shunt motor is 0.4 and it takes a no-load armature current of 2 A and runs at 1350 rpm. Find the speed when taking an armature current of 50 A if armature weakens the flux by 2%.

3. a) 	Write short notes on the construction of transformers.

b) 	Develop the phasor diagram of a single-phase transformer under loaded 	conditions.

4. a) 	With neat circuit diagrams, explain the procedure for conducting OC and SC tests on a given single-phase transformer to predetermine its regulation and efficiency.

b) 	A 100 kVA, 1000 V / 10000 V, 50 Hz, single-phase transformer has an iron loss of 1200 W.  Find the maximum efficiency at 0.8 power factor lagging if the copper loss is 500 W with 6 A in high voltage winding.  Also calculate the corresponding regulation if the equivalent leakage reactance referred to HV is

10 ohms.

5. a) 	Explain various power stages of  a 3-phase induction motor.

b) 	A 3-phase induction motor with r2 / x2 = 0.5, has a starting torque of 25.0 Nm. For negligible stator impedance and no-load current, determine the starting torque in case the rotor-circuit resistance per phase is (i) doubled (ii) halved.

(Contd…2)

Code No: 220402			.. 2 ..					Set No: 4

6. a) 	Explain neatly the constructional features of an alternator.

b) 	A 3 ph, 10-pole alternator has 2 slots per pole per phase on its stator with

10 conductors per slot. The air gap flux is distributed sinusoidally and equals to 	0.005wb.The stator has a double layer winding with a coil span of 150 electrical 	degrees. If the alternator is running at 600r.p.m, Calculate the e.m.f generated per 	phase at no load.

7. a)	Describe any two methods of starting the synchronous motors.

b)	The input to an 1100 V, 3 phase star connected synchronous motor is 60 A. The effective resistance and synchronous reactance per phase is 1 ohm and 30 ohm respectively. Find the power supplied to the motor and the induced e.m.f for a power factor of 0.95 leading.

8. a) 	Draw a diagram showing the construction of a stepper motor and discuss  its 	operation.

b) 	Discuss the various applications of stepper motor.

~ ~ ~ ~