Code No.220403

Set No.

1

II B.Tech. II-Semester Supplementary Examinations, November-2003

E M WAVES AND TRANSMISSION LINES

(Electronics and Communications Engineering and Electronics and Telematics)

Time: 3 hours							Max.Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a) 	A uniform line of length 2 m with total charge 3 nC is situated coincident to the  Z-axis with its center point 2m from the origin. At a point on the x-axis, 2m from the origin, find V and E.

b) 	A point charge of 3 nC is on the Z-axis, 2 m away from the origin. Find the 	resultant V and E.

2.a) 	Explain the concepts and significance of vector magnetic potential, and list out its mathematical relations for line, surface and volume current distributions.   Obtain its divergence and curl, and hence find out  the significance of ∇2 A, for the static fields case.

b)  	If  H  =  y  X – x  Y  A/m. on z = 0 plane, determine the current density. Define 	the relations used.

3.a)	In free space D = Dm Sin (ωt +Z)ax.  Determine B and displacement current density.

b)	Region 1, for which r1 = 3 is defined by x <	 0 and region 2, x < 0 has r2 = 5 	given H1 = 4 ax + 3ay – 6 az (A/m).  Determine H2 for x> 0 and the angles that H1 	and H2 make with the interface.

4.	An EM wave in dielectric medium 1(1, o) impinges obliquely on a boundary plane with dielectric medium 2(2, o).  Let I, t denote the incident and refraction angles respectively and show that for perpendicular polarization, reflection coefficient is equal to

sinθt−θisinθi−θt   and

transmission coefficient = 2SinθtCosθiSinθi+θt

5.a)	Explain the significance of TEM wave in a parallel plane guide, and derive an expression for the attenuation factor for TEM waves.

b)	Explain and sketch the nature of variations of attenuation with frequency in a 	parallel plate wave guide for TE, TM and TEM waves.

(Contd…2)

Code No.220403				-2-			Set No.1

6.a) 	Discuss about characteristics of TE  and  TM  waves propagating in a rectangular 	wave guide, deriving the necessary  relations.

b) 	Sketch and explain the electric and magnetic field configurations for TE10 mode 	in a rectangular wave guide

7.a)	Define the following:	(i)	Infinite line.

(ii)	Insertion loss

- Lossy and loss less lines

- Phase and group velocities

b) 	Derive the characteristic impedance of a transmission line in terms of its line 	constants.

8.a)	Give a neat sketch for a smith chart and explain clearly, step by step how would you use this chart to:

(i)	Calculate the complex reflection coefficient.

(ii)	Transfer impedance from one point to other along the line.

(iii)	Determine the length and location of a short circuited stub line for impedance matching purpose.

- Discuss the merits and demerits of stub matching techniques.

***     ***     ***

Code No.220403

Set No.

2

II B.Tech. II-Semester Supplementary Examinations, November-2003

E M WAVES AND TRANSMISSION LINES

(Electronics and Communications Engineering and Electronics and Telematics)

Time: 3 hours							Max.Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a) 	State Gauss’s law using divergence theorem and Gauss’s law, relate the displacement density D to the volume charge density ρV.

b) 	A sphere of radius “a” is filled with a uniform charge density of ‘v’ c/m3.  	Determine the electric field inside and outside the sphere.

2.a) 	An infinitely long conductor is bent into an L shape as shown in the figure. If       I = 5 mA, find the field and flux densities at (2, 2, 0).

b) 	Explain the principles of magnetic induction and establish the Faraday’s law.  Define the parameters involved.

3.a)	Within a perfect conductor E is always zero.  Using Maxwell’s equations show that H must be zero for time varying fields.  Can a steady magnetic field exist within a perfect conductor?  Show that the normal component of B, tangential component of E must be zero at the surface of a perfect conductor for both static and time varying fields.

b)	A copper wire carries a conduction current of 1 amp.  Determine the displacement current in the wire at 100 MHz.  Take  = o and conductivity = 5.8 x 107 mho/m.

4.	An EM wave in dielectric medium 1[1, o] impinges obliquely on a boundary plane with dielectric medium 2[2, o].  Let i , t denote the incident and refraction angles respectively and show that for Parallel polarization, reflection Co-efficient =	Sin 2i - Sin   2t	and

Sin 2i + Sin   2t

Transmission coefficient = 4 Sint Cos i

Sin2i + Sin2t

(Contd…2)

Code No.220403				-2-			Set No.2

5.a)	For a parallel plane wave guide having z-propogation, explain the nature of variation and sketch the variation of E and H for TM10 waves.

b)	Starting from the characteristic equation for propogation constant, establish the 	mathematical relations for the characteristics of TE and TM waves in a parallel 	plane guide.

6.a) 	A rectangular wave guide has cross section dimensions   a= 7 cm ,  b= 4 cm

determine all the modes which will propagate at a frequency of (1) 3000MHz

(2) 5000 MHz

b)	With above data, find Vp, Zo and guide wavelength for possible modes.

7.a)	Define the following terms and explain their physical significance:

- Attenuation function

- Characteristic impedance

- Phase function, and

- Phase velocity as applied to a transmission line.

b)	At 8 MHz the characteristic impedance of a transmission line is (40-j2)  and the 	propagation constant is (0.01 + j0.18) per meter.  Find the primary constants.

8.a) 	A transmission line of length 70 meters is terminated in an impedance of   ZR=125+j48.  If the frequency is 3 MHz and the characteristic impedance is      230 , find the sending end impedance using Smith chart, explaining the procedure.

b) 	What is meant by inductive loading? With the help of suitable expressions explain 	the advantages of loading and also discuss the disadvantages.

***     ***     ***

Code No.220403

Set No.

3

II B.Tech. II-Semester Supplementary Examinations, November-2003

E M WAVES AND TRANSMISSION LINES

(Electronics and Communications Engineering and Electronics and Telematics)

Time: 3 hours							Max.Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a) 	Establish Poisson’s and Laplace’s equations from Gauss’s law.

b) 	Obtain the expressions for the far field and the potential due to a small Electric 	dipole oriented along z-axis.

2.a) 	Obtain the fields due to an infinite sheet of current  in  Z = 0 plane, carrying a 	surface current density of JSY Y A.m, for    Z> 0   and  Z< 0 regions.

b)  	Find the magnitude and nature of the resulting force between two long parallel 	straight filamentary conductors, oriented along  Z directions and separated by 	a distance of 1 cm, if they carry equal currents of 5 mA in  (i) same 	direction, and (ii) opposite direction.

3.a)	In a perfect dielectric medium, the E M wave has maximum value for E of 10 V/m with r = 1 and r = 4.  Find the velocity of the wave, peak poynting vector, average poynting vector, impedance of the medium and peak value of the magnetic field.

b)	What is the inconsistency in Ampere’s Law?  How it is rectified by Maxwell?

Show that the total displacement current between the condenser plates connected 	to an alternating voltage sources is exactly the save as the value of charging 	current (conduction current).

4.	For an incident wave under oblique incident from medium of 1 to medium of 2 with parallel polarization.

a)	Define and establish the relations for the critical angle c and Brewster angle Br for non-magnetic media with neat sketches.

b)	Plot c and Br versus the ratio of 1/2.

5. 	For a parallel plane wave guide of 3 cm seperation, determine all the propogation characteristics, for a signal at 10 GHZ, for

a) TE10 waves

b) TEM waves

Explain the terms used.

(Contd…2)

Code No.220403				-2-			Set No.3

6.	An air dielectric L-band rectangular wave guide has  a/b  = 2  and a dominant mode cutoff frequency of 0.908 GHz. If the measured guide wave length is        40 cm, 	find the operating frequency, the guide dimensions, and wave number.  For the above problem  find the lowest frequency  at which a  TE21 mode would propagate.  Derive the relations used.

7.a)	Prove that a line of finite length and terminated by its characteristic impedance Z0 is equivalent to a line of infinite length.

b)	Draw the equivalent circuit of a transmission line and explain all parameters for 	the cases of (i) lossy lines, (ii) lossless line

8.a)	Explain what is meant by voltage reflection coefficient in a transmission line.  The voltage reflection coefficient due to load connected to a loss less transmission line of characteristic impedance 50  and working at 3 GHz is 0.5 |45o.  Assuming the load voltage to be 10 V, calculate the r.m.s voltage and current at intervals of one-eight wave length from the load up to a distance 5 cm.

b) 	A 75  line is terminated by a load of 120 + j80 .  Find the maximum and 	minimum impedances on the line.

***     ***     ***

Code No.220403

Set No.

4

II B.Tech. II-Semester Supplementary Examinations, November-2003

E M WAVES AND TRANSMISSION LINES

(Electronics and Communications Engineering and Electronics and Telematics)

Time: 3 hours							         Max.Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a) 	Derive an expression for the electrostatic energy stored in a capacitor of value C in terms of the total charge Q as well as the voltage V.

b) 	Consider two thin infinitely long concentric conducting cylinders with inner 	cylinder of radius a and outer cylinder of radius b. Derive an expression for its 	capacitance, with air as dielectric, assuming required charge distribution.

2.a) 	For the magnetic potential given by    A    =  x2   y  X  +  y2   x   Y  -  2 x y z   	Z  Wb/m, evaluate the magnetic field intensity at (1,2,3),  and the resulting 	magnetic flux through the surface given by  z = 1,  0 ≤ x ≤ 1,  - 1 ≤ y ≤ 1.

b) 	Explain the significance of  A    with reference to the magnetic and electric 	scalar potentials.   How are its units defined?  What are its applications?

3.	A 3 GHz, Y-polarized uniform plane wave propogates in the +X direction in a non-magnetic medium having a dielectric constant 2.5 and a loss tangent 10-2.

a)	Determine the intrinsic impedance, the wave length, the phase velocity and the group velocity of the wave in the medium.

b)	Determine the distance over which the amplitude of the propogating wave will be cut in half.

c)	Assuming Ē = 50e-x Sin (6 109 t =  /3) ây V/m, determine ‘’ and write the instantaneous expression for H for all t and x.

4.	A uniform plane wave of angular frequency ‘ω’ in a medium 1 having a refractive index ‘n1’ is incident on a plane interface at Z=0 with medium 2 having a refractive index ‘n2’ (< n1) at the critical angle.  If Eio and Eto denote the amplitudes of the incident and refracted electric field intensities respectively,  Find (a) the ratio Eto/Eio for perpendicular polarization

(b) the ratio Eto/Eio for parallel polarization

5.a)	Discuss the nature of variation of wave impedances for TE, TM and TEM waves with frequency in a parallel plane wave guide.

b)	Define and derive the equations for phase and group velocities in a parallel plane 	guide. On what factors do they depend?

(Contd…2)

Code No.220403				-2-			Set No.4

6.	A lossless air dielectric wave guide for an S-band radar has inside dimensions         a = 7.214 cm and  b = 3.404 cm. For the TM11 mode propagating at an operating frequency that is  1.1 times the cut off frequency of the mode , calculate

  - critical wave number  (b) cutoff frequency  (c) operating frequency

(d) cutoff wavelength     (e) operating wavelength (f) guide wavelength

(g) phase velocity  (h) wave impedance (i) group velocity.

7.a)	Explain the different types of transmission lines. What are the limitations to the maximum power that they can handle?

b)	A coaxial line with an outer diameter of 8 mm has 50 ohm characteristic         impedance. If the dielectric constant of the insulation is 1.60, calculate the inner diameter.

c)	Describe the losses in transmission lines.

8.a) 	Explain the method of determining the input impedance of line using Smith             chart, for a lossless line of length, at any frequency f, for a complex load of ZR.

b) 	A lossless line of 300  is terminated by a load of ZR.  If the VSWR at 200 MHz is 4.48, and the first Vmin is located at 6 cm from the load.  Calculate the reflection coefficient and ZR.

***     ***     ***