Code No: 220404.

Set No:

1

II B.Tech. II-Semester Supplementary Examinations, November 2003

ANALOG COMMUNICATION

( Common to Electronics and Communications Engineering, Electronics and Telematics  )

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1. 	Write short notes on:

a) Facsimile receiver		b) Tones in automatic telephone exchange

c) Telegraphic codes.

2. a) 	Explain the concept of frequency translation using the spectrum of DSB-SC wave.

b)	Explain with block diagram the phase-shift method of sideband suppression.

3. a)	 Distinguish between phase and frequency modulation.  Show that FM can be derived using PM and vice versa with the help of differentiator or integrator networks.

b)	Compute the bandwidth requirement for the transmission of FM signal having a frequency deviation 75 KHz and an audio bandwidth of 10KHz.

c)	In a FM system the frequency deviation constant is 1KHz/v.  A sinusoidal modulating signal of amplitude 15 V and frequency 3 MHz is applied. Calculate (i)Peak frequency deviation(ii)Modulating index.

4. a) 	Explain the envelope detector with a circuit diagram and waveforms.

b)	Draw the Foster-Seely discriminator and explain.

5. a)	Why  frequency  drift & scintillation should be very small in radio transmitter.

b)	Give & explain radio frequency spectrum used for various communications.

c)	Draw the block diagram of a filter type SSB-SC transmitter with 20 KHZ 	oscillator and emission frequency in the range of 6 MHZ.  Explain the function 	of each stage.

6.	Write short notes on the following:-

a) 	Automatic gain Control         b) 	Diode director          c) 	Mixers.

7. a) 	Explain the function of each block in FM receiver.

b) 	Write a survey of the desirable features of communication receivers and briefly 	explaining the operation of each where necessary.

8.	Obtain the expression for SNR &Figure of merit of coherent reception of SSB 	modulated wave.

~ ~ ~ ~

Code No: 220404.

Set No:

2

II B.Tech. II-Semester Supplementary Examinations, November 2003

ANALOG COMMUNICATION

( Common to Electronics and Communications Engineering, Electronics and Telematics  )

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1. a) 	State the advantages of cross bar exchange. Explain in detail how a call is 	established between two subscribers in it.

b) 	Draw and explain the trunking diagram of 1000 line exchange. What are the 	reasons for each subscriber being provided with one uniselector.

2. a) 	Explain the operation of square law modulator.

b) 	Explain the phase discrimination method for generating an SSB modulated wave.

3. a)	The sinusoidal modulating wave mt = Am cos2Πfmt is applied to a phase modulation with phase sensitivity Kp.  The unmodulated carrier wave has frequency fc and amlititude Ac.  Determine the spectrum of the resulting phase modulated wave, assuming that the maximum phase deviation βp = Kp Am does not exceed 0.5 radians.

b)	A carrier wave of frequency 100 MHz is frequency modulated by sine wave of 	amplitude 20 volts and frequency 100 KHz.  The frequency sensitivity of the 	modulation is 25 KHz per volt.  Determine the approximate bandwidth of FM 	wave using Carson’s rule.

4. a)	An FM signal Xct= A0cosω0 t+β sin ωmtis applied as input to an RC high pass network. Assume that 1ωRC>>1in the frequency band occupied by Xct. Show that the output voltage of RC network is an AM signal. Find the modulation index of AM signal.

b)	What is zero crossing detectors? Explain how it works and can be used as an FM 	demodulator?

5. a)	An AM transmitter of  1KW power is fully modulated. Calculate the power 	transmitted if it is transmitted as SSB.

b)	Calculate the filter requirement to convert DSB signal to SSB Signal, given that 	the two side bands are separated by 200HZ. The suppressed carrier is 29 MHZ.

c)	Give and explain 3 areas of applications where standard FM transmission is 	needed?

(Contd…2)

Code No: 220404.				.. 2..				Set No: 2

6.	Write short notes on:

a) Image frequency and its reduction.

b) fading and diversity reception.

c) Squelch circuit.

7. a) 	Give a block diagram of a typical frequency modulated broadcast           	receiver and explain each block.

b) 	Write about squelch circuit and BFO.

8.	Obtain the expression for SNR &Figure of merit of coherent reception of DSB 	modulated wave.

~ ~ ~ ~

Code No: 220404.

Set No:

3

II B.Tech. II-Semester Supplementary Examinations, November 2003

ANALOG COMMUNICATION

( Common to Electronics and Communications Engineering, Electronics and Telematics  )

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1. a) 	Describe Morse code and five unit code and give their relative merits and 	demerits.

b)  	How is the speed of a telegraphic transmission expressed? Is it same for all the 	different types of telegraphic codes used.

2. a) 	What are the disadvantages of SSB-SC over normal AM. And compare AM

and FM.

b) 	Explain the frequency discrimination method for generating an SSB modulated 	wave.

3. a)	In an Armstrong Modulator the crystal oscillator frequency is 200 KHz.  It is desired in order to avoid distortion to limit the maximum angular deviation to    φm = 0.2. The system is to accommodate modulation frequencies down to 40Hz.  At the output of the modulator the carrier frequency is to be 108 MHz and the frequency deviation be 80KHz.  Select multiplier and mixer oscillator frequencies to accomplish this.

b)	Explain the effect of random noise on the output of an FM receiver fitted with 	amplitude limiter.  Develop the concept of noise triangle.

4. a)	Compare and contrast the performance and applications of the various types of frequency demodulation techniques.

b)	Explain the operation of the balanced slope detector using a circuit diagram and 	draw its response characteristics. Discuss in particular the method of combining 	the outputs of the individual diodes. In what way is this circuit an improvement 	on the slope detector and in turn what are the advantages?

5. a)	Explain the  operation of ISB transmitter with  block diagram. Where it is used?

b)	What is the function of crystal filters in SSB transmitter?

c)	State and explain with respect to 'Q', various types of filters used to separate side 	bands?

6. a)	Explain the necessity for AGC in a radio communication.

b)	Discuss the consideration that governs the choice of IF in a receiver.

c)	Explain the operation of superheterodyne receiver.

(Contd…2)

Code No: 220404.				.. 2..				Set No: 3

7. a) 	Explain the working of a limiter with neat circuit diagram.

b) 	What is fading phenomenon? Explain in detail how diversity techniques are      	helpful in reducing the effect of fading.

8. a)	Show that for AM, when the noise is small compared to the signal the 	performance of the envelope detector is identical to that of synchronous detector.

b)	What is the use of calculating noise figure?

~ ~ ~ ~

Code No: 220404.

Set No:

4

II B.Tech. II-Semester Supplementary Examinations, November 2003

ANALOG COMMUNICATION

( Common to Electronics and Communications Engineering, Electronics and Telematics  )

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1. a) 	Describe in detail, the working of a cross bar telephone exchange.

b) 	Derive the Erlangs formula for traffic carried by each trunk.

2. a) 	Explain clearly the difference between AM, FM, and PM, beginning with the definition of each type and the meaning of the modulation index in each case.

b) 	Compare the various methods of DSB.

3. a)	In a FM system if modulation  index is doubled by halving modulating frequency, what will be the effect on the maximum deviation.

b)	FM is called a ‘Constant Band Width’ system – Justify with suitable illustrations.

c)	An FM wave with modulation index β = 1 is transmitted through an ideal band 	pass filter with mid band frequency fc and bandwidth is 5 fm, where fc is the 	carrier frequency and fm is the frequency of the sinusoidal modulating wave.  	Determine the amplitude spectrum of the filter output.

4. a)	Explain demodulation of FM signal with the help of PLL.

b)	Consider a square law detector using a non linear device whose transfer characteristics is defined by  v2 t =a1 v1 t + a2 v12t where a1 and a2 are constants. v1 is the input and v2 is the output.  The input consists of an AM wave v1 t = Ac1+Ka mtcos2Π fc t.Evaluate v2t and find the condition to extract the signal  v2t  .

5. a)	Explain the  operation of  plate  modulated AM generator with  circuit  diagram.

b)	Explain the operation of high level transistor collector modulator circuit.

6. a)	Distinguish between simple AGC and delayed AGC.

b)	Draw a block diagram of a superheterodyne receiver and explain the function of 	each stage.

c)	What is meant by the term “tracking error”? Explain.

(Contd…2)

Code No: 220404.				.. 2..				Set No: 4

7.	Write short notes on the following:-

a)	Receiver measurements.

b)	Squelch circuits.

c)	Noise limiter.

8.	Show that for tone modulation for a fixed peak power transmitted the output SNR of AM is 5db below that DSB-SC.

~ ~ ~ ~