Code No:220803

Set No.

1

II B.Tech., II Semester Supplementary Examinations, November-2003

FLUID MECHANICS

(Chemical Engineering)

Time:3 hours							Max Marks:80

Answer any Five Questions

All questions carry equal marks.

-----

1.a)	What is the resultant pressure on an immersed surface in a static fluid . Derive an equation for the same.  How do you find the centre of pressure.

b)	What is Pascal’s Law?

2.a)	The water flows in a pipe line, which changes from 20cm at one end ‘A’ to

50cm at another end ‘B’.  Pressures at two ends are 78.5 KN/m2 and

58.9 KN /m2 respectively.  The flow rate is 200 litres per second.  End B is

higher than end A by 3m.  Determine the direction of flow.

b)	What are the applications of Bernoullis equation.

3.a)	Derive an expression for the loss due to sudden enlargement in pipe flow.

b)	At a sudden enlargement of a water main from 240mm to 480 mm diameter the

hydraulic gradient rises by 10mm .  Estimate the rate of flow.

4.a) 	Define Mach number. What is the significance of mach number in compressible fluid flows.

b) 	A jet airplane is propelled at a velocity of 1000km/hr through air in which the atmospheric pressure is 88.3kN/m2. The temperature is –5oC and the wind velocity is negligible. Calculate the pressure intensity, the temperature, the density of air at the stagnation point on the nose of the jet, and determine its Mach number. Take R=290Nm/kg oK and k=1.4.

5.	A cyclone separator is used to remove sand grains from an air stream at 1500C. If the cyclone body is 0.6 m in diameter and the average tangential velocity is 16 m/s, what is the radial velocity near the wall of particles 20 and 40µm in size. How much greater are these values than the terminal velocity in gravity settling.

6.a) 	Explain vaccum pumps and Jet ejectors in detail.

b) 	Compare the various devices used for moving fluids	.

7.a)  	What are the advantages and disadvantages of centrifugal pumps?

b)  	What is NPSH? What happens if the required NPSH is not provided?

8.	A horizontal venturimeter having a throat diameter of 20mm is set in a 75mm  Internal diameter pipeline.  A manometer containing mercury under water measures the pressure differential over the instrument.  When the manometer reading is  500mm,  what is the flow rate?  If 12 percent of the differential is permanently lost, what is the power consumption of the meter?

!!!!!

Code No:220803

Set No.

2

II B.Tech., II Semester Supplementary Examinations, November-2003

FLUID MECHANICS

(Chemical Engineering)

Time:3 hours							Max Marks:80

Answer any Five Questions

All questions carry equal marks.

-----

1.a) 	Define (i) Total pressure (ii) Centre of pressure

b) 	Explain with a neat sketch the pressure distribution due to a static liquid                                                                                                                              	on a vertical surface not touching the liquid surface.

2.a)	Derive the continuity equation for three dimensional flow.

b)	Check whether the following velocity field represents possible flow

V=(x+y)i + (y+z)j + (x2+y2+z2)k.

3.a)	How do you find the presence of laminar flow or turbulent flow in a flow Phenomenon.

b)	What are characteristics of laminar flow?  What are the practical cases where

laminar flow is predominant.

4.a)	Show by means of diagrams the nature of propagation of disturbance in compressible flow when mach number is less than one, is equal  to one and is more than one.

b) 	A gas is flowing through a horizontal pipe which is having area of cross section as   30 cm2 and pressure is 40N/ cm2 (gauge) and temperature 15oC. At another section the area of cross - section  is 20 cm2 and pressure is 30 N/cm2 (gauge). If the mass rate of flow of gas through the pipe is 0.5 kg/s, find the velocities of the gas at these sections, assuming an isothermal change. Take R= 292 N-m/kgo K, and atmospheric pressure = 10 N/cm2.

5.	A fluidized bed containing 32,500 kgs of 0.15 mm sand is to be fluidized with air at 400oC and a pressure of 17 atm (absolute) in a cylindrical vessel of 3.05m in diameter. The density of particle is 2690 kg/m3. The viscosity of air is 0.032 centi poise. Calculate  (a) Minimum height of fluidized bed   (b) The pressure drop in the bed and   (c) Critical superficial air velocity. Minimum bed porosity is 0.55.

6.a)	In designing and installing a piping system what are the points that are to be 	considered.

b)	What are the advantages and disadvantages of positive displacement pumps.

7.	Write short notes on :

- Impellers		(b)	Centrifugal  pump characteristics.

8.	Write short notes on :

(a) Pressure Recovery		(b) Polytropic compression

!!!!!

Code No:220803

Set No.

3

II B.Tech., II Semester Supplementary Examinations, November-2003

FLUID MECHANICS

(Chemical Engineering)

Time:3 hours							Max Marks:80

Answer any Five Questions

All questions carry equal marks.

-----

1.a)	Differentiate between total pressure and center of pressure.

b) 		Obtain an expression for the horizontal and vertical components of the resultant 			force on a submerged curved surface.

2.a)	What are the types flow.

b)	Does the velocity distribution given by

V=i(4-x2+y)+j(3+2y-z)+k(x-1) represent possible fluid flow.

3.a)	What do you understand by turbulent flow? What factor decides the type of flow in

pipes

b)	Derive an expression for the loss of head due to friction in pipes.

4.a)	What do you mean by compressibility correction factor. Find an expression for compressibility factor.

b) 	Find the velocity of air flowing at the outlet of a nozzle fitted to a large vessel which contains air at a pressure of 294.3 N/cm2 (abs) and at a temperature of 20oC. The pressure at the outlet of the nozzle is 206 N/cm2(abs).

Take k= 1.4 and K= 287 J/kg Ko.

5.	Distinguish clearly between free settling and hindered settling. Calculate the diameter of a quartz sphere of specific gravity 2.65 settling in water under stoke’s condition at 200 C. The viscosity of water at 200 C is one centi poise.

6.a)	What is the necessity of providing bends or loops in the pipe

b)	Name different types of fluid moving machinery.

c)	Write short notes on pipe fittings.

7.	Write short notes on :

a)   Compressors

b)   Blowers

c)   Multistate centrifugal pumps.

8.	Explain Orifice meter in detail.  Also derive an expression for discharge through 	an orifice meter.

!!!!!

Code No:220803

Set No.

4

II B.Tech., II Semester Supplementary Examinations, November-2003

FLUID MECHANICS

(Chemical Engineering)

Time:3 hours							Max Marks:80

Answer any Five Questions

All questions carry equal marks.

-----

1.	What do you understand by the hydrostatic pressure variation? With the help of this equation derive the expression for the total thrust on a submerged inclined plane area.

2.a)	Derive the Bernoullis equation along a stream line.  State the assumptions made.

b)	Write any two applications of the Bernoullis equation.

3.a)	How will you determine the loss of head due to friction in pipes by using.

(i) Darcy formula .

b)	Find the diameter of a pipe of length 1750m when the rate of flow of water through

the pipe is 175 litres/sec and the head lost due to friction is 4m.

4.a)	Find an expression for mass rate of flow of compressible fluid flow through  an     orifice fitted to a large tank. What is condition for maximum rate of flow.

b) 	Find the Mach  number when an aeroplane is flying at 1100km/hour through still air having a pressure of 7N/cm2 and temperature –5oC. Wind velocity may be taken as zero. Take R= 287.14 J/kg Ko. Calculate the pressure and temperature of air at stagnation point on the nose of the plane. Take k=1.4.

5.	Develop the relation between drag coefficient and particle Reynold’s number in the stokes law region. Calculate the free fall terminal velocity of spherical particles of coffee extract 400 microns in diameter falling through air at 1500C and 1 atm pressure. The specific gravity of coffee is 1.03 and the viscosity of air 0.026 centi poise.

6.a)	 Explain how the frictional losses in pipe fittings and valves can be accounted for 	design calculations.

b) 	How do you classify pumps?

7.	A centrifugal fan is used to take flue gas at rest and at a pressure of 730mm Hg and a temperature of 900C  and discharge it at a pressure of 760mm Hg and a velocity of

50 m/sec.  Calculate the power needed to move 17000 m3/hour of gas.  The efficiency of the fan is 60% and the molecular weight of the gas is 32.0.

8.	Air at 93.30C is forced through a long, circular flue 914mm in diameter.  A pitot tube reading is taken at the center of the flue at a sufficient distance from flow  disturbances to ensure normal velocity distribution.  The pitot reading is 13.7mm H2O and the static pressure at the point of measurement is 387mm H2O.  The coefficient of the pitot tube in 0.98.  calculate the flow of 	air measured at 15.60C and a barometric pressure of 760mm Hg.

!!!!!