Code No: 220805

Set No.

1

II B.Tech. II-Semester Regular Examinations, November-2003

MATERIAL SCIENCE FOR CHEMICAL ENGINEERS

(Chemical Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.	Explain different components of Materials Science education and their role in	Engineering and Technology.

2. a)	Explain electropositive and electronegative elements on the basis of Periodic 	table.

b)	With regard to electron configuration, what do all elements in Group VIIA of the 	periodic table have in common?

3. a)	Show that the atomic packing factor for BCC is 0.68

b)	Calculate the radius of a vanadium atom, given that V has a BCC crystal structure, a density of 596 Kg/m2 and an atomic weight of 50.9 g/mol.

4. a)	 Draw phase diagram for the system Al2O3 - Cr2O3 and explain.

b)	An alloy of tin and lead contains 73% tin. Find the mass of eutectic in one kg of solid alloy, if the eutectic contains 64% tin.

5.	What are the various strengthening mechanisms? Explain any three methods in detail. Which method do you think be superior. Why?

6. a)	Define the parabolic law of oxidation

b)	What are the common alloying elements added to steel for improving oxidation.

c)	Briefly discuss about the oxidation resistant Materials

7. a)	Explain the terms electrical conductivity,  electrical resistively and ohm’s law.

b)	Based on the electrical resistively values; classify the materials into electrical conductors and electrical insulation.

8.	Write briefly about the characteristics of paramagnetic, diamagnetic and ferromagnetic materials with suitable examples.

###

Code No: 220805

Set No.

2

II B.Tech. II-Semester Regular Examinations, November-2003

MATERIAL SCIENCE FOR CHEMICAL ENGINEERS

(Chemical Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.	How Engineering materials are classified on the basis of chemical nature and 	atomic 	structure. Give characteristic properties, application and examples under 	each category.

2.	Give the electronic configurations for the following ions: Fe2+, Fe3+, Cu+, Ba2+, 	Br-, S2-.

3. a)	 Show that the atomic packing factor for HCP is 0.74.

b)	Zirconium has an HCP crystal structure and a density of 6.51 g/cc. What is its unit 	cell in 	cubic meter?

4.	What is an isomorphous system? Give examples of alloys that are of this type. Explain the characteristics of the isomorphous system with the help of any equilibrium diagram.

5.	Discuss the role of (a) grain boundaries (b) precipitate particles, in strengthening crystalline materials against yield at low as well as high temperatures of deformation.

6. a)	Define Corrosion, give the most common example of Corrosion, What is an 	electrode potential.

b)	What is a Galvanic Cell?  Name the different types of Galvanic cells. Explain the process taking place in a Daniel Cell.

7.	Explain the behaviour of electrical conductivity and electrical resistivity of metals based on the free electron theory.

8.	Explain the characteristics of soft and hard magnetic materials with suitable examples.

###

Code No: 220805

Set No.

3

II B.Tech. II-Semester Regular Examinations, November-2003

MATERIAL SCIENCE FOR CHEMICAL ENGINEERS

(Chemical Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.	What are advanced materials? How advanced materials are developed through combination of conventional engineering materials?

2.	How will you explain the formation of covalent bonds between neutral atoms? 	Account for the fact that inert gas elements do not usually form chemical bonds

3.	Which crystal structures are mostly found in metals? Compare their packing efficiencies in terms of coordination number and atomic packing factor.

4. a)	Explain the construction of phase diagram for brass.

b)	Explain the applications of phase diagrams with special reference to manufacture of steels.

5. 	What is the effect of the following operations on the microstructure and mechanical properties of a ductile single-phase alloy initially in the cast and annealed condition?

(a)	Increasing amount of cold work.

(b)	Annealing at successively higher temperatures after cold-work.

6. a)	Describe the electroplating operation.

b)	A nickel electroplating process consumes 20 amperes. What is the rate of 	deposition of nickel per hour if there are no side reactions?

7.	Write short notes on materials used as:

i)	Electrical resistors

ii)	Electric heating elements.

8.	Deduce the net magnetic moment of Iron, Cobalt and Nickel from the electronic 	configuration of the atoms.

###

Code No: 220805

Set No.

4

II B.Tech. II-Semester Regular Examinations, November-2003

MATERIAL SCIENCE FOR CHEMICAL ENGINEERS

(Chemical Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1. a) 	How structure of materials are classified based on scale level? How do these 	influence the properties of the materials?

b)	Briefly state methods used to study structures of a material, important 	information obtained and their influence on the properties of the materials.

2. a)	How would you explain that H2O is a liquid while H2S is a gas at ordinary 	temperature, although the molecular weight of H2O is lower than that of H2S.

b)	In what respect the Vander Walls bonds resemble the metallic bonds?

3. a)	 Explain why there is no end-centred cubic space lattice.

b)	 Name the space lattices which have four lattice parameters.

4.	What is a phase diagram? Explain the construction of phase diagrams with neat sketches for metals that are completely soluble in liquid and solid states by giving examples.

5. a)	Discuss briefly the mechanism of strain –hardening and state  the factors that             influence  recrystallization.

b)     Explain the terms (i) stress relief, (ii) recrystallization, and (iii) grain growth.

- Write short notes on the following:

i)	Standard electrode potential of Metals

ii)	Electrode composition cells

7.	Briefly discuss the application of various types of good electrical conducting 	metals in power and communication lines and High tension power line.

8.	Does a large magnetic moment per atom mean a high Curie temperature? Explain

###