Code No.221001

Set No.

1

II B.Tech. II-Semester Supplementary Examinations, November-2003

ELECTRICAL AND ELECTRONICS MEASUREMENTS

(Common to Electronics and Instrumentation Engineering and Electronics and Control Engineering)

Time: 3 hours							Max.Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	What are shunts?  Describe their constructional features and materials used.  Obtain the expression for the multiplying power of a shunt.

b)	It is desired to extend the range of a DC milliammeter of the range OTO 100 mA, to measure up to 1 A.  The meter resistance R in is 1 k.  Determine the value of the shunt to be used and its multiplying power.

2.a)	With neat diagram explain the principle and working of AC voltmeter.

b)	An AC voltmeter calibrated for sine wave is used to measure a ramp voltage 	waveform rising to a peak value of 60 V in 3 m sec. Determine the percentage 	error.

3.a)	Explain a resistance range selector circuit of a VOM.

b)	To check the distributed capacitance of a coil, the coil is resonated at 15 MHz 	with 40 PF. What is the inductance of the coil and what is the equivalent 	distributed capacitance?

4.a)	What are the constituent elements of a Digital Multimeter?

b)	For measuring small values of capacitance, a 60 MHz source is to be used in a 	capacitance meter. What value of series resistance is required if the phase shift is 	to be kept below 5.7deg for full scale capacitance reading of 1, 10 and 100 PF.

5.a)	Draw a neat figure of internal structure of CRT and explain functions of various component.

- How an electron beam is focused to a fine spot on the face of the cathode ray tube?

6.a)	Explain the working function of any one active probe.

b)	Compare the performance characteristics of different types of CRO probes.

7.a) 	Distinguish between a function generator and signal generator.

b) 	Briefly explain with block diagram the operation of a function generator.

8.a)	Draw the block diagram of a frequency counter and explain its operation using appropriate waveforms.

b)	How many displays should a frequency counter have if its accuracy and resolution 	are to be 0.001% and explain why?

***     ***     ***

Code No.221001

Set No.

2

II B.Tech. II-Semester Supplementary Examinations, November-2003

ELECTRICAL AND ELECTRONICS MEASUREMENTS

(Common to Electronics and Instrumentation Engineering and Electronics and Control Engineering)

Time: 3 hours							Max.Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Explain about the loading effect in measurements and its effect on the accuracy of measurement.  How this can be prevented?

b)	A voltmeter with a sensitivity of 1000 /v is being used, to measure the voltage 	across a 40 k resistor.  The voltage is measured on 50 V range.  Determine the 	error in the reading due to loading effect.

2.a)	Draw the block diagram of a true RMS measuring instrument and explain its 	working.

b)	A symmetrical square wave voltage of maximum value Em and time period T is 	applied to an average responding AC voltmeter with a scale calibrated in terms of 	the RMS value of a sine wave. Calculate the error in meter indication.

3.a)	Explain in detail the considerations in choosing an analog voltmeter.

b)	A coil with a resistance of 3 is connected to the terminals of the Q meter. 	Resonance occurs at an oscillator frequency of 5 MHz and resonating capacity of 	100 PF. Calculate the percentage of error introduced by the insertion resistance 	Rsh = 0.1.

4.a)	How do you measure R, L and C using electronic instruments?

b)	How do you perform all-electronic capacitance measurements where the 	measurement is not performed by a comparison? Explain one method in detail.

5.	Explain in detail with figure

- Electrostatic focusing system of a CRT

- Refraction of an electron ray at an equipotential surface.

6.a)	What are the relationships between the period of a waveform and its frequency? How is an oscilloscope used to determine frequency?

b)	How are the effects of direct current on the flux density of the current probe minimized?

7.a) 	Explain with the help of block diagram the working of a spectrum analyzer.

b) 	Explain the different applications of spectrum analyzer.

(Contd…2)

Code No.221001				-2-			Set No.2

8.a)	What minimum gate time is required for a frequency counter capable of measuring an unknown frequency, to within 10 Hz, by measuring frequency rather than period? Justify your answer.

b)	Explain the display technique used frequency counters, with an example.

***     ***     ***

Code No.221001

Set No.

3

II B.Tech. II-Semester Supplementary Examinations, November-2003

ELECTRICAL AND ELECTRONICS MEASUREMENTS

(Common to Electronics and Instrumentation Engineering and Electronics and Control Engineering)

Time: 3 hours							Max.Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Draw the circuits and compare the functioning of series type and shunt type ohm meters.

b)	A series type ohmmeter uses a 50- basic movement.  It requires a full scale 	current	of 1 mA.  The internal bettry voltage is 5V.  For half scale deflection, the 	scale marking is 2 k.  Calculate the values of resistors R1 and R2 of the meter 	circuit.

2.a)	How the elements of a single phase watthour meter are connected. Explain with 	the help of a sketch?

b)	Describe the principle of measurement and working of a domestic watthour meter.

3.a)	Draw the typical metering circuit of a solid state VOM describing various elements used.

b)	What is the lowest full-scale voltage that could be displaced with a 100 A meter 	moment with an internal resistance of 150 ? What would be the sensitivity of 	this meter in ohms /volt?

4.	Write a notes on sources of error encountered in capacitance   measuring meter.  	Explain parasitic inductance, resistance and capacitance.

5.a)	Explain the working function of a post deflection acceleration oscilloscope tube using a scan expansion mesh.

b)	Briefly summarize the characteristics of commonly used phosphors.

6.a)	With a neat block diagram explain working operation of storage oscilloscope.

b)	Compare storage oscilloscope with ordinary oscilloscope.

7.a) 	Draw the circuit diagram and explain the working of a heterodyne type wave analyzer.

b) 	Explain one application of distortion factor meter.

8.a)	With the help of a block diagram explain the multiplexed display technique used in a frequency counter.

b)	A voltmeter has a working coil of copper in series with an invariable swamp 	resistance.  What must be the ratio of swamp resistance to coil resistance if the 	error introduced by a temperature rise between 15 to 25oC is not to exceed 1% 	of the indication?

***     ***     ***

Code No.221001

Set No.

4

II B.Tech. II-Semester Supplementary Examinations, November-2003

ELECTRICAL AND ELECTRONICS MEASUREMENTS

(Common to Electronics and Instrumentation Engineering and Electronics and Control Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Explain about the loading effect in measurements and its effect on the accuracy of measurement.  How this can be prevented?

b)	A voltmeter with a sensitivity of 1000 /v is being used, to measure the voltage 	across a 40 k resistor.  The voltage is measured on 50 V range.  Determine the 	error in the reading due to loading effect.

2.a)	Give the specifications and typical values of a commercial multimeter.

b)	Design a range switch to be used in the case of an electronic voltmeter, calibrated 	in the customary range of 1-3-10 sequence with a total resistance of 12 M, to 	measure from 1V to 1000 V, the meter requires 1 V for full scale deflection, at the 	gate of the FET input device.

3.a)	State the general guidelines followed when AC current measurements are made.

b)	State the general characteristics of a digital voltmeter. Name 4 types of DVM’s 	and explain the principle and working of any one.

4.a)	What do you understand by Q of an inductor, explain with reference to series resonant circuit?

b)	Give one method of measurement of Q of a coil.

5.a)	Explain the relationship between trigger pulse and the sweep in an oscilloscope.

b)	Explain the block diagram of a dual time base for delayed trigger.

6.a)	Explain the working function of secondary emission circuit with a secondary emission curve of storage oscilloscope.

b)	Explain the working function of secondary emission circuit with floating target with a secondary emission curve of storage oscilloscope.

7.a) 	With the help of a block diagram explain an audio-range wave analyzer.

b) 	Explain the applications of spectrum analyzer.

8.a)	If the internal time base of a frequency counter is 10 MHz, what frequency range is best measured by a conventional frequency measurement and why?

b)	Define long term stability and explain the reasons for it.

c)	Define trigger level error.

***     ***     ***