Code No: 221003

Set No.

1

II B.Tech. II-Semester Supplementary Examinations, November-2003

PULSE AND DIGITAL CIRCUITS

(Common to Electronics and Instrumentation Engineering and Electronics and Control Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1. a)	Verify the output voltage of a double differentiator to which an exponential input Vi=V(1-e– t/T) is applied.

b)	Find the initial derivative of the input and of the output for the above input.

2. a) 	Give the circuits of series clipper circuits and explain their operation with the help of transfer characteristics.

b)	For the circuit shown in the figure below, sketch the input and output waveforms R = 1 K

VR = 10V,    Vi = 20 Sin t

Rf = 100  Rr = 

V = 0

3.	Define rise time and fall time of transistor switch.  Derive expressions for these in 	terms of the transistor parameters and operating currents.

4. a) 	Draw the circuit diagram of self-bias with symmetrical triggering using diodes. Explain the working of the same.

b) 	Compare between triggering at base and collectors.

5. a)	Draw a circuit of a sweep circuit using a transistor switch. Explain its working with input and output waveforms.

b)	Compare a transistor-switch sweep circuit with UJT sweep circuit with respect to sweep speed and linearity of sweep.

Contd….2

Code No. 221003					.2.			Set No.1

6. a)	Illustrate the terms ‘synchronization’ and ‘frequency division’ of a sweep generator.

b)	A free-running relaxation oscillator has sweep amplitude of 100 V and a period of 1 msec. Synchronizing pulses are applied to the device such that breakdown voltage is lowered by 50V at each pulse. The synchronizing pulse frequency is     4 kHz. What is the amplitude and frequency of synchronized oscillator waveform?

7. a)	What  is  a   sampling  gate?

Explain the operation of series gate using JFET.  Sketch the input and output Waveforms.

b)	Illustrate  the  errors  encountered  in  series  sampling  and  what  is  the  design

Procedure to minimise these errors?

8.	Explain  the  operation  of  an  RC  controlled  free  running  blocking  oscillator   	with  neat  sketch  of  circuit  and  voltage  waveforms.  Derive the  expression  	for  duty  cycle.  What are  the  advantages  of  the  circuit?

###

Code No: 221003

Set No.

2

II B.Tech. II-Semester Supplementary Examinations, November-2003

PUlSE AND DIGITAL CIRCUITS

(Common to Electronics and Instrumentation Engineering and Electronics and Control Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.	An ideal 1 m sec pulse is fed to an amplifier.  Calculate and plot the output waveform under the following conditions:

The upper 3-dB frequency is (i) 10 MHz (ii) 1 MHz (iii) 0.1 MHz

2. a)  	Draw the circuit diagram of a double clipping circuit using diodes to clip at two different reference levels and draw its transfer characteristic and explain its operation.

b)	For the circuit shown in the figure below,  sketch the input and output waveforms.

If Vi = 20 cos t, R = 2 K

3. a)	Explain the phenomenon of “latching” in a transistor switch.

b)	A transistor has fT = 50 MHz, 4FE =40, Cb'c=3PF and operates with Vcc=12 V and 	Rc=500 .  The transistor is operating initially in the neighbourhood of the cut-in 	point.  What base current must be applied to drive the transistor to saturation in 	1 sec?

4. a) 	What is a direct connected binary circuit? Explain the direct connected binary with the help of a circuit diagram.

b) 	Mention the advantages and disadvantages of the above. Mention its use.

5. 	With reference to voltage sweeps explain the following terms:

a)   Sweep speed

- Linearity of sweep

- Sweep stability

- Recovery time.

Contd….2

Code No. 221003					.2.			Set No.2

6. a)	Explain about synchronization of a sweep circuits with symmetrical signals.

b)	A UJT sweep operates with a valley voltage of Vv=2V and a peak voltage Vp=12V. A sinusoidal synchronizing voltage of 2V peak is applied between bases. The stand-off ratio is h=0.5. If the natural frequency of sweep is 1kHz, over what range of synchronizing signal frequency will the sweep remain in 1:1 synchronization with the synchronizing signal?

7.	Draw the circuit of astable blocking oscillator. The astable blocking oscillator has following parameters VCC=10V, VBB=0.5V, =2, R=1.5k, Rf=10, Vr=9V, L=3mH and C=100 pF. Calculate the frequency of oscillation and the duty cycle.

8. a) 	Compare  the  diode  controlled  and  RC  controlled  astable  operated  blocking         	oscillator.

b) 	What are the advantages of RC controlled oscillator?

c) 	List the applications of blocking oscillators.

###

Code No: 221003

Set No.

3

II B.Tech. II-Semester Supplementary Examinations, November-2003

PULSE AND DIGITAL CIRCUITS

(Common to Electronics and Instrumentation Engineering and Electronics and Control Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1. a)	A symmetrical square wave whose peak-to-peak amplitude is 2V and whose average value is zero is applied to an RC integrating circuit.  The time constant is half the period of the square wave.  Find the peak-to-peak value of the output amplitude.

b)	Write a short note on RC low pass circuit.  Draw the output if a step input is applied.

2. a)	Draw the basic circuit diagram of negative peak clamper circuit and explain its 	operation.

b)	For the circuit shown in figure below, an input voltage Vi linearly varies from 0 to 150 V is applied.  Sketch the output voltage V0 to the same time scale. Assume ideal diodes.

3. a)	Explain how transistor can be used as a switch in the circuit, under what condition 	a transistor is said to be ‘OFF’ and ‘ON’ respectively.

b)	A germanium transistor is operated at room temperature in the CE configuration.  	The supply voltage is 6 V, the collector-circuit resistance is 200  and the base 	current is 20 percent higher than the minimum value required to drive the 	transistor into saturation.  Assume the following transistor parameters:

Ico=-5A, IEO=-2A, hFE=100, and rbb’=250 .

Find VBE(Sat) and VCE(Sat).

4. a) 	Explain asymmetrical triggering in a binary and mention its uses.

b) 	Design a nonsaturated bistable multivibrator using silicon n-p-n transistors. Assume that Vcc=15 v; hfe(min)=60 and hfe(max)=250. If  Ic=1.5mA, find the maximum and minimum levels of collector, 	base and emitter voltages of both transistors.

Contd….2

Code No. 221003					.2.			Set No.3

5. a)	Draw the circuit of  a simple current sweep circuit using inductor in series with a transistor, which is used as a switch and draw the inductor-current waveform.

b)	In a simple current-sweep circuit Vcc= 20 V, L=200 mH, yoke 6esidence         RL= 20 ohms, Rd= 200 ohms. For a 500 micro sec sweep draw the waveform of inductor current and calculate the slope error of the sweep.

6. a)	Explain the operation of a sweep circuit as a divider.

b)	The UJT relaxation oscillator is to be used as a 3:1 divider for pulses which occur at a 2500 Hz rate. The available supply voltage is 30 V. The pulses are applied at base B2. Draw the circuit and calculate pulse amplitude.

7. a)	Distinguish  between  logic  gate  and  sampling  gate.

b)	Why is a sampling referred as a linear gate?

c)  	Illustrate  the  principle  of  operation  of  a  linear  gate  using  series  switch  and  	shunt switch.  What are the disadvantages?

8. a)	Discuss two applications of comparators:

(i) Square wave from a Sine wave

(ii) A/D converter.

b)	Explain the method of synchronization of sweep circuits with sinusoidal signals.

###

Code No: 221003

II B.Tech. II-Semester Supplementary  Examinations, November-2003

Set No.

4

PULSE AND DIGITAL CIRCUITS

(Common to Electronics and Instrumentation Engineering and Electronics and Control Engineering)

Time: 3 hours							      Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.	A symmetrical square wave whose average value is zero has a peak to peak amplitude of 20 V and a period of 2 m sec.  This wave form is applied to a low pass circuit whose upper 3dB frequency is ½π MHz.  Calculate and sketch the steady state output waveform.  What is the peak to peak output amplitude?

2. a)  	For the circuit shown in figure below, sketch transfer characteristics, assume ideal diodes.

b)	Draw the basic circuit diagram of a DC restorer circuit and explain its operation.

3. a)	Explain the reverse recovery of a semiconductor diode.  How does the recovery 	time place a limitation on the diode speed?

b)	What is a non-saturation switch and where it is used.

4. a)	Outline various design steps of a collector coupled bistable multivibrator.

b) 	Design a collector coupled bistable multivibrator using the transistors having following parameters: hFE(min)=50, Ic(sat)=5 mA, VCE(sat)=0 V, VBE(cutoff)=-0.5V and VBE(sat)=0.6V. Assume Vcc=10V and VBB=-6V.  Calculate the stable state currents and voltages of the designed circuit.

5. a)	Distinguish between voltage and current time base generators.

b)	In the current-sweep circuit, how linearity can be corrected through adjustment of driving waveform. Illustrate with an example.

Contd….2

Code No. 221003					.2.			Set No.4

6. a)	Draw the block diagram and waveforms for a divider without phase jitter.

b)	Frequency division of 6:1 is obtained with an astable multivibrator negative pulses are applied to both bases of the n-p-n transistors. The OFF time of Q1 is twice that of Q2. Sketch the wave shapes at B1 and B2, showing superimposed pulses.

7.	Explain the operation of bi-directional sampling gate using diodes.  Give the equivalent circuit and derive the expression for gain.  Derive the expressions for minimum control voltages required.

- In a  diode  controlled  free  running  blocking  oscillator,  given  that                VCC  =  10V,  VBB  =  0.5V, n = 2, R = 1.5 K ,  Rf  = 10  ,  Vr  =  9V, L = 3mH  and  C = 100 pF. Calculate the frequency of oscillation and duty cycle.

###