Code No:221401

Set No.

1

II B.Tech., II-Semester Supplementary Examinations, November-2003

ELECTRONIC DEVICES AND CIRCUITS

(Mechatronics)

Time: 3hours								Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

----

1.a)	Derive expressions for rectification efficiency, ripple factor, transformer utilization factor, form factor and peak factor of an half wave rectifier with resistive load.

b)	In a half wave rectifier an a.c. voltage peak value of 24 V is connected is series with a silicon diode and load resistance of 480  .  If the forward resistance of the diode is 20 , find the current flowing through the diode.

2.a)	Draw the VI characteristics of  zener diode and explain its operation.  Show that the zener diode can be used as a voltage regulator.

b)	From the energy band diagram explain the VI characteristics of tunnel diode.  List applications of it and mention its advantages and disadvantages.

3.a)	Draw the circuit using p-n-p BJT in emitter follower configuration and explain its characteristics.  Mention its applications and salient features.

b)	For a given circuit using npn silicon transistor VCC=24V, RC=10 k  RE=480 , β=50.  The biasing resistor R is connected between the base terminal and collector terminal and collector terminal of the BJT. Biasing should be such that VCE=5V.  Determine the value of R.

4.a)	Draw the structure of p channel JEET and explain its static drain and gate characteristics qualitatively.

b)	Explain about the small signal model of JEET.  How do you determine the parameters rd, gm  and  experimentally?

5.a)	What are the conditional to be satisfied for oscillations?  Explain.

b)	Explain why LC oscillators are preferred over RC oscillators at high frequencies.

c)	Derive expressions for oscillations frequency of Wein bridge oscillator.

6.a)	Show that voltage series feedback increases the input impedance and decreases voltage gains of an amplifier.

b)	An amplifier with open loop voltage gain AV =1000 +− 100 is available.  It is necessary to have an amplifier whose voltage gain varies by no more than +−0.1 percent.  Find the reverse transmission factor β of the feed back network and the gain with feed back.

7.a)	Using hybrid Π model prove that

(i) hfe=gm  Vb’e      (ii) hie= rbb’ + rb’e .

If rbb’   << rb’e.  ;   Prove how does b’e vary with Ie.

b)	Derive the expression for 3dB bandwidth of multistage with identical single stage.

(Contd..2)

Code No:221401					-2-			Set No:1

8.	Answer the following :

(a)	MOSFET enhancement mode

(b)	UJT

(c)	Bridge rectifier.

!!!!

Code No:221401

Set No.

2

II B.Tech., II-Semester Supplementary Examinations, November-2003

ELECTRONIC DEVICES AND CIRCUITS

(Mechatronics)

Time: 3hours								Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

----

1.a)	What is the principle of voltage doubling? Explain the same using half wave rectifier as doubler.

b)	In a bridge rectifier, the transformer is connected to 220V, 50Hz mains and turns ratio of step down transformer is 11:1.  Assuming the diodes to be ideal, find (i) the voltage across the load (ii) Ide and (iii) PIV.

2.a)	Explain VI characteristics of PN diode and different current components of it.

b)	The temperature dependence  of the reverse  saturation current of Ge diode is

0.11/ oC, Io=5 A at reverse voltage of 10V , the temperature dependence is 0.07/ oC.  Determine the value of resistance, shunting the diode which has caused this change.

3.a)	Draw the Ebers mole model for a PNP transistor and give the equations for emitter current and collector current.

b)	In a CE Ge transistor amplifier using self bias circuit, RC=2.2 k, β=50, VCC= 9V,the required operating print is IC=2 mA and VCE=3V.  Determine the values of R1,R2 and RE.

4.a)	Compare JFET with BJT.

b)	In an N-channel JFET biased by potential divider method, it is desired to set the operating print at ID=2.5 mA. and VDS=8 V.  If VDD=30V, R1= 1M  and R2=500 k.

Find the value of RS.  The parameters of JFET are IDSS =10mA and VP=-5V.

5.a)	Briefly explain the operation of Darlignton Emitter follower and also derive expressions for its performance measures.

b)	What are the advantages of negative feed back and explain.

6.a)	Explain the operation of voltage series feedback amplifier.

b)	Determine voltage gain, input and output impedance of negative feedback amplifier having A=100, Ri=10k Ro=20k for a feedback factor of β=0.1 and 0.5.

7.a)	Explain Berkhausen criterion.  Derive expressions for frequency of  oscillation and hfe, Re and R relationship for the RC phase shift oscillator.

b)	Design colpitts oscillator to produce since wave at 10MHz using a BJT  having hie=2k, hfe=100.  Choose C1=C2=100 pf.  Show the circuit diagram.  Use VCC=+12V.  Derive the expressions used.

8.	Explain the following :

(a)	Hartley oscillator   	(b)	FET biasing    	(c)	UJT

!!!!!

Code No:221401

Set No.

3

II B.Tech., II-Semester Supplementary Examinations, November-2003

ELECTRONIC DEVICES AND CIRCUITS

(Mechatronics)

Time: 3hours								Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

----

1.a)	Explain the drift and diffusion currents of a semiconductor.

b)	Explain the terms (i)  Static resistance  (ii)  dynamic resistance (iii)  junction resistance and reverse resistance of a diode.

c)	The reverse saturation current Io in germanium diode is 6A.  Calculate the current flowing through the diode when the applied forward bias voltages are 0.2, 0.3 and

0.4 V at room temperature.

2.a)	Explain Avalanch break down and zener break down and explain how zener diode can be used as voltage regulator.

b)	What is the principle of providing thermal stabilization by means of different methods of transistor biasing ?  How does this differ from the compensation techniques using a diode.

3.a)	Draw the small signal hybrid model of  CE amplifier and derive the expression for its AI, AV. Ri, and Ro.

b)	The hybrid parameters for a transistor used in CE configuration are hfe=150, hie=1000, hre =1.2x10-4 , hoe=25x10-6 Ʊ .  The transistor has a load resistance of 10 k  in the collector and source resistance of 5 k.  Compute the values of (i) Ri (ii)Ro (iii) AJ (iv) AV.

4.a)	Derive an expression for output impedance, input impedance voltage gain, current gain of CS amplifier interms of small ‘h’ parameter model.

b)	A CS amplifier has the following specifications R1=3.3 M, R2=1.2M, R0=3.9 k, RS =3.9 k, RL=82k and gm=6000 Ʊ and rd=70k.  Calculate Zi, Zo and AV and AI.

5.a)	With a neat sketch describe the principle of operation of RC coupled amplifier and also derive the necessary relations for voltage gain and current gain at low and high frequencies in terms of medium frequency values.

b)	With a neat sketch explain the operation of cascode amplifier and also derive an expression for its performance measure.

6.a)	What are the different types of feed back connections employed in amplifier and obtain expressions for input impedance and output impedance of current series feed back.

b)	If an amplifier has a gain of 60 ds.  It has an output impedance Z0=12 k, it is required to modify its output impedance to 600 by applying negative feed back.  Calculate the value of the feed back factor.  Also find the percentage change in over all gain, for 10% change in the gain of the internal amplifier.

(Contd..2)

Code No:221401					-2-			Set No:3

7.a)	Give the circuit of a Hartley oscillator and give the conditions for oscillation and derive the frequency of oscillation.

b)	A Hartley oscillator is designed with L1=2 mH, L2=20 mH and a variable capacitance.  Determine the range of capacitance if the frequency of oscillation required is 950 to 2050 KHz.

8.	Answer the following :

(a)	Wein bridge oscillator

(b)	FET biasing

(c)	Tunnel diode.

!!!!!

Code No:221401

Set No.

4

II B.Tech., II-Semester Supplementary Examinations, November-2003

ELECTRONIC DEVICES AND CIRCUITS

(Mechatronics)

Time: 3hours								Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

----

1.a)	Explain qualitatively the V-I characteristics and current components of a p.n. junction diode.

b)	The p and n regions of a silicon diode have resistivities of 10-2  -m and 10-4 -m respectively at 300 o K.  Find the value of contact difference of potential, given

ni =2.5x1019/m3 , =1300 cm2/V-Sec, p=500 cm2/V-Sec .

2.a)	Draw the circuit of a full wave rectifier circuit with L section filter and derive the expression for ripple factor.

b)	Determine the value of inductor needed to produce a 10% ripple factor for a FWR circuit with inductor filter.  The input is 170 sin 314 t, RL=1k.

3.a)	Compare the characteristics of BJT is CE, CB and CC  configurations.  Give typical values of the parameters and mention the application of the different configurations.

b)	An npn Si BJT has =0.98, IC=2A.  It is connected in CE configuration with VCC=12V and RL=4 k.  What is the minimum base current required in order that the transistor enters into its saturation region ?

4.a)	Derive the general expression for ID in the case of JFET interms of VGS and VP.  What is the condition for biasing JFET for zero drift current.

b)	Design a JFET biasing circuit for zero drain current drift, given VP=-3V, gmo=1.8 m A/V,  IDSS=1.75 m A, RD=5k.

5a)	Show that the current shunt feed back decreases input impedance and increases output impedance.  Derive the equations.

b)	Design a colpitts oscillator to produce sine wave at 10 MHz using a BJT having hie=2k and hfe=100.  Choose C1=C2=100 pf.  Show the circuit diagram.  Use VCC==12V.  Derive the expression used in the design.

6.a)	Mention the different methods of biasing a transistor.  Explain any two in detail.

b)	Analyse a transistor amplifier circuit using h parameters.

7.a)	Explain Barkhausen  criterion.  Derive expressions for frequency of oscillation and hfe, Re and R relationship for RC phase shift oscillator.

b)	Prove that the frequency stability of any LC oscillator is directly proportional to the  of the tank circuit employed.

8.	Write short notes on the following ;

(a)	Varactor diode	  (b)   Crystal oscillators    (c)   Cascode amplifier.

!!!!!