Code No: 221801

Set No.

1

II B.Tech. II-Semester Supplementary Examinations November-2003.

THERMODYNAMICS AND KINEMATICS

(Metallurgy and Material Technology)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Briefly explain intensive extensive properties.

b)	Define thermodynamic equilibrium.

c)	Consider a gas enclosed in a piston-cylinder occupies a volume of 0.2 m3 at a pressure of 500 kPa.  The gas is taken to a final state where P2=100 kPa by the following two different processes. Calculate work done in each case.

(i) The volume of the gas is inversely proportional to the pressure.

(ii) The process follows the path Pvr=C.   r=1.4.

2.a)	What is the difference between an open and a flow system?

b)	Obtain an expression for the flow work.

c)	In a study flow system 50 kJ of work is done per kg of the fluid.  The values of specific volume, pressure and velocity at the inlet and exit sections are 0.4m3/kg, 6 bar, 15 m/sec and 0.6 m3/kg 1 bar, 250 m/sec respectively.  The inlet is 3m above the exit.  The heat loss from the system is 8 kJ/kg.  Calculate change in internal energy.

3.a)	Define Entropy.

b)	Briefly explain Clausius inequality.

c)	An insulated chamber of volume 2V1 is divided by a thin rigid partition into two halves.  One chamber contains an ideal gas at a temperature T1.  The other chamber is evacuated.  The partition is suddenly removed and the equilibrium is reestablished.  Determine (i) the final temperature  (ii) change in entropy.

4.a)	Define second law of thermodynamics.

b)	A mass m of liquid at temperature T1 is mixed with an equal amount of the same liquid at the temperature T2.  If the specific heat of the liquid is C show that the total change in entropy is

ΔS=2mCln(T1+T2)2T1T2

c)	One kg of super heated steam at 0.2 MPa and 200oC contained in a piston-cylinder assembly is kept at ambient conditions of 300K till the steam is condensed to saturated liquid at constant pressure.  Calculate the change in the entropy of the universe associated with this process.

Contd…..2

Code No:221801					-2-			Set No:1

5.a)	Briefly explain the need for statistical thermodynamics.

b)	Define micro and macro state.

c)	A system consisting of a large number of distinguishable particles, has available to it for non degenerate energy levels with energies 1, 2, 3, 4 units.  Determine the fractional distribution of particles into the energy level if the average energy per particle is 1.5 units.

6.a)	Show ∂u∂TP=Cp−Pνβ and ∂u∂νP=Cpνβ−P.

b)	Derive a relation Cp-Cv=Tνβ2K.

c)	Estimate the rise in temperature if liquid water at 25oC is compressed isentropically from 0.1 MPa.  For liquid water Cp=4.2 kJ/kgoK and        =2.07x10-4K-1.

7.a)	Briefly explain fugacity and activity.

b)	A mixture of A and B with ratio of 1:1 is fed in to a reactor which is maintained at temperature T and 0.1 MPa pressure.  The following two independent reactions occur in the mixture.

A+BC+D	K1=0.4

A+2BC+E	K2=0.5067

Determine the equilibrium composition of the mixture.

8.a)	Briefly explain and derive an expression for Clapeyron equation.

b)	Carbon tetra chloride boils at 76.75oC at 101 kPa.  The latent heat of vaporization of carbon tetra chloride is 194.8 kJ/kg.  Determine the boiling point at 200 kPa.

- - -

Code No:221801

Set No.

2

II B.Tech. II-Semester Supplementary Examinations November-2003.

THERMODYNAMICS AND KINEMATICS

(Metallurgy and Material Technology)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	What is a property?  What are the methods of calssifying properties?  Give examples.

b)	Show how the ideal gas temperature scale is developed.  State its connection with the thermodynamic (Kelvin) scale.

c)	A closed system contains 1 kg of steam at 5 bar and 0.3 m3.  Calculate changes in h, s, , T and x when it executes a constant volume process till the pressure reduces to 2 bar.

2.a)	State the conservation of energy and explain how it was modified because of first law.

b)	Derive first law for a cycle.

c)	A closed system containing 10 kg of liquid water at 1 bar and 30oC absorbs 25 MJ of heat at constant pressure.  Calculate internal energy 2 (final).

3.a)	State and explain Kirchhoff’s equation.

b)	Define the terms heat of formation and heat of reaction.  How they are related?

c)	Calculate the calorific value of Ethane (C2 H6).

4.a)	Derive the Clausius inequality.

b)	Derive second law equation for a flow system.

c)	500 grams of water at 300oK is stirred in an insulated vessel.  The energy required for the stirring is obtained from a mass of 50 kg falling from 20 meter height.  Compute change in entropy of water.

5.a)	What is a Born diagram?  Explain how it is constructed.

b)	Define Helmholtz and Gibbs function.

c)	Using cyclic relation prove that ∂p∂Tν=βkT.

Contd…..2

Code No:221801					-2-			Set No:2

6.a)	Define partition function.

b)	Explain Macro and Micro states.

c)	A system consists of three particles each of which can exist among the four levels of energy with energies , 2, 3 and 4.  The respective degeneracies are 1, 1, 2 and 2.  The total energy of the system is 9.

(i) List the macro states and the number of micro states associated with the macro states for all the three statistics.

(ii) Sketch the micro states for the most probable macro state for all the three statistics.

7.a)	Prove Gibb’s phase rule.

b)	Derive expression to estimate the change in the enthalpy of Clausius gas when it is changed from the ideal state P1T1 to the final state P2T2.

8.a)	What is zero order reactions?

b)	Discuss briefly collision theory.

c)	How the order of reaction is determined?

- - -

Code No:221801

Set No.

3

II B.Tech. II-Semester Supplementary Examinations November-2003.

THERMODYNAMICS AND KINEMATICS

(Metallurgy and Material Technology)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	How thermodynamic systems are classified?

b)	What is meant by quasi-static process?

c)	A reversible heat engine operates between two reservoirs at temperatures of 600oC and 40oC.  The engine drives a reversible refrigerator which operates between reservoirs at temperatures of 40oC and -20oC.  The heat transfer to the engine is 2 MJ and work out put of combined engine and refrigerator plant is 360 kJ.  Find the heat transfer to the refrigerant and the net heat transfer to the reservoir at 40oC.  Also find there values if the efficiency of the heat engine and COP of the refrigerator are each 40% of their maximum values.

2.a)	What is an isothermal process?  Derive an expression for workdone during an isothermal process.

b)	What are the assumptions made for a steady flow process?  Write its general equation.

c)	In a gas turbine the gases flow at the rate of 5 kg/sec.  The gases enter the turbine at a pressure 7 bar with a velocity 120 m/sec and leaves at a pressure 2 bar with velocity 250 m/sec.  The turbine is insulated.  If the enthalpy of the gas at inlet is 900 kJ/kg and at outlet 600 kJ/kg.  Determine the capacity of the turbine.

3.a)	Discuss the following processes are reversible or not.  If they are not reversible specify the source of reversibility.

(i) A saturated liquid water at 100oC is evaporated to saturated steam at the same temperature in a boiler.

(ii) Liquid water at 30oC is continuously stirred by a paddle wheel till water attains a temperature of 40oC.

b)	One kg of saturated steam at 1.00 MPa and 300oC contained in a cylinder-piston assembly undergoes a reversible adiabatic expansion to a pressure of 100 kPa.  Calculate the workdone by the steam.

4.a)	Derive an expression for the change in the entropy for the isothermal process in terms of volumes.

b)	An ideal gas of mass 0.25 kg has a pressure of 3 bar, temperature of 80oC and a volume of 0.07 m3.  The gas undergoes an irreversible adiabatic process to a final pressure of 3 bar and volume of 0.10 m3, during which the workdone on the gas is 25 kJ.  Evaluate Cp and Cv of the gas and increase in entropy.

Contd…..2

Code No:221801					-2-			Set No:3

5.	Briefly discuss the following:

(a) Quantum statistics

(b) Boes Einstein statistics

(c) Ferbni-Dirac statistics.

6.	Explain the following:

(a) Grand Canomical Ensemble

(b) Micro Canomical Ensemble

(c) Isothermal-Isobaric Ensemble.

7.a)	Briefly explain phase equilibrium. (Gibbs phase rule).

b)	One kg of liquid water at -10oC and 136 MPa is converted into ice at the same temperature and pressure.  Calculate change in entropy during the process.  Latent heat of fusion at 0oC is 334.92 kJ/kg .  Cp of the liquid is 4.186 kJ/kgoK and that of ice is  2.093 kJ/kg oK.

8.a)	Briefly Explain first order and second order reactions.

b)	What is meant by consecutive and simultaneous reactions?  Give examples.

c)	Briefly discuss the role of catalysis in chemical reactions.

- - -

Code No:221801

Set No.

4

II B.Tech. II-Semester Supplementary Examinations November-2003.

THERMODYNAMICS AND KINEMATICS

(Metallurgy and Material Technology)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Define path and point functions.

b)	What is an isothermal process?  Derive expression for the isothermal process in terms of ratio of pressures.

c)	0.1 m3 of air at a pressure of 1.5 bar is expanded isothermally to 0.5 m3.  Calculate the final pressure of the gas and heat supplied during the process.

2.a)	What is meant by heterogeneous and homogeneous system?

b)	Define Jeroth law of thermodynamics.

c)	State any two applications of steady flow energy equations and derive expressions for them.

3.a)	What are the assumptions made in the kinetic theory of gases?

b)	Derive an expression for kinetic energy per kg molecule of a gas.

c)	Find kinetic energy of the molecule of 1 kg of helium at 0oC if Ru=8.314 kJ/kgoK and molecular mass of helium is 4.

4.a)	Briefly explain Clausius inequality.

b)	Explain Entropy.

c)	A lump of steel weighing 30 kg at a temperature of 427oC is droped in 150 kg of oil at 27oC.  The special heat of steel and oil are 0.5 kJ/kgoK and 2.5 kJ/kgoK respectively.  Estimate the change in entropy of the steel, the oil and the system.

5.a)	Briefly explain Micro and Macro states.

b)	What is meant by partition function?  Explain.

c)	For certain kind of particle the allowed energy levels have energies 1, 2, 3, 4 and 5 units.  These energy levels are non degenerate.  Consider a system of three such particles.  How many macro states are formed if the total energy of the system is 12 units?  Also detail the corresponding micro states.

Contd…..2

Code No:221801					-2-			Set No:4

6.a)	Entropy as a function of T and V derive the following:

Tds=cv dT=TβKdν.

b)	If a functional relation f(x, y, z)=0 derive the reciprocal relation ∂x∂yZ=1∂y∂xZ.

c)	The latent heat of vaporization of water at 100oC is 2257/kJ/kg.  Estimate the same at 120oC and compare value given in the steam tables.  Cpg=1.55 kJ/kgoK Cpf=4.23 kJ/kgoK .

7.a)	State and prove Gibbs phase rule.

b)	Briefly explain absolute reaction rates.

c)	What is the role of catalysis in reaction rates?

8.a)	Define Molecularity and order of a reaction.

b)	Explain Collision theory.

c)	What is the effect of addition of inert gas on the chemical reactions?

- - -