Code No. 222001

Set No.

1

II Year II-Semester Supplementary Examinations November 2003

THERMAL ENGINEERING AND HEAT TRANSFER

(Common to Production Engineering and Mechatronics)

Time: 3 hours							Max. Marks: 80

Answer any Five questions

All question carry equal marks

***

1.	Explain with neat sketches the working of a Two Stroke Petrol engine.

2.a)	Briefly explain the historical developments of internal combustion 	engines.

b)	Compare Otto, Diesel and Dual cycles for the

i) Same compression ratio and heat input.

ii) Same maximum pressure and heat input.

3.a)	Explain the principle of carburetion.

b)	With a neat sketch explain the working principle of a simple carburettor.

4.a)	What is the purpose of supercharging an engine?

b)	Discuss various methods of super charging.

5.	In a gas turbine plant air at 150C is compressed from 1.01 bar through a pressure ratio of 6:1, after which it is heated to a temperature of 7500C. It is then expanded in a turbine to 1.01 bar. Assume the following.

Isentropic efficiency of the compressor = 0.8.

Isentropic efficiency of the turbine = 0.85.

Cp for air and products of combustion = 1.005 kJ/kg K.

Ratio of specific heats = 1.4.

Calculate the cycle efficiency and work ratio.

6.a)	Write a note on thermal conductivity variation with temperature for solids and liquids.

b)	Compute the heat loss from the surface of a furnace wall 25cm thick. The inner and outer surface temperatures are 3000C and 500C respectively. The conductivity of the wall material is given by K=0.002T -10-6 T-2 W/mK where T is in degree celsius. Determine the average value of conductivity in this range of temperatures.

7.a)	From dimensional analysis, develop a relation amongst the dimensionless numbers for free convection in a standard form.

b)	A vertical plate 25cm high is maintained at a temperature of 700C in a still atmosphere of air at 250C. Calculate the boundary layer thickness at the trailing edge of the plate.

(Contd…2)

Code No:222001				:: 2 ::				Set No.1

8.	Write short note on the following.

a)	Distinguish between brake mean effective pressure and indicated mean effective pressure.

b)	Explain the concept of momentum and thermal boundary layers.

-*-*-*-

Code No. NR-222001

Set No.

2

II Year II-Semester Supplementary Examinations November 2003

THERMAL ENGINEERING AND HEAT TRANSFER

(Common to Production Engineering and Mechatronics)

Time: 3 hours							Max. Marks: 80

Answer any Five questions

All question carry equal marks

***

1.a)	What are the fundamental and physical differences between the Otto and Diesel engines?

b)	What are the requirements of I.C. engine fuels.

2.	What are the objectives of supercharging in I.C. engines? How does supercharging effects engine performance?

3.	A four-stroke engine has a cylinder diameter of 25 cm and stroke 45 cm. The effective diameter of the brake is 1.6 m. The observations made in a test of the engine were as follows.

Duration of test = 40 min.

Total number of revolutions = 8080.

Total number of explosions = 3230.

Net load on the brake = 90 kg.

Mean effective pressure = 5.8 bar.

Volume of gas used = 7.5 m3.

Pressure of gas indicated in meter = 136 mm of water gauge

Atmosphere temperature = 170C.

Calorific value of gas = 19 mJ/m3 at NTP.

Rise in temperature of jacket cooling water = 450C

Cooling water supplied = 180 kg.

Prepare the heat balance sheet.

4.	The following data apply to a gas turbine set employing a heat exchanger.

I sentropic efficiency of compressor = 75%

I sentropic efficiency of turbine = 85%

Mechanical efficiency = 95%

Pressure ratio 6:1

Maximum cycle temperature = 1100 K.

Combustion efficiency = 98%

Calorific value of the fuel = 40,000 kJ/kg.

Mass flow rate of air = 20kg/sec.

Ambient condition = 1 bar and 288 K.

Calculate (i) Net output and (ii) Overall cycle efficiency.

(Contd…2)

Code No: 222001				:: 2 ::				Set No.2

5.	A brick wall 25 cm thick is forced with concrete of 5 cm thick. The thermal conductivity of brick is 0.69 W/mK while that of concrete is 0.93 W/mK. If the temperature of the exposed brick face is 300C and that of the concrete is 50C. Find the heat loss per hour through wall of 10 m×5 m. Also determine the interface temperature.

6.a)	Derive the continuity equation for the two dimensional velocity boundary layer over a flat plate.

b)	A flat plate is kept in air stream at a temperature of 200C. The velocity of the air is 3 m/s. the plate measures 50 cm×20 cm and is maintained at a uniform temperatures of 1000C. Calculate the heat loss from the plate when the air flows parallel to 50 cm.

7.	Find the heat lost by radiation per metre of length of 80 mm diameter pipe at 3500C if (i) located in a large room with red brick walls at a temperature of 300C (ii) Enclosed in a 160 mm dia red brick at a temperature of 300C. Take E pipe =0.8 and E brick = 0.9.

8.a)	What are the various components to be lubricated in an engine and explain how it is accomplished.

b)	Explain the concept of thermal resistance.

-*-*-*-

Code No. NR-222001

Set No.

3

II Year II-Semester Supplementary Examinations November 2003

THERMAL ENGINEERING AND HEAT TRANSFER

(Common to Production Engineering and Mechatronics)

Time: 3 hours							Max. Marks: 80

Answer any Five questions

All question carry equal marks

***

1.a)	Discuss about the qualities of SI engine fuels.

b)	Explain the working of Otto Cycle with the aid of actual cycle.

2.a)	Explain the working of a simple carburettor.

b)	What are the functional requirements of an injection system.

3.	The following details were noted in a test on a four cylinder, four stroke engine: diameter=100 mm. stroke=120 mm, speed of the engine=1600 rpm, fuel consumption =0.2 kg/min, calorific value of the fuel is 44000 kJ/kg, difference in tension on either side of the brake pulley = 40 kg, brake circumference is 300 cm. If the mechanical efficiency is 80%, calculate (i) brake thermal efficiency           (ii) indicated thermal efficiency (iii) indicated mean effective pressure               (iv) brake specific fuel consumption.

4.a)	How do you improve the performance of a Gas Turbine? Explain them with suitable sketches.

b)	List out the advantages and disadvantages of a Gas Turbine over IC engine.

5.a)	Explain the mechanism of three modes of heat transfer.

b)	A spherical mass of 1 m diameter receives heat from a source at 1600C by radiation and rejects heat to the surroundings at 300C. The convection co-efficient being 45 W/m2K. Determine the steady state temperature of the solid.

6.a)	What is meant by critical radius of insulation?

b)	Define thermal conductivity, thermal resistance, and thermal conductance.

c)	What is the approximate range of thermal conductivity for solids, liquids and gases?

7.a)	Derive Wein’s  displacement law from Planck’s law.

b)	Define the following.

(i) Black body  (ii)  Grey body (iii) Coloured body (iv) Emissivity.

8.a)	What is the purpose of supercharging an engine?

b)	Explain the following:

(i) Boundary layer (ii) Free convection (iii) Forced convection.

-*-*-*-

Code No. NR-222001

Set No.

4

II Year II-Semester Supplementary Examinations November 2003

THERMAL ENGINEERING AND HEAT TRANSFER

(Common to Production Engineering and Mechatronics)

Time: 3 hours							Max. Marks: 80

Answer any Five questions

All question carry equal marks

***

1.a)	How are internal combustion engines classified?

b)	What are the fundamental and physical differences between the Otto and Diesel engines?

2.a)	Discuss various methods of supercharging in I.C. engines.

b)	Discuss various types of fuel injection systems.

3.a)	Enumerate the advantages of gas turbines over IC engines.

b)	An open cycle gas turbine plant works between the pressure range of 1.01 bar and 6 bar and temperature range of 3000K and 1023 K. The calorific value of the fuel is 41.800 kJ/kg. Find :

(i)   Air-fuel ratio	(ii) Thermal efficiency of the plant.

(iii) Power developed if flow rate of air is 10 kg/sec.

Isentropic efficiency of compression and expansion is 80% and there is a pressure loss of 0.15 bar in the combustion chamber.

4.a)	Give a comparison between the three modes of heat transfer.

b)	A steel pipe has an outside diameter of 5cm. It is covered with a 0.6 cm layer of one insulation (K = 0.17 W/m0c) followed by a 2.5 cm layer of a second insulation (K = 0.048 W/m0C). The pipe surface temperature is 3150C and the outer insulation surface temperature is 400C. Calculate the temperature at the junction of two insulators.

5.a)	Discuss the following:

(i) thermal conductivity (ii) Thermal resistance (iii) Critical insulation thickness.

b)	Derive an expression for the conduction heat flux through a spherical wall.

6.a)	With the help of dimensional analysis obtain a relation between non-dimensional  numbers for free convection.

b)	Find the electrical power required to maintain vertical plate heater at 1300 C in ambient air at 200C. The plate is 15 cm high and 10 cm wide.

7.a)	What is a black body? Explain the concept of black body?

b)	Define the following:

(i) Emissive power (ii) Absorptivity. (iii) Reflectivity (iv) Transmissivity.

(Contd…2)

Code No: 222001				:: 2 ::				Set No.4

8.	Discuss the following:

a) Simple carburettor.

b) Gas turbine performance and its improvement.

-*-*-*-