Code No. NR-222101

Set No.

1

II Year II-Semester Supplementary Examinations November 2003

FLUID MECHANICS AND MACHINERY

(Aeronautical Engineering)

Time: 3 hours							Max. Marks: 80

Answer any Five questions

All question carry equal marks

---

1. a)	Define viscosity.  Derive the Newton’s law of viscosity.

b)	A piston of 80 cm long and 20 cm dia moves in a cylinder concentrically of 21 cm dia and longer length.  The gap is filled with oil of viscosity 14 poise.  What is the torque required to maintain a speed of rotation of 60 rpm.

2. a)	Prove that centre of pressure is always below the centre of gravity, when a plane surface is immersed in a fluid.

b)	A plane surface of size 10 m  10 m is placed with 30o inclination is water with its top edge being in water surface.  Specific weight of water is 9810 N/m2.  Determine the total pressure and centre of pressure of total pressure on the surface.

3. a)	Derive the continuity equation from fundamentals.

b)	A varying cross section pipe line carrying water in laid out such that the difference of elevation between two sections A and B is 5 m.  The pressures at A and B are 40 kN/m2 and 60 kN/m2 respectively.  The diameters of sections A and B are 25 cm and 45 cm respectively.  The flow rate in pipe line is 0.55 m3/sec.  Determine the direction of flow.

4. a)	Derive the expressions for Reynold’s number and Mach number.

b)	Explain the applications of Momentum equation in detail.

c)	Define and explain free and forced vortex phenomenon.

5. a)	How does the pressure vary within the boundary layer.

b)	What is laminar sub-layer?

c)	How do you estimate the thickness of the boundary layer?

6. a)	What is the principle on which venturimeter works?  Derive the equation for discharge of venturimeter.

b)	A pitot tube is used to measure the velocity of flow of water in a river.  The stagnation pressure is 3.5 kg/cm2 and the static pressure is 1.2 kg/cm2.  If coefficient of pitot lube is 0.76, what is the velocity in river?

7. a)	Explain how a differential manometer works.  Write the manometric equation.

b)	What is Rotameter?  What is its principle?

8.	Write short notes on the following:

a)	Compression and expansion waves.	b)	Super sonic flow

c)	Lift and drag coefficient		d)	Wedge probes.

###

Code No. NR-222101

Set No.

2

II Year II-Semester Supplementary Examinations November 2003

FLUID MECHANICS AND MACHINERY

(Aeronautical Engineering)

Time: 3 hours							Max. Marks: 80

Answer any Five questions

All question carry equal marks

---

1. a)	Find the equations for total pressure and centre of pressure when an inclined plane surface is immersed in a fluid with same inclination.

b)	A square surface 3 m  3 m lies in a vertical plane.  Determine the position of centre of pressure and the total pressure on the square on one side, when its upper edge is horizontal and 16 m below the water surface.

2. a)	Show that potential flow function and stream function satisfy Laplace equation.

b)	The stream function and velocity potential for a flow are  = 2 xy and  = x2 – y2. Show that the conditions of continuity and irrotational flow are satisfied.

3. a)	Derive the Bernoulli’s equation form fundamentals.

b)	The following details refer to a pipe line laid to carry municipal water.  Find flow direction

Diameters at sections x and y  = 10 cm and 25 cm respectively.

Elevation difference between sections 	= 250 cm

Pressures at sections x and y    =  20 kN/m2 and 42 kN/m2 respectively.

The discharge in pipe line	=   2 m3 / sec.

4. a)	What are the applications of momentum equations.

b)	Explain free vortex and forced vortex motion in detail.

5. a)	What are the causes which result in separation of boundary layer.

b)	What is laminar sub layer?

c)	How do you define the thickness of boundary layer?

6. a)	Derive the equation for discharge through venturimeter.

b)	A vertical Venturimeter has a throat diameter of 5 cm and inlet diameter of 12 cm.  The differential manometer shows a mercury difference of 23 cm.  The pipe carrying water is horizontal.  Calculate the discharge.  Take Cd = 0.92 and specific gravity of mercury as 13.6.

7. a)	What is pitot tube?  Where is the stagnation point in the Pitot tube?

b)	Explain the working of a differential manometer.  Write the manometric equation.

8.	Write short notes on the following:

a)	Compression and expansion waves.	b)	Shock waves

c)	Wedge probes				d)	Lift and Drag coefficients.

###

Code No. NR-222101

Set No.

3

II Year II-Semester Supplementary Examinations November 2003

FLUID MECHANICS AND MACHINERY

(Aeronautical Engineering)

Time: 3 hours							Max. Marks: 80

Answer any Five questions

All question carry equal marks

---

1. a)	Differentiate between the following:

(i) Rotational and irrotational flow	(ii) Uniform and non-uniform flow.

(iii) Steady and unsteady flow.

b)	A piston 800 mm in diameter and 200 mm long moves centrally to and fro in a cylinder of 810 mm diameter.  If the annular space is filled with oil of viscosity 14 poise; calculate the speed of descent of piston in vertical position.  The weight of load on piston including self weight is 12 N.

2. a)	Define stream function and velocity potential.  How is flow net developed?

b)	A circular plane surface of diameter 2.5 m is immersed with 30o inclination in water such that the top is in the surface.  The specific weight of water is 9810 N/m3.  Find the total force and the centre of pressure on the plane surface.

3. a)	Derive the Bernoullis Equation from fundamentals stating the assumptions made.

b)	The velocity components in x and y directions are u = 2xy3/3 – x2y and                 v = xy2-2yx3/3.  Check whether the given velocity distribution is a possible field of flow.

4. a)	What are the applications of momentum equation.

b)	Explain free vortex and forced vortex motion in detail, stating examples.

5. a)	Define the boundary layer thickness. How do you estimate the same?

b)	Explain how flow separation takes places.  What are the factors causing flow separation?

c)	What is laminar sub layer?

6. a)	Explain the working of pitot static tube and derive the equation.

b)	A 20 cm water pipe has in it a venturimeter of throat diameter 12.5 cm such that the elevation difference between inlet and throat is 3 m.  The differential manometer shows a mercury deflection of 28 cm.  Find the discharge in the pipe if Cd = 0.92 and if the throat is at higher level.  Specific gravity of mercury is 13.6.

7. a)	Explain how hot wire anemometer works.

b)	What are different types of manometers?

c)	What are flow visualization techniques?

8.	Write short notes on the following:

a)  Shock Waves			b)  Super Sonic flow

c) Rotameter				d)  Life and drag coefficients.

###

Code No. NR-222101

Set No.

4

II Year II-Semester Supplementary Examinations November 2003

FLUID MECHANICS AND MACHINERY

(Aeronautical Engineering)

Time: 3 hours							Max. Marks: 80

Answer any Five questions

All question carry equal marks

---

1. a)	How are fluids classified as Newtonian and Non Newtonian fluids?

b)	Find the equation for total pressure acting on a vertical plane surface.

c)	A plane surface of size 4.5  4.5 m is placed vertically in a water tank with its top side horizontal and at a depth of 8 m from top.  Find the total force acting on any one side of plane surface and its point of action.  Take specific weight as         9810 N/m3.

2. a)	Define potential function and stream function.  What is the relation between them?

b)	The stream function of flow of fluid is given by  = x3 – 3xy2.  Determine whether the flow is rotational or irrotational.

3. a)	Derive the momentum equation from fundamentals.  What is momentum correction factor.

b)	Explain the free vortex and forced vortex flow phenomena in detail.

4. a)	Derive continuity equation from mass conservation principle.

b)	The following are the data given of a change in diameter effected in laying a water supply line.  The change in diameter is gradual from 20 cm at A to 57 cm at B.  Pressures at A and B are 80 kN/m2 and 60 kN/m2 respectively with the end B being 3 m higher than A.  If the flow in pipe line is 20 m3/sec find the flow direction.

5. a)	Explain how flow separation takes place with the help of Boundary layer sketch.

b)	Differentiate between the characteristics of laminar and turbulent boundary layers.

c)	What is a laminar sub layer?

6. a)	What are the types of pressure gauges.

b)	Explain the working of pitot tube.  Where will be the stagnation pressure?

c)	A pitot tube inserted in a pipe line records a pressure difference of 2.88 kg/cm2.  If Cv = 0.86 and the fluid is water find the velocity in the pipe line.

7. a)	Explain how a differential manometer works.  Write the manometric equation.

b)	What is hot wire anemometer?

c)	Explain how Rotameter works?

8.	Write short notes on the following:

a) 	Shock waves			b)	Super Sonic flow

c)	Drag and lift coefficient	d)	Wedge probes

###