Code No: 220252

## OR

II B.Tech. II Semester Supplementary Examination November-2003

THERMODYNAMICS

(Electrical and Electronics Engineering)

Time: 3 hours						       	Max. Marks: 70

Answer any Five questions

All question carry equal marks

- - -

1.a)	Define and discuss the following terms:

(i) Control volume and control surface.   (ii) Open system and isolated system.

b)	What is Zeroth law of thermodynamics?

c)	A mass of 1.5 kg of air is compressed in a quasistatic process from 0.1 MPa to  0.7 MPa for which pv = constant.  The initial density of air is 1.16 kg/m3.  Find the work done by the piston to compress the air.

2.a)	Explain the following terms:

(i) Internal energy   (ii) Entropy.

b)	One kg of ice at -5 oC is exposed to the atmosphere which is at 25 oC.  The ice melts and comes into thermal equilibrium with the atmosphere.  Determine the entropy increase of the universe.  Take cp of ice of 2.093 kJ/kg oC and the latent heat of fusion of ice is 333.3 kJ/kg.

3.a)	Draw the p-v diagram of water and explain the various saturated states.

b)	What is Mollier diagram?  Explain.

c)	Find the enthalpy and entropy of steam when the pressure is 2 MPa and the specific volume is 0.09 m3/kg.  Take the following values when p = 2 MPa.

vf = 0.001177 m3/kg		vg = 0.09963 m3/kg

hf = 908.79 kJ/kg		hfg = 1890.7 kJ/kg

sf = 2.4474 kJ/kgK		sfg = 3.8935 kJ/kg K

4.a)	State and explain Carnot theorem.

b)	A heat pump working on the Carnot cycle takes in heat from a reservoir at 5 oC and delivers heat to a reservoir at 60 oC.  The heat pump is driven by a reversible heat engine which takes in heat from a reservoir at 840 oC and rejects heat to a reservoir at 60 oC.  The reversible heat engine also drives a machine that absorbs 30 kW.  If the heat pump extracts 17 kJ/s from the 5 oC reservoir, determine        (i) the rate of heat supply from the 840 oC source and (ii) the rate of heat rejection to the 60 oC tank.

5.a)	Distinguish between SI engine and CI engine.

b)	Sketch a typical valve timing diagram of a four-stroke cycle diesel engine.

c)	Describe with the help of a neat sketch, two stroke diesel engine.

Contd…….2

Code No:220252					-2-				OR

6.a)	What is the function of steam nozzle?

b)	Distinguish between gas turbines and steam turbines.

c)	A gas turbine plant operates on the Brayton cycle between Tmin = 300 K and              Tmax = 1073 K.  Find the maximum work done per kg of air and the corresponding cycle efficiency.

7.a)	With the help of neat sketch, explain the principle and working of rotary compressor.

b)	What are the advantages of multi-stage compression?

8.a)	Classify the various applications of refrigertion in broad groups.

b)	Define the term “Coefficient of Performance” of a refrigerator.

c)	Describe the operation of air refrigerator working on the Bell-Colemann cycle.

- - -