Code No. 220253

OR

II B.Tech. II-Semester Supplementary Examinations, November-2003

ELECTRONIC DEVICES

(Electrical and Electronics Engineering)

Time: 3 hours							       Max. Marks:70

Answer any Five questions

All questions carry equal marks

- - -

1.a)	Derive the expression for current density in a conductor.

b)	Explain the energy band diagrams of N type and P type semiconductors.

c)	Differentiate diffusion and drift currents in a semiconductor.

2.a)	Explain the current component of the diode.

b)	Derive the semiconductor diode current equation? And discuss its temperature dependence.

3.a)	With the help of  the energy band diagram discuss the operation of a tunnel diode and also list out its applications.

b)	Write the applications of LED?.

4.a)	Explain the constructional details and working of UJT and its V-I characteristics.

b)	What is early effect? And what are the effects produced by Base-width modulation.

5.a)	What are the advantages of MOSFET over JFET .

b)	Explain the drain characteristics of JFET.

c)	Explain how JFET can be used as a voltage variable resistance.

6.a)	Derive an expression for the deflection sensitivity of a cathode ray beam acted upon by a magnetic field.

b)	In a cathode ray tube, the length of the deflecting plates in the direction of the beam is 1.5cm; the spacing of the plates 0.4cm and the distance of the fluorescent screen from the center of the plates is 20cm. Calculate the deflection sensitivity, if the final anode voltage is 900V.

Contd…2.

Code No. 220253					-2-				OR

7.a)	Distinguish between zener and avalanche phenomena of breakdown.

b)	The saturation currents of the two diodes in the figure are 1µA and 2µA. The breakdown voltages of the diodes are the same and are equal to 100V. Calculate the current and voltage for each diode if V=110V.

- Write the advantages of LCD over LED.

8.a)	Define Ripple factor, regulation and rectification effeciancy of a rectifier.

b)	Compare Halfwave, center tapped Fullwave and Bridge rectifiers.

c)	Derive the expressions for ripplefactor  and recification efficiency of a fullwave rectifier.

-x-