Code No. 220552

OR

II B.Tech. II Semester Supplementary Examinations November, 2003

ELECTRONIC CIRCUITS AND LINEAR IC’s

(Common to Computer Science and Information Technology and Computer Science and Engineering)

Time: 3 hours							    Max. Marks:70

Answer any Five questions

All question carry equal marks

- - -

1.a)	Compare half wave , full wave  and bridge rectifiers.

b)	What are the advantages of bridge rectifier?

c)	An ac supply of 230 V is applied to a half wave rectifier circuit through a transformer of turns ratio 5:1 Assume the diode is an ideal diode.  The load resistance is 300. Find

(i)   DC output voltage     (ii)   PIV	(iii)	Maximum and

(iv) Average values of power delivered to the load.

2.a)	What are the three factor that contribute to thermal instability.

b)	Calculate the values of R1 and RC in the voltage divides bias circuit.  So that Q point is at VCE=6V and IC=2ma.  Assume the transistor parameter are =0.985, ICBO=4a and VBE=0.2V

3.a)	Explain the main difference between amplifier and oscillator.

b)	Draw the Wien bridge oscillator circuit and briefly explain  its operation.

c)	In the Wien bridge oscillator circuit if the RC network consists of resistors

200 k  and the capacitors of 300 p.f find its frequency of oscillation.

4.a)	Define input offset current, input offset voltage CMRR and slew rate of an operational amplifiers and explain their significance.

b)	Draw the circuit of voltage to current converter using OP amp for a non floating load and obtain the condition to be satisfied and hence the expression for the load current.

5.a)	Explain how OP AMP can be used as a comparator

b)	What are the advantages of Active filter?  Explain the operation of an OPAMP

integrator.

6.a)	Distinguish between semiconductor diode and photodiode. Explain the characteristics of a constant base amplifier

b)	What ate the advantages of negative feed back? Explain the basic concepts of feed back amplifier.

7.a)	Distinguish between oscillator and amplifier.

b)	Distinguish between a timer and counter. How a 555 timer can be used as astable multivibrator?

8.	Write short notes on :

(a) PLL	(b) Voltage regulators		(c) Transducer Amplifier.

!!!!!