Code No: 310102.

Set No:

1

III B.Tech. I-Semester Regular Examination November 2003

STRUCTURAL ANALYSIS-I

( Civil Engineering  )

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1. 	Draw the bending moment diagram and locate the point of inflections for the 	propped cantilever beam shown in figure.

2.	Determine the moments and reactions at the supports of the fixed-fixed beam 	which is loaded by a concentrated load of 10kN at a point 3 m from the left 	support. The span of the beam is 8 m. Use method of consistent deformations. EI 	is constant

3.	The pin jointed truss shown in Figure is loaded with two point loads of 20kN and 	10kN at the upper joints.  Evaluate the forces in the members using the method of 	tension coefficients.

4. 	Analyse the continuous beam shown in Figure by Clapeyron's theorem of three 	moments, if support C sinks by 5mm. Also sketch the BMD and SFD. Take

E = 15 kN/mm2 and I = 4 x 109mm4.

5.	State and explain Castigliano’s first theorem taking any example.

(Contd…2)

Code No: 310102.				.. 2..				Set No: 1

6.	A train of wheel loads as shown crosses a girder of 25m span with 120kN load leading.

Wheel load (kN) : 180	   160	      160	 120

Spacing (m)       :        2             2           3

Determine the maximum B.M at a section 8m from the left end and the absolute      maximum B.M on the girder.

7.	A beam CABD is simply supported at A and B and has overhangs on both the 	supports. Overhang CA=2m, span AB=10m and overhang BD=2.5m. Draw the 	influence lines for B.M at A,B and at the centre of AB.

8. 	Draw the shear force and bending moment diagram for the given cantilever beam. Use graphical procedure.

~ ~ ~ ~

Code No: 310102.

Set No:

2

III B.Tech. I-Semester Regular Examination November 2003

STRUCTURAL ANALYSIS-I

( Civil Engineering  )

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1. 	Draw the bending moment diagram and locate the point of inflections for the 	propped cantilever beam shown in figure.

2.	A fixed- fixed beam of span 8 m is subjected to a couple of 4 kN-m at a distance 	of 2m from the fixed support and a concentrated force of 6kN at the mid-point 	along the span. Using method of differential equations, find the reactions at the 	supports.

3.	The Pratt truss is loaded as shown in Figure. Find the forces in members AF, AC, 	CF, CD, DF and FG using the method of tension coefficient.

4. 	Analyse the continuous beam shown in Figure by Clapeyron's theorem of three 	moments if support C settles down by 5mm. Take E = 15 kN/mm2. Moment of 	inertia is constant throughout and is equal to 5 x 109 mm4. Also sketch the BMD, 	SFD and elastic curve.

(Contd…2)

Code No: 310102.				.. 2..				Set No: 2

5.	Define Strain energy. Derive an expression for Strain energy for a linear elastic 	system under shear force.

6. 	The system of concentrated loads shown below rolls from left to right on a girder of span 16m, 30kN load leading. For a section 4m from left support , determine the maximum B.M and S.F

Wheel load (kN) : 		20	50	50	40	30

Spacing (m)       :                          1.5       1.5        2          1

7.	A beam ABCD is simply supported at A,B,C and D and contains two internal 	hinges in the span BC at E and F. AB=12m, BC=16m, CD=12m BF=4m and 	FC=4m. Draw the influence lines for reactions at supports B and D and B.M at G 	where G is 5m from A.

8.	Draw the shear force and bending moment diagram for the given over hanging 	beam as loaded below, locate the points of contra flexure and find the maximum 	bending moment. Use graphical procedure.

~ ~ ~ ~

Code No: 310102.

Set No:

3

III B.Tech. I-Semester Regular Examination November 2003

STRUCTURAL ANALYSIS-I

( Civil Engineering  )

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1. 	Draw the bending moment and shear force diagrams for the propped cantilever beam shown in figure.

2.	A beam AB is 6 metres long is fixed at A and simply supported at B and carries a 	point load  at 4 metres from A. Find the  fixing moment at A and the reactions at 	two supports. Draw the shear force and bending moment diagram.

3.	A pin jointed warren truss of span 5m carries loads at the upper joints, as shown 	in Figure. Calculate the forces in members AC, AD, CD, and DE using tension 	coefficients.

4. 	Analyse the continuous beam shown in Figure by Clapeyron's theorem of three 	moments. Sketch the BMD. The supports A, B and D sink by 1.5mm, 3mm and 	2mm respectively. Assume I = 15 x 107 mm and E = 200 GPa.

5.	Define Strain energy. Derive an expression for strain energy due to bending 	moment.

(Contd…2)

Code No: 310102.				.. 2..				Set No: 3

6.	A train of concentrated loads as shown below  move form left  to right on a simply supported girder of span 16m, with the 40kN load leading

Wheel load(kN) : 		20	60	80	40

Spacing(m)        :                          3         2          2

Determine the absolute maximum shear force and bending moment developed in the beam.

7.	A beam ABC is supported at A,B and C and has an internal hinge at D at a            	distance of 3m from A. AB=6m and BC=9m .Draw the influence lines for the            	reactions at   supports and S.F and B.M at a point 1m from B in the span BC.

8. 	Draw the S.F.D and B.M.D for the given simply supported beam shown in figure.

Also show the location and magnitude of maximum bending moment. Use graphical procedure.

~ ~ ~ ~

Code No: 310102.

Set No:

4

III B.Tech. I-Semester Regular Examination November 2003

STRUCTURAL ANALYSIS-I

( Civil Engineering  )

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1. 	Draw the bending moment and shear force diagrams for the propped cantilever beam shown in figure.

2.	A fixed-fixed beam of span 8 m is subjected to a linearly varying load of           	8kN/m from one support to 6 kN/m to the other support. Find the support 	reactions and moments. Draw the shear force and bending moment diagrams.

3.	A symmetrical pin jointed roof truss is loaded as shown in Fig. 5. Evaluate the 	forces in various members using the method of tension coefficients.

4. 	Analyse the continuous beam shown in Figure by Clapeyron's theorem of three 	moments, if support B sinks by 8mm. Also sketch the BMD, SFD and elastic 	curve. Take E = 200 kN/mm2 and I = 0.8 x 108 mm4.

5.	Define Strain energy. Derive an expression for strain energy for a linear elastic 	system under axial load.

(Contd…2)

Code No: 310102.				.. 2..				Set No: 4

6.	A train of 5 wheel loads as shown below, crosses a simply supported beam of span 24m from left to right , with the 120kN load leading.

Wheel load (kN) : 		80	80	200	180	120

Spacing (m)       :        	      	      2           3           2          3

Calculate the maximum positive and negative shear force values at the centre of the span and the absolute maximum B.M anywhere in the span

7.  		An over hanging beam DABC, 14m long is supported at A and B. DA=BC=2m;

AB=10m. Draw the influence lines for the reactions at A and B, shear and bending     moment at section 3m from A. Hence obtain their values for a uniformly     distributed load of 10kN/m, 5m long acting from A.

8. 	Find the reaction at the fixed end and draw the S.F.D and B.M.D for the 	cantilever beam. Use graphical procedure.

~ ~ ~ ~