Code No:310256

OR

III B.Tech. I-Semester Supplementary Examinations, November-2003.

ELECTRO MECHANICS-III

(Electrical and Electronics Engineering)

Time: 3 hours							Max. Marks: 70

Answer any FIVE questions

All questions carry equal marks

- - -

1.	Draw a winding diagram for 3- phases 4- pole, 2 layer, lap winding with two slots per pole per phase what are the modifications to be made in the diagram if it is wave winding.

2.	Write short notes on the following:

a) Armature reaction and leakage reactance.

b) Phasor diagram of an alternator loaded.

3.	A 600KVA, 3300V, 8-pole, 3-phase, 50Hz alternator has the following characteristics

| Amp-turns/pole | 4000 | 5000 | 7000 | 10000 |
| --- | --- | --- | --- | --- |
| Terminal EMF | 2850 | 3400 | 3850 | 4000 |

There are 200 conductors in series per phase. Find the SC characteristics, the field ampere-turns for full load 0.8 P.f (Lagging) and the voltage regulation, having given that the inductive drop at full load is 7% and that the equivalent armature reaction in amp-turns per pole = 1.06 X ampere-conductor per phase per pole

4.	Two similar 6000V, 3-phase generators are running in parallel at constant voltage and frequency bus bars. Each has an equivalent resistance and reactance of 0.05 ohms and 0.5 ohms respictively and supplies one half of a total load of 10000KW at a lagging power factor of 0.8, the two machines being similarly excited, If the excitation of one machine is adjusted until the armature current is 438 A and the steam supply to the turbine remains unchanged, find the armature current, the emf and the power factor of the other alternator.

5.	A synchronous motor has an equivalent armature reactance of 3.3 .  The exciting current is adjusted to such a value that the open circuit emf is 950V.  Find the pf at which the motor would operate when it takes 80kW from 800V supply line.

Contd…..2

Code No:310256				-2-				OR

6.a)	Explain the operation of a synchronous motor with variable load at constant excitation.

b)	Derive the torque developed in a synchronous motor.

7.a)	Draw the equivalent circuit of a single phase induction motor and discuss the 	 	experimental procedure to determine the parameters

b) 	Find the mechanical power out put of a 185Watts, 4pole 110volts, 50Hz single phase induction motor whose constants are given below at a slip of 0.05

R1  = 1.86 ohms 		X1= 2.56 ohms

xm  = 53.5 ohms		R2= 3.56 ohms

x2   = 2.56 ohms		Core loss = 4.0 Watts

Friction and windage lossses = 13.0 Watts

8.a)	Discuss the difference between capacitor start induction run motor and capacitor start and run induction motor.  Draw the performance characteristics.

b)  	The following tests results were obtained in respect of 230Volts single phase induction motor

No load tests	230V, 6.25A	360Watts

locked rotor test	126v,  15.0A.	577Watts

Stator winding resistance   =  1.5 ohms

Draw  the equivalent circuit diagram with parameters

- - -