Code No.310351

OR

III B.Tech. I-Semester Supplementary Examinations, November-2003

THERMAL ENGINEERING - II

(Common to Mechanical Engineering, Production Engineering and Mechanical Manufacturing Engineering)

Time: 3 hours							Max.Marks:70

Answer any FIVE questions

All questions carry equal marks

---

1.a)	What are the main differences between 4-stroke and two-stroke engines?

b)	A petrol engine works on otto-cycle the pressure, temperature and volume of air 	at the beginning of the cycle are 0.93 bar, 38oC and 0.028 cu.m. The pressures at 	the end of compression and at the end of heat addition are 7.87 bar and 21.97 bar. 	If there are 250 working cycles per minute, find the ideal power developed by the 	engine. Also find the efficiency of the cycle.

2.a)	Describe the working of a single jet carburettor and list its limitations.

b)	Explain why a single jet carburettor gives richer and richer mixture with the 	increase in speed of the engine.

3.a)	Describe with neat sketch the working of fuel pump used in high speed diesel 	engine.

b)	What are the specific requirements of fuel injection system used in compression 	ignition engines.

4.a)	What type of ignition system is used in scooters and mopeds? Explain the same 	with circuit diagram.

b)	Draw the simple diagram of spark plug and explain its function to initialize spark.

5.a)	What are the advantages and disadvantages of water cooled system over air 	cooled system?

b)	With the help of line diagram explain forced circulation water cooled system from 	I.C. Engine.

c)	What are the advantages of lubrication systems for I.C. Engines? Explain the 	method of wet-sump lubrication for engines.

6.a)	Describe the different combustion stages in S.I engines.

b)	Explain the difference between

(i) pre-ignition		(ii) auto-ignition  and 	(iii) detonation.

Explain the favourable circumstances for the occurance of the above phenomena.

(Contd…2)

Code No.310351				-2-			OR

7.a)	Explain the method of finding the mass of air supplied to an engine 	experimentally.

b)	A 4-cylinder 4-stroke S.I engine has compression ratio of 6:1.

On test the following date recorded:

Net brake load = 20N; brake arm = 0.5m;  imep = 6 bar

N = 2500 rpm;	 F.C = 10 kg/hr;  C.V = 45000 kJ/kg;

Cylinder bore = 86mm;  stroke = 100 mm

Calculate (i) mechanical efficiency	(ii) Brake Power Efficiency

8.a)	What are the advantages of constant pressure open cycle gas turbine over constant 	volume open cycle gas turbine?

b)	Discuss the means of improving the specific output and thermal efficiency of 	simple open-cycle gas turbine plant.

c)	What are the fundamental differences between the jet propulsion and rocket 	propulsion?

***     ***     ***