Code No.310851

OR

III B.Tech. I-Semester Supplementary Examinations, November-2003

MECHANICAL UNIT OPERATIONS

(Chemical Engineering)

Time: 3 hours							Max.Marks:70

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Give an account of the characterization of solid particles.

b) 	The screen analysis shown below applies to a sample of crushed quartz. The density of the particles is 2650 kg/m3 and the shape factors are: a = 2 and           Φs = 0.571. For the material between 4 mesh and 200 mesh in particle size calculateDs and Dw.

| Mesh no. | Dpi, mm | Mass fraction, Xi | Mesh no. | Dpi, mm | Mass fraction, Xi |
| --- | --- | --- | --- | --- | --- |
| 4 | 4.699 | 0.0000 | 35 | 0.417 | 0.9616 |
| 6 | 3.327 | 0.0251 | 48 | 0.295 | 0.9718 |
| 8 | 2.362 | 0.1501 | 65 | 0.208 | 0.9795 |
| 10 | 1.651 | 0.4787 | 100 | 0.147 | 0.9853 |
| 14 | 1.168 | 0.7278 | 150 | 0.104 | 0.9894 |
| 20 | 0.833 | 0.8868 | 200 | 0.074 | 0.9925 |
| 28 | 0.589 | 0.9406 | Pan | --- | 1.0000 |

2.	Discuss in detail about:

- kneaders, dispersers and masticators

b)   mixer-extruders.

3.a) 	What is a fluid energy mill?

b) 	Draw a neat sketch of a fluid energy mill and name its various parts.

c)	Explain mechanism of size reduction of particles in it.

4.a) 	Discuss the principle of centrifugal filtration.

b) 	A rotary drum filter with 25% submergence is to be used to filter a concentrate 	aqueous slurry of CaCo3 containing 250 kg of solids per cubic metre of water.  	The pressure drop is 500 mm Hg. If the filter cake contains 50% moisture (wet 	basis), calculate the filter area required to filter 10 kg / min of slurry when the 	filter cycle time is 5 min. Assume that filter medium resistance is negligible.  The 	temperature during test cycle is 20oC.

Given	:	0	=	1.2 x 1011 m/kg ; s = 0.25

  = 	0 (p)s

For water	:	 = 1 CP ;  = 980 kg/m3

Density of calcium carbonate = 250 kg/m3

(Contd…2)

Code No.310851				-2-			OR

5.a)	Write about concentration polarization in ultrafiltration membrane.

b)	Discuss about the ultrafiltration of skimmed milk in a hollow fiber UF module.

6.a)	Define Flocculation.

b)	Discuss the settling characteristics of fine solids from liquids using batch 	sedimentation test.

7. 	A flat-blade turbine with six blades is installed centrally in a vertical tank. The 	tank is 1.5 m in diameter; the turbine is 0.5 m in diameter and is positioned 0.5 	from the bottom of the tank. The turbine blades are 125 mm wide. The tank is 	filled to a depth of 1.5 m with rubber-latex compound at 65oC having a viscosity 	of 1200 P and a density of 1129 kg/m3. The turbine is operated at 95 rpm. The 	tank is baffled. What power will be required to operate the mixer?

8.	What is a crystal? Write about invariant crystals and about crystal size and shape factors?

***     ***     ***