Code No: 311152

OR

III B.Tech., I-Semester Supplementary Examinations, November-2003

DIGITAL CIRCUITS DESIGN AND APPLICATIONS

(Bio-Medical Engineering)

Time: 3hours								Max.Marks

Answer any FIVE questions

All questions carry equal marks

----

1.	The pulse from a high-voltage generator rises linearly for 0.05 μ Sec. and then remains constant for 1 μ Sec. The rate of rise of the pulse is measured with an RC differentiating circuit whose time constant is 250 p sec. If the positive output voltage from the differentiator has a maximum value of 50V, what is the peak voltage of the generator?

2.	Consider a symmetrical collector-coupled astable multi using n-p-n silicon transistors. The circuit and device parameters are VCC =6V. RC = 560Ω, R=5.6K. C=50PF, hFE = 40 and rbb’ = 200 Ω. Calculate (a) the waveforms at the base and collector of one transistor and plot to scale (b) the frequency of oscillation and (c) the recovery time.

3.	A deflection coil has an inductance L and is fed from a voltage source V through a Resistance R. If the input is of the form V=A+Bt, show that the current is iL=A−KLR1−e−RtL+Kt where K=B/R. In order that the sweep be precisely linear, A must equal KL if the pedestal does not have this proper value, what happens to the sweep?

4.a)	Design an OR-to-AND gate combinational network for the Boolean algebra expressionABCD+ABCD+ABCD+ABC {D+(ABCD+ABCD).

The two terms in parentheses are don’t care terms.

b)	Using only NAND gates, design a combinational network for the Boolean algebra function given in (a).

5.a)	Define encoder. Indicate a diode matrix encoder to transform a decimal number into a binary code.

b)	Show how a multiplexer may be used as parallel-to-serial converter.

6.	Show the system of a 4 bit parallel binary adder, constructed from single bit full address. Explain its operation.

7.a)	Draw the block diagram for an up-down counter. Explain its operation.

b)	Explain how to modify a ripple counter so that it divides by N, where N is not a power of 2.

(Contd…2)

Code No:311152				:: 2 ::				OR

8.a)	Explain how the symmetrical excitation and for LCD drive is obtained in practice.

b)	What are the merits and demerits of LCD over LED.

-*-*-*-