Code No. 311454

OR

III B.Tech. I-Semester Supplementary Examinations, November-2003

# Introduction of Electronic Circuits

(Mechatronics)

Time: 3 hours							Max.Marks:70

Answer any FIVE questions

All questions carry equal marks

---

1.a)	A transistor with  = 100 is to be used in a CE configuration with collector-to-base bias, as shown in the figure.  The collector circuit resistance is Rc = 1K and     Vcc = 10 V.  Assume VBE = 0.  Choose Rb so that the quiescent collector-to-emitter voltage is 4V.  Find the stability factor S.

b)	Determine the stability factor S for the circuit shown in the  Figure.

Contd….2.

Code No.311454			:: 2 ::						OR

2.a)	Compare the voltage gain, current gain, input impedance, and output impedance of transistor amplifiers with different configurations (CE, CC, CB).

b)	For the amplifier shown in figure, compute AI = Io/Ii, Avs = Vo/Vs, and Ri.  Transistor h-parameters are as given below.

hie = 1100 ;			hre = 2.5 x 10-4

hfe = 50;			hoe = 25 x 10-6 A/V.

3.	Find the voltage gain Avs = Vo/Vs, AI = Io/Ii, and input impedance Ri of the cascaded amplifier shown in the figure.  Assume hie = 1000;  hre = 10-4;  hfe = 50; hoe = 10-8 A/V.

Contd….2.

Code No.311454			:: 3 ::						OR

4.a)	The amplifier shown in the figure, utilizes an n-channel FET for which Vp = -2.0V, gmo = 1.6 mA/V, and IDSS = 1.65 mA.  It is desired to bias the circuit at ID = 0.8 mA, using VDD = 24V.  Assume rd>> Rd.  Find VGS, gm, Rs, and Rd, such that the voltage gain is at least 20dB, with Rs by passed with a large capacitance Cs.

b)	Calculate the voltage gain Av = Vo/Vi at 1 kHz for the circuit shown in figure.  The FET parameters are gm = 2 mA/V and rd = 10K.

5.a)	Discuss the effects of voltage-series negative feedback on the voltage gain, current gain, input impedance, and output impedance of the amplifier.

b)	An ac amplifier has a mid-band voltage gain of 100, and the lower and upper      3-dB frequencies of 10 kHz and 100 kHz.  If 10% negative feedback is applied, what would be the mid-band gain, and the lower and upper 3-dB frequencies.

6.a)	Draw the circuit diagram of an RC phase shift oscillation, and derive the condition for producing oscillations in it.

- Draw the circuit diagram of Colpitts oscillator and explain its functioning.

7.a)	Explain in detail about the transformer coupled transistor power amplifier.

b)	Describe with a neat circuit diagram the working of a class B power amplifier.  Prove that its maximum conversion efficiency is about 78.5%.

8.	Write short notes on any THREE of the following:

a) Thermal runaway in transistor	b) Common drain FET amplifier

c) Wein-bridge oscillator		d) Current-shunt feed back.

@@@@@