Code No.321301

Set No.

1

III B.Tech. II-Semester Supplementary Examinations, November-2003

COMPUTER NETWORKS

(Electronics and Control Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Write a detailed note on the ISDN system architecture.

b)	Explain three transmission media of physical layer.

2.a)	Compare virtual circuits with circuit switching

b)	How is transmission done in ATM networks?

3.a)  	Give the algorithm for CRC method of error checking.

b)	Given 1101011011 data frame and generator polynomial  G(x)=x4+x+1

Derive the transmitted frame.

4.a) 	Explain in detail the following protocol

(i)1-persistence CSMA

(ii)non-persistence

(iii)P-persistence CSMA.

b)	Explain the operation of  CSMA/CD protocol.

5.a)  	Explain the concept of load shedding and give its advantages over other 	congestion control techniques.

b)	What is meant by subnet mask?  For a class B network with subnet mask of

255.255.240.0, what is the maximum number of hosts per subnet?

6.a)	What is dynamic binding?

b)	What is the time to live field in an IP packet?

7.a)	Consider the problem of crash recovery. If the interval between the writing and 	sending an acknowledgement, or vice versa, can be made relatively small, explain 	the best sender– receiver strategies for minimizing the chance of protocol failure.

b)	What do you mean by a transport entity? Explain with an example.

8.a)	Explain the architecture and services of E-mail.

b)	State and explain typical mail handling commands.

***     ***     ***

Code No.321301

Set No.

2

III B.Tech. II-Semester Supplementary Examinations, November-2003

COMPUTER NETWORKS

(Electronics and Control Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Write short notes on interfaces and services of ISO layers

b)	Two networks each provide a  reliable  connection oriented service.  One of them 	offers a reliable byte stream and other offers a reliable message stream. Are they 	identical? If so, why is distinction made. If not give an example of how they 	differ.

2.a)	Define:		(i) TELNET	(ii) FTP 	(iii) SPX        (iv) IMP .

b)	Write short notes on X.25  networks.

3.a)    	What is the difference between single mode and multi mode fibres in fibre optic 	networks? Explain.

b)	Explain carrier sense multiple access (CSMA) Protocol.

4.a) 	Explain point to point protocol with frame format.  Explain how it provides the 	facility through link control protocol and network protocol.

b)	What is the remainder obtained by dividing x7+x5+1 by the generator

polynomial x3+1 ?

5.a)	What are the two types of services that the internet provides to its applications? 	List some characteristics of each of these services?

b)	It has been said that flow control and congestion control are equivalent. Is this 	true for the Internets connection-oriented service? Are the objectives of flow 	control and congestion control the same?

6.a)	Why is IP packet fragmentation sometimes necessary?

b)	List some major functions of IP.

7.a)	What is the need of multiplexing the connections? Explain the different forms of 	multiplexing.

b)	Explain the different cases for releasing a connection using three – way 	handshaking.

8.a)	State and explain the fields used in the RFC 822 message header.

b)	What is MIME? And describe it clearly.

***     ***     ***

Code No.321301

Set No.

3

III B.Tech. II-Semester Supplementary Examinations, November-2003

COMPUTER NETWORKS

(Electronics and Control Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Explain  the dependencies among various protocols of TCP/IP.

b)	Justify the need for framing giving an example.

c)	Discuss the original ARPANET  design.

2.a)	What is the need for SMDS to be designed as a connectionless network and frame 	relay as a connection oriented one.

b) 	Does the Novell Netware architecture look more like X.25 or like the internet. 	Explain.

c) 	Give the advantages and disadvantages of frame relay over a leased telephone 	line.

3.a)	Given data Frame 1101011101 and Generator Polynomial is 1100

Derive the transmitted Frame

b)	Explain how slotted aloha offers better efficiency. Derive an expression for it.

4.a)	Write the comparison of IEE 802.4 and IEE 802.5 Frame format.

b) 	Explain MAC sub layer functions.

5.a)	Suppose you are developing a standard for a new type of network. You need to 	decide whether your network will use VCs or datagram routing. What are the pros 	and cons for using VCs?

b)	What are the five layers in the Internet protocol stack? What are the principal 	responsibilities for each of these layers?

6.a)	What is a data gram service?

b)	What is the packet assembler/disassembler?

7.	Explain the following with reference to transport layer:

a) Flow control

b) Buffering.

8.a)	State and explain MIME types and subtypes defined in RFC 1521.

b)	Write short notes on POP3, IMAP and DMSP.

***     ***     ***

Code No.321301

Set No.

4

III B.Tech. II-Semester Supplementary Examinations, November-2003

COMPUTER NETWORKS

(Electronics and Control Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a) 	What is Store and Forward packet switching network?. Explain.

b)	What is Communication subnet boundary. Between which layers are they formed. 	Why?

2.a)	What are the different broadband ISDN protocols ? Explain.

b)    	What are the advantages of cell switching when compared with circuit switching?

3.a)	What are the various methods of accessing a broadcast channel?

b) 	Describe the expression for throughput S, for an infinite population pure

ALOHA system.

4.a)	Discuss the principle of working of a token Ring protocol. Bring out the 	advantages of this compared to CSMA/CD.

b)	Explain the principle of Binary Exponential Backoff algorithm.

5.a)	What is hierarchical routing?

b)	How does link state routing overcomes the problem described for the

Bellman-Ford algorithm?

6.a)	Distinguish between network congestion and deadlock.

b)	List some ways to deal with congestion.

7.a)	Describe the TCP service model.

b)	Give a general overview of TCP by defining the related terminology.

8.a)	What is Hyperlink? State and explain different types of browsers.

b)	What is HTTP and URL? Explain different parts in URL.

***     ***     ***